SECTION 11 53 00

LABORATORY EQUIPMENT

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2014 -    2018 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** NuAire Laboratory Equipment Supply ; laboratory equipment.
This section is based on the products of NuAire Laboratory Equipment Supply , which is located at:<BR />2100 Fernbrook Lane N.<BR />Plymouth, MN 55447 <BR />Toll Free Tel: 800-328-3352 <BR />Tel: 763-553-1270 <BR />Fax: 763-553-0459<BR />Email: request info (nuaire@nuaire.com)
Web: http://www.nuaire.com    
 [ Click Here ] for additional information.
Quality and dependability for the future. For over 40 years NuAire has been universally recognized as the leader in providing laboratory professionals with reliable products for the most demanding environments.
NuAire manufactures and provides Biological Safety Cabinets, CO2 Water-Jacketed and Direct Heat Incubators, Laminar Air Flow Equipment, Custom Biological Enclosures, Ultra Low Temperature Freezers, Centrifuges, Small Research Animal Handling Equipment, Pharmacy Barrier Isolators, Polypropylene Fume Hoods and Casework, and variety of complementary products and systems to serve the needs of the world's laboratory community.
.
In every NuAire product you'll find brilliant but practical design, keen attention to detail in every phase of the fabrication and assembly process, thoroughly tested, outstanding value, and dependable service. These are the foundation for NuAire's international reputation, and practical reasons why NuAire is universally recognized as the finest.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Laboratory equipment of the following types:

      - Animal research products.

      - Microbiological CO2 incubators.

      - Custom biological incubators.

      - Laminar airflow products.

      - Compounding pharmacy equipment.

      - Polypropylene fume hoods.

      - Ultra-low temperature freezers.

      - Laboratory centrifuges.

      - Biological safety cabinets.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 26 05 00 - Common Work Results for Electrical.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: provide schematics of mechanical and electrical connections.

    - Verification Samples: For each finish product specified when different colors are available, two samples, minimum size 6 inches (150 mm) square representing actual product.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Minimum 5 years experience manufacturing similar products.

    - Installer Qualifications: Minimum 2 years experience installing similar products.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

      - Finish areas designated by Architect.

      - Do not proceed with remaining work until workmanship is approved by Architect.

      - Rework mock-up area as required to produce acceptable work.

  - PRE-INSTALLATION MEETINGS

    - Convene minimum two weeks prior to starting work of this section.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver and store products in manufacturer's unopened packaging bearing the brand name and manufacturer's identification until ready for installation.

    - Handling: Handle materials to avoid damage.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - SEQUENCING

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

  - WARRANTY

** NOTE TO SPECIFIER ** NuAire's laboratory equipment warranties are as follows:

    - Warranty: Manufacturer's warranty shall not apply to any product or part thereof which has been subject to misuse, abuse, accident, shipping damage, improper installation or service, or damage by fire, flood or acts of God. If the serial number of this product is altered, removed or defaced as to be illegible, the warranty shall be null and void in its entirety.

      - Class II, Type A2 Bio-safety Cabinets with DC-ECM Motors:

        - 60 months (North America, Parts & Labor). 36 months parts and labor.

        - 72 months (International, Parts Only).

      - Standard Airflow Products (BSCs) - 36 Months.

      - Incubators - 24 Months.

      - Freezers - 24 Months.

      - Polypropylene Fume Hoods - 12 Months.

      - Polypropylene Casework - 12 Months.

      - Special Products (Determined by product built).

      - International Warranty - 48 Months (Parts only).

      - Refrigerated and Incubator products have a 24 month warranty.

      - All Products sold internationally are covered with a 48 months Parts-Only warranty.

    - Air Flow Products:

      - Manufacturer warrants that it will repair F.O.B. its factory or furnish without charge, a similar part to replace any material in the equipment within 36 months after the date of sale:

        - If proven to the satisfaction of the company to have been defective at the time it was sold.

        - Failed during the covered period, provided that all parts claimed defective shall be returned, properly identified with a return authorization.

      - Factory installed, customer specified equipment or accessories are warranted only to the extent guaranteed by the original manufacturer, and this warranty shall not apply to any portion of the equipment modified by the user/customer. Claims under this warranty should be directed to Manufacturer Technical Service Department setting forth in detail the nature of the defect, the date of the initial installation and the serial and model numbers of the equipment.

- GENERAL

  - MANUFACTURERS

    - Acceptable Manufacturer: NuAire Laboratory Equipment Supply , which is located at:<BR />2100 Fernbrook Lane N.<BR />Plymouth, MN 55447 <BR />Toll Free Tel: 800-328-3352 <BR />Tel: 763-553-1270 <BR />Fax: 763-553-0459<BR />Email: request info (nuaire@nuaire.com);Web: http://www.nuaire.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Delete if not required.

  - ANIMAL RESEARCH PRODUCTS

** NOTE TO SPECIFIER ** NuAire mobile cage changing stations offer HEPA filtered airflow for product protection on the work surface while providing allergen protection by creating a vacuum air barrier between the user and work zone to assure minimal exposure to animal hair/dander as well as cage bedding material. Delete if not required.

    - Animal Transfer Stations as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AllerGard ES 619 Cage Changing Station offers ISO Class 4 protection for research animals providing allergen protection shed by animals and bedding. The 619 improves productivity offering personnel comfort and convenience through ergonomic design. Delete if not required.

      - AllerGard ES (Energy Saver) NU-619 Dual Access Animal Transfer Station.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-619-300. 36-5/8 (930) W by 28-1/4 inches (718) D worktray dimensions.

        - Model NU-619-400. 42-5/8 (1083) W by 28-1/4 inches (718) D worktray dimensions.

        - Model NU-619-500. 54-5/8 (1387) W by 28-1/4 inches (718) D worktray dimensions.

** NOTE TO SPECIFIER ** Delete option not required.

        - Provide Magnifying Glass Window.

        - Provide One Sided Enclosure Panel w/ Stability Bracket.

        - Provide Feed Hoppers.

        - Provide Stainless Steel Folder Shelf.

        - Provide Shelf Option for User Standing.

        - Provide Light Timer.

        - Provide Removable Divider Panel.

        - Provide Automated Hand Sanitizer.

        - Provide Mouse Guard.

        - Provide Disposable Pre-filters.

        - Provide Watering Systems.

        - Provide Additional Outlets.

        - Provide Custom Solutions as required or indicated.

** NOTE TO SPECIFIER ** The AllerGard NU-619 Innovive Edition allows technicians to prepare the work zone with the Innovive disposable IVC caging system before procedural work to maximize workflow. This method allows for the ability to open, prepare, and maintain 25 irradiated INNOCAGE mouse under ISO Class 4 conditions to maintain sterility. Delete if not required.

      - AllerGard ES (Energy Saver) NU-619 Innovive Edition Animal Transfer Station for INNOCAGE Mouse.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-619-400. 42-5/8 (1083) W by 28-1/4 inches (718) D by 26 inches (659 mm) interior dimensions.

        - Model NU-619-500. 54-5/8 (1387) W by 28-1/4 inches (718) D by 26 inches (659 mm ) interior dimensions.

** NOTE TO SPECIFIER ** Delete option not required.

        - Provide AQUAVIVE Mouse Water Bottle Holder.

        - Provide Magnifying Glass Window.

        - Provide One Sided Enclosure Panel w/ Stability Bracket.

        - Provide Feed Hoppers.

        - Provide Stainless Steel Folder Shelf.

        - Provide Light Timer.

        - Provide Automated Hand Sanitizer.

        - Provide Disposable Pre-filters.

        - Provide Watering Systems.

        - Additional Outlets.

        - Provide Custom Solutions as required or indicated.

** NOTE TO SPECIFIER ** The AllerGard model NU-620 is an Animal Cage Changing / Transfer Station that has been modeled off a Class II Biological Safety Cabinet. Dynamic Air Barrier separating the work zone from laboratory environment. True Laminar Airflow minimizing cross contamination Delete if not required.

      - AllerGard ES (Energy Saver) NU-620 High Containment Animal Transfer Station.

** NOTE TO SPECIFIER ** NuAire Animal Research Biosafety Cabinets provide personnel, product, and environmental protection for sensitive animal handling and cage changing procedures. When used with proper technique, NuAire Safety Cabinets are an effective laboratory aid in obtaining the optimum control over product quality while reducing the potential for exposure of both product and personnel to airborne biological or particulate chemical agents in low to moderate risk-hazard research and drug preparation or product operations, as prescribed by the Center for Disease Control (CDC) Atlanta, Georgia. Delete if not required.

    - Biological Safety Cabinets as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The LabGard ES 602 Animal Handling Biological Safety Cabinet provides personnel, product, and environmental protection for animal handling and cage changing procedures. The 602 offers multiple configurations to customize your application. Delete if not required.

      - LabGard ES (Energy Saver) NU-602 Class II, Type A2 Animal Handling Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-602-400. 46-1/2 x 24-1/2 x 26-1/2 (1232 x 622 x 673) interior dimensions.

        - Model NU-602-500. 58-1/2 x 24-1/2 x 26-1/2 (1486 x 622 x 673) interior dimensions.

        - Model NU-602-600. 70-1/2 x 24-1/2 x 26-1/2 (1791 x 622 x 673) interior dimensions.

** NOTE TO SPECIFIER ** Delete option not required.

        - Provide Germicidal UV Light.

        - Provide Decorative Front Panel.

        - Provide Metal Framed HEPA Filters.

        - Provide Lay in Sorbent Exhaust Filter.

        - Provide Base Stand Risers.

        - Provide Additional Duplex Outlets.

        - Provide GFI.

        - Provide Exhaust Transitions.

        - Provide Exhaust Interlocks.

        - Provide Service Valves.

        - Provide Water Systems.

        - Provide IV Bar w/ 6 Hooks.

        - Provide Custom Solutions as indicated or required.

** NOTE TO SPECIFIER ** The LabGard ES 610 Pass Through Biological Safety Cabinet provides Class II protection by creating a barrier between rooms. Bringing new animals into a facility, the 602 allows technicians control in handling animals during transfers. Delete if not required.

      - LabGard ES (Energy Saver) NU-610 Dual Access Class II, Type A2 Animal Handling Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-610-500. 44 x 28-1/4 x 25 (1118 x 718 x 635) interior dimensions.

        - Model NU-610-600. 64 x 28-1/4 x 25 (1626 x 718 x 635) interior dimensions.

** NOTE TO SPECIFIER ** Delete option not required.

        - Provide Germicidal UV Light.

        - Provide Exhaust Alarm Systems.

        - Provide Service Valves.

        - Provide Water Systems.

        - Provide Exhaust Transitions.

        - Provide Custom Solutions as indicated or required.

** NOTE TO SPECIFIER ** Personnel, Product, and Environmental protection provided by a NSF/ANSI listed Class II Biological Safety Cabinet. A large 12 inch (305 mm) access opening accommodates the largest mouse and rat cages. NSF Listed. Three sizes available 4 ft. (1.2m), 5 ft. (1.5m), 6 ft. (1.8m). Energy Saver Technology. Delete if not required.

      - LabGard ES (Energy Saver) NU-677 Class II, Type A2 Animal Handling Biological Safety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-677-400. 46-3/8 x 28 x 25 (1178 x 718 x 635) Work Zone dimensions.

        - Model NU-677-500. 58-3/8 x 28 x 25 (1483 x 718 x 635) Work Zone dimensions.

        - Model NU-677-600. 70-3/8 x 28 x 25 (1788 x 718 x 635) Work Zone dimensions.

** NOTE TO SPECIFIER ** Delete option not required.

        - Provide Germicidal UV Light.

        - Provide Ground Fault Interrupter for Electrical System.

        - Provide Additional Service Valves for Gas, Air, Vacuum.

        - Provide Additional Outlet.

        - Provide Alarm Systems.

        - Provide Sinks with Hot/Cold or DI Water Faucets.

        - Provide Storage Pull-Out Trays.

        - Provide Lay in Sorbent Exhaust Filter.

        - Provide Decorative Side Panels (hides plumbing fixture connections).

        - Provide HEPA Filters 99.999% at 0.3 Micron.

        - Provide Stainless Steel Armrest.

        - Provide Custom Solutions as indicated or required.

** NOTE TO SPECIFIER ** NuAire Animal Bedding Disposal Stations create a safe and efficient environment when disposing of animal cage waste. Class I Personnel Protection ensures lab technicians will not be affected by odors, aerosols, or lab animal dander. NuAire's customizable features allow even the most cumbersome lab animal cages to be handled with ease. Delete if not required.

    - Bedding Waste Disposal Systems as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AllerGard ES 607 Class I Animal Waste Station minimizes the risk of exposure to allergens and animal infections for the technician handling low to moderate risk materials through HEPA filtration and an air barrier. Delete if not required.

      - AllerGard ES (Energy Saver) NU-607 Class I Animal Bedding Disposal Station.

      - Interior Dimensions (W x D x H): 43-1/4 inches x 27-1/2 inches x 24 inches (1099 mm x 699 mm x 610 mm).

** NOTE TO SPECIFIER ** Delete configuration not required.

      - Chute Design Configuration: Round.

      - Chute Design Configuration: Rectangle.

      - Chute Design Configuration: Double Rectangle.

      - Exhaust HEPA Filter: 99.99% at 0.3 Micron.

      - FlowGard Control System: A pressure monitor constantly inspects plenum pressure alerting to any imbalances in airflow.

      - Energy Saver Motor: An ultra high efficiency motor saves on your energy bills while increasing filter life lowering your total cost of ownership.

      - Dynamic Air Barrier: Strong inflow air velocity at 100 fpm (.51 m/s) creates personnel protection.

** NOTE TO SPECIFIER ** Delete options not required.

      - Provide Spare Disposal Bags.

      - Provide Spare Disposal Can.

      - Provide Replacement Filters.

** NOTE TO SPECIFIER ** Select a NuAire Small Animal Cage Isolator below to produce a mini clean room environment to isolate new animal product when entering the facility or isolate a colony in speculation of infection. Delete if not required.

    - Portable Animal Cleanroom Enclosure as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The LabGard 605 Small Animal Portable Isolator provides protection for animals, technicians, or both simultaneously. The 605 creates a mini clean room to isolate infected or sterile product. Delete if not required.

      - LabGard NU-605 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-605-500: 66 inches W x 79 inches D x 78 inches H (1676 mm x 2007 mm x 1981 mm).

        - Model NU-605-600: 78 inches W x 79 inches D x 78 inches H (1981 mm x 2007 mm x 1981 mm).

      - Recycled Airflow: The LabGard 605 creates a mini clean room through recirculated airflow. The recycling feature produces ultra clean air for both personnel and animals.

      - HEPEX Zero Leak Airflow System: The HEPEX plenum inflates to provide quiet uniform air without transferring vibration. Positive pressure chambers and ducts shall be surrounded by negative pressure eliminating potential leaks. HEPEX fabrics shall be durable and flameproof, provide a barrier against water, gases, fluid, and dust as well as resistance to chemicals, oils, and greases. It shall hold antibacterial properties and is impregnable meaning harmful materials become trapped in the filters.

** NOTE TO SPECIFIER ** Delete options not required.

      - Provide Additional Stainless Steel Rack.

      - Provide Additional Stainless Steel Shelves.

      - Provide Clean Room Canopy.

** NOTE TO SPECIFIER ** Create personnel protection from allergens shed by research animals through an air barrier created at the front of the workstation. Delete if not required.

    - Clean Air Workstation as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The LabGard 603 Reverse Horizontal Flow Cabinet provides Personnel Protection allowing room air to flow over the work zone to create a barrier controlling contaminates generated in the work zone from migrating into the laboratory. Delete if not required.

      - LabGard NU-603 Reverse Horizontal Flow Safety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-603-424: 46-1/2 inches W x 24 inches D x 24 inches H (1181 mm x 610 mm x 610 mm) interior dimensions.

        - Model NU-603-430: 46-1/2 inches W x 24 inches D x 30 inches H (1181 mm x 610 mm x 762 mm) interior dimensions.

        - Model NU-603-624: 70-1/2 inches W x 24 inches D x 24 inches H (1791 mm x 610 mm x 610 mm) interior dimensions.

        - Model NU-603-630: 70-1/2 inches W x 24 inches D x 30 inches H (1791 mm x 610 mm x 762 mm) interior dimensions.

      - Dynamic Air Barrier: Strong inflow air velocity of 100 fpm (0.51 m/s) creates maximum personnel protection.

      - Essentials Control System: Simple switches control the blower, lights, outlets while a magnehelic gauge measures plenum performance.

      - Pre-filter System Extends Life of HEPA Filters: Air enters a pre-filtration system to capture animal hair, dander, and bedding before it enters the equipment's plenum extending the life of the exhaust and supply HEPA filters.

** NOTE TO SPECIFIER ** Delete options not required.

      - Provide Germicidal UV Light.

      - Provide Service Valves.

      - Provide Watering Systems.

      - Provide Stainless Steel Work Surface.

      - Provide Storage Pull Out Trays.

      - Provide Base Stands.

      - Provide Custom Solutions as indicated or required.

** NOTE TO SPECIFIER ** Delete if not required.

  - MICROBIOLOGICAL CO2 INCUBATORS

** NOTE TO SPECIFIER ** NuAire Direct Heat (Air Jacketed) CO2 Incubators provide a stable in-vivo growth model with heating elements located on all 6 sides of the chamber. High density R5 insulation stabilizes the interior chamber temperature requiring less energy to maintain. Unique features such as dual sterilization cycle, humidity and hypoxia control ensure your research needs are met. Delete if not required.

    - Direct Heat (Air Jacket) CO2 Incubators as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5800 provides the basic functionality to promote tissue cell growth by controlling Temperature and CO2 gas levels in the growth chamber. A relative humidity of 90% is produce through convention by a water pan. The Closed Loop HEPA Filtration System creates ISO Class 5 Clean-room like conditions to minimize potential contamination entering the growth chamber. Dual Wave Infrared (IR) Sensor. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5800 Direct Heat CO2 Incubator.

        - Chamber Dimensions (W x D x H) in. (mm): 21 inches x 20 inches x 28-1/2 inches (540 x 509 x 724).

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 6 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES (Energy Saver) NU-5810 Microbiological CO2 Incubator features a 145 degrees C dry sterilization cycle and 95 degrees C humidified decontamination cycle as an added defensive against incubator contamination providing an optimal controlled environment for cell growth. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5810 Direct Heat CO2 Incubator with Dual Sterilization Cycles.

        - Chamber Dimensions (W x D x H) in. (mm): 21 inches x 20 inches x 28-1/2 inches (540 x 509 x 724).

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Closed Loop HEPA Filtration.

        - ISO Class 5 Cleanroom Conditions.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Temperature Uniformity.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 6 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5820 Microbiological CO2 Incubator features relative humidity (RH) control for cell lines prone to dessication by utilizing a sensor that measures humidity in the chamber and injects 99.99% HEPA filtered vapor into the growth chamber to maintain setpoint. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Relative Humidity (RH) Sensor Control. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5820 Direct Heat CO2 Incubator featuring Relative Humidity (RH) Control.

        - Chamber Dimensions (W x D x H) in. (mm): 21 inches x 20 inches x 28-1/2 inches (540 x 509 x 724).

        - Relative Humidity (RH) Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Closed Loop HEPA Filtration.

        - ISO Class 5 Cleanroom Conditions.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Temperature Uniformity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 6 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5831 Direct Heat Hypoxic CO2 Incubator can lower oxygen levels in the growth chamber down to 0.5% accurate with in &plusmn;0.25% through the use of a Zirconia Ceramic O2 Sensor. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Zirconia Oxygen (O2) Sensor Control 0.5 - 21%.

      - In-VitroCell ES (Energy Saver) NU-5831 Direct Heat Hypoxic CO2 Incubator featuring Zirconia Oxygen Control.

        - Chamber Dimensions (W x D x H) in. (mm): 21 inches x 20 inches x 28-1/2 inches (540 x 509 x 724).

        - Hypoxic Zirconia Ceramic O2 Sensor Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 6 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5841 provides an easy method to monitor and control humidity and oxygen levels in the chamber for hypoxic stress studies, stem cell culture, carcinogenic tumors, toxicity, and more. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified).    Relative Humidity (RH) Sensor Control. Zirconia Oxygen (O2) Sensor Control 0.5 - 21%. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5841 Direct Heat Hypoxic CO2 Incubator featuring Humidity and Zirconia Oxygen Control.

        - Chamber Dimensions (W x D x H) in. (mm): 21 inches x 20 inches x 28-1/2 inches (540 x 509 x 724).

        - Hypoxic Zirconia Ceramic O2 Sensor Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Relative Humidity (RH) Control.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 6 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5700 provides the basic functionality to promote tissue cell growth by controlling Temperature and CO2 gas levels in the growth chamber. A relative humidity of 90% is produce through convention by a water pan. The Closed Loop HEPA Filtration System creates ISO Class 5 Clean-room like conditions to minimize potential contamination entering the growth chamber. Dual Wave Infrared (IR) Sensor. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5700 Direct Heat CO2 Incubator.

        - Chamber Dimensions (W x D x H) in. (mm): 20-1/4 inches x 20-11/16 inches x 24 inches (514 x 525 x 610).

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Constant Contamination Control    (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES (Energy Saver) NU-5710 Microbiological CO2 Incubator features a 145 degrees C dry sterilization cycle and 95 degrees C humidified decontamination cycle as an added defensive against incubator contamination providing an optimal controlled environment for cell growth. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5710 Direct Heat CO2 Incubator with Dual Sterilization Cycles.

        - Chamber Dimensions (W x D x H) in. (mm): 20-1/4 inches x 20-11/16 inches x 24 inches (514 x 525 x 610).

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Closed Loop HEPA Filtration.

        - ISO Class 5 Cleanroom Conditions.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Temperature Uniformity.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5720 Microbiological CO2 Incubator features relative humidity (RH) control for cell lines prone to dessication by utilizing a sensor that measures humidity in the    chamber and injects 99.99% HEPA filtered vapor into the growth chamber to maintain setpoint. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Relative Humidity (RH) Sensor Control. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5720 Direct Heat CO2 Incubator featuring Relative Humidity (RH) Control.

        - Chamber Dimensions (W x D x H) in. (mm): 20-1/4 inches x 20-11/16 inches x 24 inches (514 x 525 x 610).

        - Relative Humidity (RH) Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Closed Loop HEPA Filtration.

        - ISO Class 5 Cleanroom Conditions.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Temperature Uniformity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5731 Direct Heat Hypoxic CO2 Incubator can lower oxygen levels in the growth chamber down to 0.5% accurate with in &plusmn;0.25% through the use of a Zirconia Ceramic O2 Sensor. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Zirconia Oxygen (O2) Sensor Control 0.5 - 21%.

      - In-VitroCell ES (Energy Saver) NU-5731 Direct Heat Hypoxic CO2 Incubator featuring Zirconia Oxygen Control.

        - Chamber Dimensions (W x D x H) in. (mm): 20-1/4 inches x 20-11/16 inches x 24 inches (514 x 525 x 610).

        - Hypoxic Zirconia Ceramic O2 Sensor Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 section door

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-5741 provides an easy method to monitor and control humidity and oxygen levels in the chamber for hypoxic stress studies, stem cell culture, carcinogenic tumors, toxicity, and more. Dual Wave Infrared (IR) Sensor. Dual Sterilization Cycles (145 degrees C Dry / 95 degrees C Humidified). Relative Humidity (RH) Sensor Control. Zirconia Oxygen (O2) Sensor Control 0.5 - 21%. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-5741 Direct Heat Hypoxic CO2 Incubator featuring Humidity and Zirconia Oxygen Control.

        - Chamber Dimensions (W x D x H) in. (mm): 20-1/4 inches x 20-11/16 inches x 24 inches (514 x 525 x 610).

        - Hypoxic Zirconia Ceramic O2 Sensor Control.

        - Dual Sterilization Cycles.

        - NuTouch Intelligent Interface.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) Sensor.

        - Relative Humidity (RH) Control.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 section door

        - Provide Custom Solutions as required or as indicated

** NOTE TO SPECIFIER ** NuAire Water Jacket Microbiological CO2 Incubators provide a stable in-vitro growth environment by heating chamber walls utilizing water. Water circulates within the jacket walls producing a temperature uniformity of &plusmn;0.2 degrees C. The larger the mass, the less susceptible the environment within the chamber is to environment fluctuations outside. It also adds cabinet stability for the growth of vibration sensitive cells. Delete if not required.

    - Water Jacketed CO2 Incubators as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-8600 provides the basic functionality to promote tissue cell growth by controlling Temperature and CO2 gas levels in the growth chamber. A relative humidity of 90% is produce through convention by a water pan. The Closed Loop HEPA Filtration System creates ISO Class 5 Clean-room like conditions to minimize potential contamination entering the growth chamber. Dual Wave Infrared (IR) Sensor. Delete if not required.

      - In-VitroCell ES (Energy Saver) NU-8600 water Jacket CO2 Incubator.

        - Chamber Dimensions (W x D x H) in. (mm): 20-3/8 inches x 20-5/8 inches x 24 inches (517x 524 x 610).

        - NuTouch Intelligent Interface.

        - Infrared (IR) Sensor.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) CO2 Sensor.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Humidity Display.

        - Provide Gas tight 4 section door

        - Provide water jacket cooling coil to control temperature down to 5 deg. C.

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** The In-VitroCell ES NU-8631 Water Jacket Hypoxic CO2 Incubator can lower oxygen levels in the growth chamber down to 0.5% accurate within & +/-0.25% through the use of a Zirconia Ceramic O2 Sensor. Dual Wave Infrared (IR) Sensor. Zirconia Oxygen (O2) Sensor Control 0.5 - 21%.

      - In-VitroCell ES (Energy Saver) NU-8631 Water Jacket Hypoxic CO2 Incubator featuring Zirconia Oxygen Control.

        - Chamber Dimensions (W x D x H) in. (mm): 20-3/8 inches x 20-5/8 inches x 24 inches (517 x 524 x 610).

        - Hypoxic Zirconia Ceramic O2 Sensor Control.

        - NuTouch Intelligent Interface.

        - Constant Contamination Control (C3):

          - Closed Loop HEPA Filtration.

          - ISO Class 5 Cleanroom Conditions.

          - Temperature Uniformity.

        - Single Beam, Dual Wave Infrared (IR) CO2 Sensor.

        - Incubator Humidity.

        - Remote monitoring and communication

          - Remote alarm connection

          - 4 to 20 mA connection

          - RS232 connection

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Copper Surfaces.

        - Provide Automatic CO2 Tank Switch (External).

        - Provide CO2 Tank Alarm.

        - Provide Left Hinge Door Swing.

        - Provide Additional Shelves with Slide Brackets.

        - Provide CO2 Analyzer Fyrite Kit (Dry) 0-20%.

        - Provide Replacement Fluid for CO2 Analyzer.

        - Provide Surge Protector.

        - Provide CO2 Regulator (2 Stage).

        - Provide Gas tight 4 sectioned door

        - Provide water jacket cooling coil to control temperature down to 5 deg. C.

        - Provide Custom Solutions as required or as indicated.

** NOTE TO SPECIFIER ** Delete if not required.

  - CUSTOM BIOLOGICAL INCUBATORS

** NOTE TO SPECIFIER ** Custom designed to your exact specifications, NuAire Laminar Airflow Workstations will ensure your application involving high-throughput robotic systems or customized workflow operations will provide proper product sterility while producing less heat buildup with high efficiency motors and proper work dimensions. Delete if not required.

    - Laminar Airflow Workstations as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AireGard ES 101 Custom Vertical Airflow Workstation offers 5 airflow variations to create class 5 air quality for robotic automation systems in life science applications and aid in Pharmacies sterilize preparations of IV admixture, CSP's, or antibiotics. The 101 is custom built to your specifications. Please submit an online inquiry or call 1.800.328.3352 to begin your custom equipment consultation.

      - AireGard ES (Energy Saver) NU-101 Vertical Airflow Clean Workstation.

** NOTE TO SPECIFIER ** Delete airflow design not required.

        - Airflow Design: Total Exhaust.

        - Airflow Design: Exhaust / Recirculation.

        - Airflow Design: Open Base.

        - Airflow Design: Wash Out.

        - Airflow Design: Perforated Wash Through.

        - Essentials Control System.

        - HEPEX Zero Leak Airflow System.

        - Attenumount: Vibration and sound control.

        - Airflow System.

        - 99.99% HEPA Filtration.

        - Air Intake Grill and Pre-Filter.

        - Cool White Fluorescent Lighting.

        - Centrally Located Control Center.

        - Powder Coat Urethane Finish.

        - Interior Cold-Rolled Finished Steel.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Duplex outlets.

        - Provide Exhaust Chambers.

        - Provide Exhaust Transitions.

        - Provide Stainless Steel Interior.

        - Provide IV Bar.

        - Provide Ultraviolet Lights.

        - Provide Service valves for Air, Gas, Vacuum.

        - Provide Side Panels.

        - Provide Tables and Table Tops.

        - Provide Stainless Steel work Surfaces.

        - Provide Increased work Zone Height.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** NuAire Custom Class II Biological Safety Cabinets offer Class II protection for personnel, product, and the environment for applications involving high-throughput robotic systems, large high-speed cell sorters, ultra-centrifuges, and laboratory equipment requiring class II protection from potentially harmful aerosols and airborne particles. Delete if not required.

    - Biological Safety Cabinets as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AutoLabGard ES 125 Mini-Room Biological Safety Cabinet provides Class II protection for applications involving robotic systems, cell sorters, ultra-centrifuges, and any laboratory equipment requiring Class II protection from harmful aerosols and airborne particles. Delete if not required.

      - AutoLabGard ES (Energy Saver) NU-125 Mini-Room Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-125-436: 4 ft. (1.2 m).

        - Model NU-125-636: 6 ft. (1.8 m).

        - Model NU-125-648: 6 ft. (1.8 m).

        - Model NU-125-836: 8 ft. (2.4 m).

        - Model NU-125-848: 8 ft. (2.4 m).

        - Model NU-125-1036: 10 ft. (3 m).

        - Model NU-125-1048: 10 ft. (3 m).

        - Model NU-125: Custom Dimensions.

        - Essentials Control System.

        - Energy Saver Motor.

        - TruLaminar Airflow.

        - Dynamic Air Barrier.

        - HEPEX Zero Leak Airflow System.

        - Attenumount: Vibration and sound control.

        - Airflow System.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Stainless Steel Portable Table w/ Auto Life & Casters.

        - Provide Exhaust Transitions.

        - Provide Additional Duplex Outlets.

        - Provide Service Valves.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** The LabGard ES S425 Biological Safety Cabinet is a custom designed to provide personnel, product, and environmental protection to robotic automation systems with the airflow characterizes of a Class II, Type A2 biosafety cabinet. Delete if not required.

      - LabGard ES (Energy Saver) NU-S425 Custom Designed Class II, Type A2 Biosafety Cabinet.

        - Built for your Application.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

** NOTE TO SPECIFIER ** The LabGard ES S430 Total Exhaust Biological Safety Cabinet is a custom designed for robotic systems seeking personnel, product, and environmental protection from non-flammable or explosive volatile chemicals and radionuclides. Delete if not required.

      - LabGard ES (Energy Saver) NU-S430 Custom Designed Class II, Type B2 Biosafety Cabinet.

        - Built for your Application.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

** NOTE TO SPECIFIER ** Delete if not required.

  - LAMINAR AIRFLOW PRODUCTS

** NOTE TO SPECIFIER ** NuAire Air Showers are mounted on your facilities ceiling as an inexpensive way of creating a clean room providing both HEPA filtered air and fluorescent lighting in a single unit. The modules may be placed as needed to provide light and clean air over critical areas. Delete if not required.

    - Ceiling Mounted Air Showers as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AireGard 105 Cleanroom Air Shower was developed to provide an inexpensive solution for laboratories and facilities looking create a sterile clean room having Class 100 Air through HEPA filtration. Delete if not required.

      - Model AireGard NU-105 Air Cleaning Ceiling Module.

        - Size: NU-105-400.

        - Size: NU-105-424.

        - Size: NU-105-430.

        - Size: NU-105-530.

        - Size: NU-105-630.

        - Cleanroom Creating Air Shower.

        - Single-Pass Air Shower.

        - Pre-filter System Extends Life of HEPA Filters.

        - Exhaust HEPA Filter 99.99% at 0.3 Microns.

        - Flow Control Valve.

        - Bottom Filter Service.

        - 16 Gauge Welded Construction.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Fluorescent Lighting.

        - Provide Duplex Outlets.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** NuAire Horizontal Laminar Airflow Hoods provide a HEPA filtered clean work area that is ideal for operations requiring a particle-free, bacteria-free, clean air environment. The clean work area provides an excellent work space for small laboratory appliances, microscopes, pipetting, sterile drug preparations, IV admixture preparation, or any application requiring work-in-progress protection from outside potentially harmful contaminants. Delete if not required.

    - Horizontal Laminar Airflow Workstations as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AireGard ES 201 Horizontal Laminar Airflow Workstation provides a HEPA filtered clean work area for laboratory appliances, microscopes, pipetting, compounding sterile drug preparations or IV admixture, or any applications requiring a clean air environment. Delete if not required.

      - AireGard ES (Energy Saver) NU-201 Table Top Horizontal Airflow Workstation.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model/ Exterior Dimensions/ Interior Dimensions/ Weight.

        - Model NU-201-224/ 2 ft. (0.6 m)/ 26 x 32 x 46 (660 x 813 x 1168) / 23 3/4 x 20 x 22 3/8 (603 x 508 x 568)/ 255 lbs. / 116 kg.

        - Model NU-201-230/ 2 ft. (0.6 m)/ 26 x 32 x 52 (660 x 813 x 1321)/ 23 3/4 x 20 x 28 3/8 (603 x 508 x 721)/ 255 lbs. / 116 kg.

        - Model NU-201-324/ 3 ft. (0.9 m)/ 38 x 32 x 46 (965 x 813 x 1168)/ 34 3/4 x 20 x 22 3/8 (883 x 508 x 568)/ 350 lbs. / 159 kg.

        - Model NU-201-330/ 3 ft. (0.9 m)/ 38 x 32 x 52 (965 x 813 x 1321)/ 34 3/4 x 20 x 28 3/8 (883 x 508 x 721)/ 350 lbs. / 159 kg.

        - Model NU-201-336/ 3 ft. (0.9 m)/ 38 x 32 x 58 (965 x 813 x 1473)/ 34 3/4 x 20 x 34 3/8 (883 x 508 x 873)/ 350 lbs. / 159 kg.

        - Model NU-201-424/ 4 ft. (1.2 m)/ 50 x 32 x 46 (1270 x 813 x 1168)/ 46 3/4 x 20 x 22 3/8 (1187 x 508 x 568)/ 380 lbs. / 172 kg.

        - Model NU-201-430/ 4 ft. (1.2 m)/ 50 x 32 x 52 (1270 x 813 x 1321)/ 46 3/4 x 20 x 28 3/8 (1187 x 508 x 721)/ 380 lbs. / 172 kg.

        - Model NU-201-436/ 4 ft. (1.2 m) / 50 x 32 x 58 (1270 x 813 x 1473)/ 46 3/4 x 20 x 34 3/8 (1187 x 508 x 873)/ 380 lbs. / 172 kg.

        - Model NU-201-524/ 5 ft. (1.5 m)/ 62 x 32 x 46 (1575 x 813 x 1168)/ 58 3/4 x 20 x 22 3/8 (1492 x 508 x 568)/ 520 lbs. / 236 kg.

        - Model NU-201-530/ 5 ft. (1.5 m)/ 62 x 32 x 52 (1575 x 813 x 1321)/ 58 3/4 x 20 x 28 3/8 (1492 x 508 x 721)/ 520 lbs. / 236 kg.

        - Model NU-201-536/ 5 ft. (1.5 m)/ 62 x 32 x 58 (1575 x 813 x 1473)/ 58 3/4 x 20 x 34 3/8 (1492 x 508 x 873)/ 520 lbs. / 236 kg.

        - Model NU-201-624/ 6 ft. (1.8 m)/ 74 x 32 x 46 (1880 x 813 x 1168)/ 70 3/4 x 20 x 22 3/8 (1797 x 508 x 568)/ 550 lbs. / 249 kg.

        - Model NU-201-630/ 6 ft. (1.8 m)/ 74 x 32 x 52 (1880 x 813 x 1321)/ 70 3/4 x 20 x 28 3/8 (1797 x 508 x 721)/ 550 lbs. / 249 kg.

        - Model NU-201-636/ 6 ft. (1.8 m)/ 74 x 32 x 58 (1880 x 813 x 1473)/ 70 3/4 x 20 x 34 3/8 (1797 x 508 x 873)/ 550 lbs. / 249 kg.

        - Model NU-201-830/ 8 ft. (2.4 m)/ 98 x 32 x 52 (2489 x 813 x 1321)/ 94 3/4 x 20 x 28 3/8 (2397 x 508 x 721)/ 760 lbs. / 345 kg.

        - Essentials Control System.

        - Energy Saver Motor.

        - Experience Ergonomics.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

        - Class 100 Air Quality.

        - Clear Top / Side Acrylic Panels.

        - Textured Powder Coat Urethane Finish.

        - Smooth White Formica Work Surface.

        - Front Maintenance.

        - Florescent Lighting.

        - Protective Grill.

        - Glass Disposable Pre-filter.

        - Base Support Stands.

        - Germicidal UV Light.

        - Vinyl Curtain - Front Closure for UV Light.

        - Duplex Outlets.

        - Service Valves.

        - GFI.

        - IV Bar w/ 6 Hooks.

        - Storage Trays.

        - Stainless Steel Work Surface.

        - Stainless Steel Sidewalls.

        - Seismic Brackets.

        - Custom Solutions.

** NOTE TO SPECIFIER ** The AireGard ES 301 Horizontal Laminar Airflow Workstation provides a Class 5 clean work area for laboratory appliances, microscopes, pipetting, compounding sterile drug preparations, IV admixture, electronic testing, manufacturing, or inspections. Delete if not required.

      - AireGard ES (Energy Saver) NU-301 Console Horizontal Airflow Workstation.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model/ Exterior Dimensions/ Interior Dimensions/ Weight.

        - NU-301-330/ 3 ft. (0.9 m)/ 38 x 32 x 72 1/2 in. (965 x 813 x 1842 mm)/ 34 3/4 x 20 x 28 3/8 in. (883 x 508 x 721 mm)/ 410 lbs. / 186 kg.

        - NU-301-424/ 4 ft. (1.2 m)/ 50 x 32 x 66 1/2 in. (1270 x 813 x 1689 mm)/ 46 3/4 x 20 x 22 3/8 in. (1187 x 508 x 568 mm)/ 470 lbs. / 213 kg.

        - NU-301-430/ 4 ft. (1.2 m)/ 50 x 32 x 72 1/2 in. (1270 x 813 x 1842 mm)/ 46 3/4 x 20 x 28 3/8 in. (1187 x 508 x 721 mm)/ 470 lbs. / 213 kg.

        - NU-301-436/ 4 ft. (1.2 m)/ 50 x 32 x 78 1/2 in. (1270 x 813 x 1994 mm)/ 46 3/4 x 20 x 34 3/8 in. (1187 x 508 x 873 mm)/ 470 lbs. / 213 kg.

        - NU-301-530/ 5 ft. (1.5 m)/ 62 x 32 x 72 1/2 in. (1575 x 813 x 1842 mm)/ 58 3/4 x 20 x 28 3/8 in. (1492 x 508 x 721 mm)/ 525 lbs. / 238 kg.

        - NU-301-536/ 5 ft. (1.5 m)/ 62 x 32 x 78 1/2 in. (1575 x 813 x 1994 mm)/ 58 3/4 x 20 x 34 3/8 in. (1492 x 508 x 873 mm)/ 525 lbs. / 238 kg.

        - NU-301-624/ 6 ft. (1.8 m)/ 74 x 32 x 66 1/2 in. (1880 x 813 x 1689 mm)/ 70 3/4 x 20 x 22 3/8 in. (1797 x 508 x 568 mm)/ 650 lbs. / 295 kg.

        - NU-301-630/ 7 ft. (1.8 m)/ 74 x 32 x 72 1/2 in. (1880 x 813 x 1842 mm)/ 70 3/4 x 20 x 28 3/8 in. (1797 x 508 x 721 mm) 650 lbs. / 295 kg.

        - NU-301-636/ 6 ft. (1.8 m)/ 74 x 32 x 78 1/2 in. (1880 x 813 x 1994 mm)/ 70 3/4 x 20 x 34 3/8 in. (1797 x 508 x 873 mm)/ 650 lbs. / 295 kg.

        - NU-301-648/ 6 ft. (1.8 m)/ 74 x 32 x 90 1/2 in. (1880 x 813 x 2299 mm)/ 70 3/4 x 20 x 46 3/8 in. (1797 x 508 x 1178 mm)/ 650 lbs. / 295 kg.

        - NU-301-830/ 8 ft. (2.4 m)/ 98 x 32 x 72 1/2 in. (2489 x 813 x 1841 mm)/ 94 3/4 x 20 x 28 3/8 in. (2397 x 508 x 721 mm)/ 940 lbs. / 426 kg.

        - Essentials Control System.

        - Energy Saver Motor.

        - Experience Ergonomics.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System Class 100 Air Quality.

        - Clear Top / Side Acrylic Panels.

        - Textured Powder Coat Urethane Finish.

        - Smooth White Formica Work Surface.

        - Front Maintenance.

        - Florescent Lighting.

        - Protective Grill.

        - Glass Disposable Pre-filter.

        - Leg Levelers.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Germicidal UV Light.

        - Provide Vinyl Curtain - Front Closure for UV Light.

        - Provide Casters.

        - Provide Duplex Outlets.

        - Provide Service Valves.

        - Provide GFI.

        - Provide IV Bar w/ 6 Hooks.

        - Provide Storage Trays.

        - Provide Stainless Steel Work Surface.

        - Provide Stainless Steel Sidewalls.

        - Provide Seismic Brackets.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** NuAire Mobile Air Scrubbers are portable self-contained units capable of filtering particulates where ISO class 7 or 8 air quality is a requirement. Cabinets utilize rough disposable pre-filters to help prolong the life cycle of the main 99.99% HEPA filter. Each scrubber can be used in a variety of different locations, depending upon the need for clean air. Delete if not required.

    - Portable Mobile Filtration Units as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AireGard ES 114 Air Scrubber provides ISO Class 7 or Class 8 air in a Clinical or Medical setting where occupants' infection status is a concern, construction, or bio-terrorism to capture and contain microbes. Delete if not required.

      - AireGard ES (Energy Saver) NU-114 Portable Air Scrubber.

        - Exterior Dimensions (W x D x H) in. (mm): 26 1/2 x 26 x 55 5/8 (673 x 660 x 1413).

        - Weight: 270 lbs. / 122 kg.

        - Inflow Velocity: 90 fpm (0.46 m/s).

        - Quality Assurance: UL, UL-C.

** NOTE TO SPECIFIER ** NuAire Vertical Flow Hoods are engineered to provide the ideal particle-free, bacteria-free clean-air environment needed for laboratory work, testing, manufacturing, inspection, or pharmaceutical procedures. Exclusive design innovations provide exceptional performance in the most demanding environments. Delete if not required.

    - Vertical Laminar Airflow Workstation as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The AireGard ES 126 Vertical Laminar Airflow Workstation is creates a clean-air environment needed for laboratory or pharmaceutical procedures. Great for use with a thermal cycler, centrifuge, preparations of IV Admixture, Antibiotics, or other drugs. Delete if not required.

      - Model AireGard ES (Energy Saver) NU-126 Vertical Airflow Workstation.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-126-300.

        - Model NU-126-400.

        - Model NU-126-600.

        - FlowGard Control System.

        - Energy Saver Motor.

        - Experience Ergonomics.

        - TruLaminar Airflow.

        - Class 5 Air Quality.

        - Cool White Fluorescent Lighting.

        - Clear Acrylic Side Walls.

        - Hinged Clear Polycarbonate Viewing Window.

        - Two Service Ports.

        - 99.99% at 0.3 Micron Supply HEPA Filter.

        - Fiberglass Pre-Filter.

        - White Formica Work Surface.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Germicidal UV Light w/ Front Enclosure.

        - Provide Service Valves.

        - Provide Stainless Steel Work Surface.

        - Provide IV Bar w/ 6 Hooks.

        - Provide Base Support Stands.

        - Provide Seismic Brackets.

        - Provide Custom Solution.

** NOTE TO SPECIFIER ** Delete if not required.

  - COMPOUNDING PHARMACY EQUIPMENT

** NOTE TO SPECIFIER ** NuAire Positive Pressure Recirculating Compounding Aseptic Isolators is designed to provide a sterile positive pressure work environment for the compounding of non-hazardous drugs. HEPA filtered unidirectional flow supply at 20 air changes per minute within both the work zone and interchange areas to assure ISO Class 5 conditions. Laminar airflow assures a continuous stream of HEPA filtered air across the work zone and interchange areas assuring sterility and minimizing cross contamination. Delete if not required.

    - Compounding Aseptic (CAI) as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The PharmaGard ES PR797 Compounding Pharmacy Isolator creates a recirculating positive pressure ISO Class 5 clean air controlled environment for aseptic compounding of non-hazardous drugs guaranteed to meet your USP 797 compliance requirements. Delete if not required.

      - Model PharmaGard ES (Energy Saver) NU-PR797 Compounding Aseptic Isolator.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-PR797-400. 4 feet (1219 mm).

        - Model NU-PR797-600. 6 feet (1829 mm).

        - Experience Ergonomics.

        - Quick Change Sharps and Waste.

        - Counter Balance Inner Transfer Window.

        - Sliding Work Tray.

        - Energy Saver Motor.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

        - Attenumount: Vibration and sound control.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Bench Mount Support.

        - Provide Telescoping Basestand.

        - Provide Auto Lift Basestand.

        - Provide Additional IV Bar w/ 6 Hooks.

        - Provide Cord Pass Through.

        - Provide Pull Out Stainless Steel Tray.

        - Provide Storage Trays.

        - Provide Right Side Transfer Compartment.

        - Provide Dual Side Transfer Compartments (6 ft. Model Only).

        - Provide Three or Four Glove Ports (6 ft. Model Only).

        - Provide Additional Hospital Grade Duplex.

        - Provide PharmaGard Digital Monitor.

        - Provide Seismic Brackets.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** Choose NuAire Compounding Aseptic Containment Isolators when compounding hazardous drugs to minimize airborne toxins from entering the laboratory environment. Negative air pressure is maintained in the work area relative to the interchange compartment. This level of protection ensures chemotherapy agents or other hazardous materials will not escape the work area. Delete if not required.

    - Compounding Aseptic Containment (CACI) as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The PharmaGard ES NR797 Compounding Containment Pharmacy Isolator creates negative pressure recirculating ISO Class 5 clean air environment for aseptic compounding of hazardous drugs and minute quantities of chemotherapy drugs to meet USP 797 requirements. Delete if not required.

      - PharmaGard ES (Energy Saver) NU-NR797 Recirculating Aseptic Containment Isolator.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model NU-NR797-400.

        - Model NU-NR797-600.

        - Experience Ergonomics.

        - Quick Change Sharps and Waste.

        - Counter Balance Inner Transfer Window.

        - Energy Saver Motor.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

        - Attenumount: Vibration and sound control.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Bag-In Bag-Out Exhaust HEPA Filter.

        - Provide Bench Mount Support.

        - Provide Telescoping Basestand.

        - Provide Auto Lift Basestand.

        - Provide Additional IV Bar w/ 6 Hooks.

        - Provide Cord Pass Through.

        - Provide Pull Out Stainless Steel Tray.

        - Provide Storage Trays.

        - Provide Right Side Transfer Compartment.

        - Provide Dual Side Transfer Compartments (6 ft. Model Only).

        - Provide Three or Four Glove Ports (6 ft. Model Only).

        - Provide Additional Hospital Grade Duplex.

        - Provide PharmaGard Digital Monitor.

        - Provide Seismic Brackets.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** The PharmaGard ES NTE797 Total Exhaust Compounding Containment Pharmacy Isolator creates negative pressure ISO Class 5 clean air environment for aseptic compounding of hazardous drugs and chemotherapy drugs to meet USP 797 requirements. Delete if not required.

      - PharmaGard ES (Energy Saver) NU-NTE797 Total Exhaust Aseptic Containment Isolator.

        - Model NU-NR797-400.

        - Model NU-NR797-600.

        - Quick Change Sharps and Waste.

        - Counter Balance Inner Transfer Window.

        - Sliding Work Tray.

        - Energy Saver Motor.

        - TruLaminar Airflow.

        - HEPEX Zero Leak Airflow System.

        - Attenumount: Vibration and sound control.

        - Bag-In Bag-Out Exhaust HEPA Filter.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Bench Mount Support.

        - Provide Telescoping Basestand.

        - Provide Auto Lift Basestand.

        - Provide Additional IV Bar w/ 6 Hooks.

        - Provide Cord Pass Through.

        - Provide Pull Out Stainless Steel Tray.

        - Provide Storage Trays.

        - Provide Right Side Transfer Compartment.

        - Provide Dual Side Transfer Compartments (6 ft. Model Only).

        - Provide Three or Four Glove Ports (6 ft. Model Only).

        - Provide Additional Hospital Grade Duplex.

        - Provide Seismic Brackets.

        - Provide Custom Solutions.

** NOTE TO SPECIFIER ** Delete if not required.

  - POLYPROPYLENE FUME HOODS

    - Vertical Laminar Airflow Fume Hood:

** NOTE TO SPECIFIER ** The FumeGard 156 Vertical Laminar Airflow Fume Hood for high acid and trace metal analysis meets NSF / ANSI Standards for personnel, product, and environmental protection through HEPA filtration. Durable polypropylene construction is rust resistant. Delete if not required.

      - Product: FumeGard NU-156 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: 4 feet (1232 mm) 24 inches (610 mm) work surface depth.

        - Model: 4 feet (1232 mm) 30 inches (610 mm) work surface depth.

        - Model: 5 feet (1524 mm) 24 inches (610 mm) work surface depth.

        - Model: 5 feet (1524 mm) 30 inches (610 mm) work surface depth.

        - Model: 6 feet (1829 mm) 24 inches (610 mm) work surface depth.

        - Model: 6 feet (1829 mm) 30 inches (610 mm) work surface depth.

        - Model: 8 feet (2438 mm) 24 inches (610 mm) work surface depth.

        - Model: 8 feet (2438 mm) 30 inches (610 mm) work surface depth.

        - Certification: NSF No. 49. Meet or exceed Federal Standard 209e, Class 10 air quality conditions, and have been independently tested for containment properties in accordance to ASHRAE Standard 110-1995 for Fume Hoods.

        - Design: Constructed almost entirely of polypropylene, the fume hood contains the HePeX Zero Leak Airflow System, available only from NuAire. The HePeX system prevents uneven particulate loading by eliminating direct blasts to the HEPA filter, dispensing air over 100% of the surface. The system surrounds positive pressure airflow chambers and ducts with negative air pressure relative to the laboratory. This limits the possibility of leaks and guards against filter failure.

        - Performance: Down Flow Velocity: 60 LFPM (0.30 m/s) Inflow Velocity: 105 LFPM (0.53 m/s) at 10 inch Access Opening.

    - Conventional Fume Hood:

** NOTE TO SPECIFIER ** The FumeGard 162 Conventional Corrosive-Resistant Polypropylene Fume Hood is designed to capture, contain, and exhaust fumes, gases, vapors, mists, aerosols, and particulate matter generated within the hood interior. Delete if not required.

      - Product: FumeGard NU-162 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: 4 feet (1232 mm) 24 inches (610 mm) work surface depth.

        - Model: 4 feet (1232 mm) 30 inches (610 mm) work surface depth.

        - Superstructure Constructed from 1/2 inch (13 mm) Stress Relieved, All Seam Welded Polypropylene.

        - All Negative Pressure Interior.

        - Cool White Fluorescent Lighting.

        - Electronic Fluorescent Lamp Ballast.

        - Counterbalanced Lexan View Screen Slides 18 inch (457 mm) Maximum Opening to Fully Closing.

        - Flush Mounted Exterior/Interior Plumbing Chase Access Panels.

        - Connection Provided for Venting a Base Cabinet.

        - Removable Rear Baffle for Maintenance.

        - Reinforced 1/2 inch (13 mm) Thick Polypropylene Work Surface.

        - Lower Work Access Opening Airfoil.

        - PVC Electrical Junction Box.

        - Modular Electrical Component Construction.

** NOTE TO SPECIFIER ** Delete options not required.

        - PVDF Lined Interior or Flame Retardant Polypropylene Fume Hood.

        - Composition Stone/Modified Epoxy Resin Work Surfaces.

        - Air Flow Alarm Systems.

        - Remote Controlled Service Valves for Air, Vacuum, and/or N2.

        - Cascade or Single Rinse Tanks with/without Nitrogen Purge.

        - Rear Exhaust Duct Wash-down System.

        - Polypropylene, Teflon or PVDF Gooseneck Faucets.

        - Custom Polypropylene Sinks with Chemically Resistant "P"-Trap.

        - Teflon, PVDF, or Polypropylene Dip Tanks with/without Drains.

        - Polypropylene or PVDF Sinks with Hot/Cold or D.I. Water Faucets.

        - Teflon Liquid/Air Aspirators to Syphon Chemicals.

        - D.I. Water or N2 Teflon Spray Guns.

        - Exhaust Interlocks for Building Controls.

        - Polypropylene Exhaust Damper with Quadrant Lock.

        - Ground Fault Circuit Interrupter for Duplex Outlet(s).

        - Additional Duplex Outlet(s) with PVC Covers.

    - By-Pass Fume Hood:

** NOTE TO SPECIFIER ** The FumeGard 164 By-Pass Polypropylene Fume Hood is constructed from stress relieved white polypropylene intended for use in high acid environments, trace metal analysis, materials analysis, toxicology, semi-conductor development, prototype/production and/or etching/plating operations. Delete if not required.

      - Product: FumeGard NU-164 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: 4 feet (1232 mm) 24 inches (610 mm) work surface depth.

        - Model: 4 feet (1232 mm) 30 inches (610 mm) work surface depth.

        - Model: 5 feet (1524 mm) 24 inches (610 mm) work surface depth.

        - Model: 5 feet (1524 mm) 30 inches (610 mm) work surface depth.

        - Model: 6 feet (1829 mm) 24 inches (610 mm) work surface depth.

        - Model: 6 feet (1829 mm) 30 inches (610 mm) work surface depth.

        - Model: 8 feet (2438 mm) 24 inches (610 mm) work surface depth.

        - Model: 8 feet (2438 mm) 30 inches (610 mm) work surface depth.

        - Superstructure Constructed from 1/2 inch (13 mm) Stress Relieved, All Seam Welded Polypropylene.

        - All Negative Pressure Interior.

        - Air By-Pass for More Uniform Face Velocity During View Screen Manipulation.

        - Cool White Fluorescent Lighting.

        - Electronic Fluorescent Lamp Ballast.

        - Counterbalanced Lexan View Screen Slides 18 inch (457 mm) Maximum Opening to Fully Closing.

        - Flush Mounted Exterior/Interior Plumbing Chase Access Panels.

        - Polypropylene Service Valves (Except Natural Gas).

        - Connection Provided for Venting a Base Cabinet.

        - Rear Baffle Removable for Maintenance.

        - Reinforced 1/2 inch (13 mm) Polypropylene Work Surface.

        - Lower Work Access Opening Airfoil.

        - PVC Electrical Junction Box.

        - Modular Electrical Component Construction.

** NOTE TO SPECIFIER ** Delete options not required.

        - PVDF Lined Interior or Flame Retardant Polypropylene Fume Hood.

        - Composition Stone/Modified Epoxy Resin Work Surfaces.

        - Air Flow Alarm Systems.

        - Remote Controlled Service Valves for Gas, Air, Vacuum.

        - Cascade or Single Rinse Tanks with/without Nitrogen Purge.

        - Rear Exhaust Duct Wash-Down System.

        - Polypropylene, Teflon or PVDF Gooseneck Faucets.

        - Custom Polypropylene Sinks with Chemically Resistant "P"-Trap.

        - Teflon, PVDF, or Polypropylene Dip Tanks with/without Drains.

        - Polypropylene or PVDF Sinks with Hot/Cold or D.I. Water Faucets.

        - Teflon Liquid/Air Aspirators to Syphon Chemicals.

        - D.I. Water or N2 Teflon Spray Guns.

        - Exhaust Interlocks for Building Controls.

        - Polypropylene Exhaust Damper with Quadrant Lock.

        - Ground Fault Circuit Interrupter for Duplex Outlet(s).

        - Additional Duplex Outlet(s) with PVC Covers.

** NOTE TO SPECIFIER ** Delete if not required.

  - ULTRA-LOW TEMPERATURE FREEZERS

** NOTE TO SPECIFIER ** NuAire Upright Ultra low temperature laboratory freezers offer reliable biological storage. Ergonomic design, optional backup systems, and the industry's best temperature uniformity ensure your NuAire ultra low temperature freezers will store samples in confidence. Delete if not required.

    - Upright Ultra-low Temperature Freezers:

** NOTE TO SPECIFIER ** The Glacier 9483 Upright 17.1 ft3 (483 L) Capacity Ultra Low Temperature Laboratory Freezer creates a stable, uniform, and reliable environment for the preservation of your cells while minimizing frost build up. Delete if not required.

      - Model Glacier NU-9483 Upright Mid Capacity -86 degrees C Ultra-low Temperature Freezer.

        - Interior Volume: 17.1 ft3 (487 L).

        - GC model includes voltage booster.

        - Access port.

        - Heavy-gauge steel cabinet.

        - Heavy-duty swivel casters.

        - Key lock.

        - Four compartments.

        - Four shelves, three adjustable.

** NOTE TO SPECIFIER ** Delete options not required.

        - Cobex temperature recorder.

        - CO2 back-up system [NU-UB2].

        - Painted steel interior.

        - Floor anchor system.

        - Inventory system.

** NOTE TO SPECIFIER ** The Glacier 9668 Upright 23.5 ft3 (668 L) Capacity Ultra Low Temperature Laboratory Freezer creates optimal storage conditions for the preservation of your research through reliable long lasting control systems.

      - Model Glacier NU-9668 Upright Large Capacity -86 degrees C Ultra Low Freezer.

        - Capacity: 3.5 cu.ft. (668 L).

        - GC model includes voltage booster.

        - Access port.

        - Heavy-gauge steel cabinet.

        - Heavy-duty swivel casters.

        - Key lock.

        - Four compartments.

        - Four shelves, three adjustable.

** NOTE TO SPECIFIER ** Delete options not required.

        - Cobex temperature recorder.

        - CO2 back-up system [NU-UB2].

        - Painted steel interior.

        - Floor anchor system.

        - Inventory system.

** NOTE TO SPECIFIER ** Twin compressor and efficient heat exchange within the refrigeration system allows the "Polar Edition" to run quieter and more efficient. Vacuum Insulated Panel Technology allow for more storage of fibroblasts, lymphoctes, hybridomas, stem cells, mammalian cells, tissues, bone marrow and more. Interior Volume: 25.7 ft3 (728 L). Freezer Fiberboard Storage Boxes. Fiberboard Box Maximum Capacity: 2 inch (50 mm) high: 57,600; 3 inch (76 mm) high: 38,400. Vacuum Insulated Panel Technology. Operating Noise: 47 dba. Delete if not required.

      - Model Glacier Polar Edition -86 degrees C Ultra-low Temperature Freezer.

        - Interior Volume: 25.7 ft3 (728 L).

        - Insulated Inner Door: Steel frame, high impact plastic, w/ foam-in place insulation.

        - Inner Door Configuration: Two (2).

        - Door Gaskets: Multi-point compression gaskets.

        - Condenser Filter: Electrostatic filter, front easy access, no tools.

        - Interior Corners: Rounded for easy cleaning.

        - Shelf Bracket: Adjustable.

        - Casters: Four (4) Heavy Duty w/ leveling feet.

        - Seismic Brackets: Standard; hard connection to facility wall.

        - Access Ports: Multiple ports allow use of injection tubes, probes, leads, instrumentation.

** NOTE TO SPECIFIER ** Delete if not required.

  - LABORATORY CENTRIFUGES

    - Ventilated Benchtop Centrifuge:

** NOTE TO SPECIFIER ** The NuWind NU-C200V Multifunction Ventilated Bench Top Laboratory Centrifuge offers 4 x 500ml volume capacity for research, industrial, or clinical diagnostic applications involving virology, bacteriology, immunology, genetics, and more. Delete if not required.

      - Product: NuWind NU-C200V as manufactured by NuAire Laboratory Equipment Supply.

        - Capacity of 4x500ml.

        - Maximum speed of 4500RPM/4188xg with swing-out rotor.

        - Maximum speed of 15300RPM/22380xg with fixed angle rotor.

        - Full touchscreen display.

        - Automatic lid lock.

        - Rotor imbalance detector.

        - Rotor recognition with automatic limitation of maximum speed.

        - Power Factor Correction to filter and correct the incoming and outgoing line current.

        - Control center for easy servicing.

        - Motor accessible and removable from the top of the unit.

    - Refrigerated Benchtop Centrifuge:

** NOTE TO SPECIFIER ** The NuWind NU-C200R Multifunction Refrigerated Bench Top Laboratory Centrifuge offers 4 x 500ml volume capacity for sample and cell preparations in clinical and life science laboratories. Delete if not required.

      - Product: NuWind NU-C200R as manufactured by NuAire Laboratory Equipment Supply.

        - Capacity of 4x500ml.

        - Maximum speed of 5100RPM/5350xg with swing-out rotor.

        - Maximum speed of 18000RPM/30980xg with fixed angle rotor.

        - Full touchscreen display.

        - Automatic lid lock.

        - Rotor imbalance detector.

        - Rotor recognition with automatic limitation of maximum speed.

        - Power Factor Correction to filter and correct the incoming and outgoing line current.

        - Control center for easy servicing.

        - Motor accessible and removable from the top of the unit.

    - High-Speed Centrifuges:

** NOTE TO SPECIFIER ** Model CR22N makes high speed separations made easy through the reliability of Hitachi manufactured centrifuges. Delete if not required.

      - Hitachi Koki himac CR22N High-Speed Refrigerated Centrifuge.

        - Max RCF 55,200xg.

        - Max Capacity 6 Litre.

** NOTE TO SPECIFIER ** NuAire provides General Purpose Ultracentrifuges from Hitachi Koki in the North American market with performance capabilities up to 100,000 rpm (803,000 xg). Delete if not required.

    - Ultracentrifuges:

** NOTE TO SPECIFIER ** The CP100NX is a general-purpose Ultracentrifuge with the performance ability up to 100,000 rpm assisting in a wide range of separation protocols in cell biology, biochemistry, molecular biology and more. Delete if not required.

      - Hitachi Koki himac CP100NX Ultracentrifuge.

      - Max Speed 100,000 rpm.

      - Max RCF 803,000 xg (P100AT2 rotor).

    - Micro Ultracentrifuges:

** NOTE TO SPECIFIER ** The Hitachi Floor Standing Micro Ultracentrifuge provides users the optimum working height to access samples and enter protocol information via touch screen when separating proteins, lipoproteins, cell organelle, DNA, RNA, viruses, carbon nanotubes, nano-particles, and more.

      - Hitachi Koki himac CS150FNX Floor Standing Micro Ultracentrifuge.

        - Max Speed 150,000 rpm.

        - Max RCF 1,050,000 xg (S140AT rotor).

        - Max Capacity (nominal) 30ml x 6 tubes (S50A rotor).

        - Max Capacity (nominal) (Swinging Bucket Rotor) 7ml x 4 tubes (S50ST rotor).

** NOTE TO SPECIFIER ** A table top design allows this slim Micro Ultracentrifuge to save laboratory space when working with proteins, lipoproteins, cell organelle, DNA, RNA, viruses, carbon nanotubes, nano-particles, and more. Delete if not required.

      - Hitachi Koki himac CS150NX Tabletop Micro Ultracentrifuge.

        - Max Speed 150,000rpm.

        - Max RCF 1,050,000 xg (S140AT rotor).

        - Max Capacity (nominal) 30ml x 6 tubes (S50A rotor).

        - Max Capacity (nominal) (Swinging Bucket Rotor) 7ml x 4 tubes (S50ST rotor).

    - Mini-centrifuge:

** NOTE TO SPECIFIER ** The economical and reliable Prism Mini is a compact microcentrifuge. Out of the box functionality and simple operation make the Prism Mini the ideal personal microcentrifuge to outfit each workstation. Delete if not required.

      - Prism LN-C1801 Mini Microcentrifuge.

        - 8 x 1.5 / 2.0 ml tube rotor.

        - 4 x 0.2 ml tube strip / 32 x PCR tube rotor.

** NOTE TO SPECIFIER ** Refer to manufacturer's literature for accessories required and available.

    - Micro-centrifuge:

** NOTE TO SPECIFIER ** The economical and reliable Prism is a compact air-cooled microcentrifuge. Out of the box functionality and simple operation make the Prism the ideal personal microcentrifuge to outfit each workstation. Delete if not required.

      - Prism LN-C2500 Micro Microcentrifuge.

        - 24 x 1.5 / 2.0 ml tube rotor.

        - 16 x 0.2 ml tube strip.

** NOTE TO SPECIFIER ** The economical and reliable Prism R is a compact refrigerated microcentrifuge. Out of the box functionality and simple operation make the Prism R the ideal personal microcentrifuge to outfit each workstation. Delete if not required.

      - Prism LN-C2500R Micro Microcentrifuge.

        - 24 x 1.5 / 2.0 ml tube rotor.

        - 16 x 0.2 ml tube strip.

** NOTE TO SPECIFIER ** Refer to manufacturer's literature for accessories required and available.

    - Provide the Following Accessories: ______.

** NOTE TO SPECIFIER ** Delete if not required.

  - BIOLOGICAL SAFETY CABINETS

** NOTE TO SPECIFIER ** NuAire Class I Biological Safety Cabinets provide personnel protection for handling of low risk aerosols and particulate matter. Room air enters the Cabinet, flowing past the operator into the work access opening providing an air barrier which minimizes contaminants generated within the cabinet work zone from migrating into the room. The air flows through the cabinet carrying contaminants with it, exiting through a HEPA filter as a Class I Cabinet. Delete if not required.

    - Biological Safety Cabinets Class I as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER ** The LabGard ES 813 Class I Biological Safety Cabinet can be used as a balance enclosure or ductless fume hood for personnel protection for the safe handling of low risk powders, liquids, and animal necropsy.

      - Product: LabGard ES (Energy Saver) NU-813 Bench Top Class I Containment Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-813-300/E. Interior: 34-3/8 inches (873 mm) W by 24 inches (610 mm) D by 28 inches (711 mm) H.

        - Model: NU-813-400/E. Interior: 46-3/8 inches (1178 mm) W by 24 inches (610 mm) D by 28 inches (711 mm) H.

        - Model: NU-813-600/E. Interior: 70-3/8 inches (1788 mm) W by 24 inches (610 mm) D by 28 inches (711 mm) H.

** NOTE TO SPECIFIER ** Delete stand height not required.

        - Stand: 30 inches (762 mm) high.

        - Stand: 36 inches (914 mm) high.

** NOTE TO SPECIFIER ** Delete if not required.

        - Provide petcocks.

        - Performance: ANSI/ASHRAE 110 - surrogate powder, NSF/ANSI 49 Class I. Sound Pressure Level of <62 dba.

** NOTE TO SPECIFIER ** Delete exhaust filter medium not required.

        - Exhaust: HEPA exhaust filter 99.995% (0.3 micron effective).

        - Exhaust: Purafil exhaust filter 99.995% (0.3 micron effective).

        - Exhaust: Charcoal exhaust filter 99.995% (0.3 micron effective).

        - Work Access Opening: 8 inches (203 mm). Viewing panel raised to create a 24 inches (610 mm) opening.

        - Lighting: Externally mounted fluorescent 50 to 150 fc (538 to 1600 lux).

        - Average Inflow Velocity: Nominal 60 to 100 fpm (0.30 to 0.51 m/s).

        - Control System: Constant monitoring and alarm set points for high/low air flow.

** NOTE TO SPECIFIER ** The LabGard ES 819 Class I Enclosure is designed to contain and remove contaminates form the work surface with the aid of an external exhaust system for applications involving liquids, powders, animal necropsy, or equipment. The 819 can be used as a dead air box, connected for a facility exhaust system, or be used with NuAire's HEPA Filter Blower Module to create a Class I Biological Safety Cabinet. Delete if not required.

      - Product: LabGard ES (Energy Saver) NU-819 Bench Top Class I Vented Enclosure.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-819-300. Interior: 34-1/4 inches (870 mm) W by 19-1/4 inches (489 mm) D by 17-3/4 inches (451 mm) H.

        - Model: NU-819-400. Interior: 46-1/4 inches (1175 mm) W by 19-1/4 inches (489 mm) D by 17-3/4 inches (451 mm) H.

        - Recommended Inflow Velocity: Nominal 75 to 100 fpm (0.38 to 0.51 m/s).

** NOTE TO SPECIFIER ** Delete stand height not required.

        - Stand: 30 inches (762 mm) high.

        - Stand: 36 inches (914 mm) high.

** NOTE TO SPECIFIER ** Delete exhaust filter if not required.

        - Exhaust: HEPA Filter Blower Module exhaust filter 99.995% (0.3 micron effective).

** NOTE TO SPECIFIER ** Delete if not required.

        - Low air flow alarm.

** NOTE TO SPECIFIER ** NuAire Class II, Type A2 Biological Safety Cabinets offer personnel, product, and environmental protection to obtain optimum control over product quality while reducing the potential for exposure of both product and personnel to airborne biological or particulate chemical agents in low to moderate risk-hazard research and drug preparation or product operations, Delete if not required.

    - Biological Safety Cabinets Class II Type A2 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER **The CellGard HD 481 Biological Safety Cabinet offers a safer environment for the compounding of hazardous drugs and powders through a three tiered filter system. HEPA filters under the work surface provide additional protection.

      - Product: CellGard ES (Energy Saver) HD (Hazardous Drug) NU-481 Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-481-400. Interior: 46-3/8 inches (1178 mm) W by 26-1/8 inches (664 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-481-500. Interior: 58-3/8 inches (1483 mm) W by 26-1/8 inches (664 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-481-600. Interior: 70-3/8 inches (1788 mm) W by 26-1/8 inches (664 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - Provide Germicidal UV Light w/ Window Interlock.

        - Provide GFI.

        - Provide Additional Duplex Outlets.

        - Provide IV Bar w/ 6 Hooks.

        - Provide Automatic Adjustable Support Stand.

        - Provide Microscope Window.

        - Provide Additional Service Valves.

        - Provide ULPA Filters 99.999% at 0.12 Microns.

        - Provide Storage Pull-Out Trays.

        - Provide Decorative Side Panels.

        - Provide Sinks w/ Hot/Cold or DI Faucets.

        - Provide Heated Work Surface.

        - Provide Seismic Brackets.

        - Provide custom solutions as indicated or required.

** NOTE TO SPECIFIER **The LABGARD ES Energy Saver NU-540 Laminar Airflow Biological Safety Cabinet. An effective primary engineering control in obtaining the optimum product quality while reducing the potential for exposure of both product and personnel to airborne biological or particulate chemical agents in low to moderate risk-hazard research and drug preparation. LabGard ES offers economical operation through the use of an ultra-high efficiency motor to maximize filter life and monolithic silicone-free construction for long product life all for an affordable price.

      - Product: LABGARD ES Energy Saver NU-540 Class II, Type A2 Biosafety Cabinet.

        - Model: NU-540-300. Interior: 34-3/8 inches (873 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540-400. Interior: 46-3/8 inches (1178 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540-500. Interior: 58-3/8 inches (1483 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540-600. Interior: 70-3/8 inches (1788 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - 8 inch (203mm) Access Opening at 105fpm (.53 m/s) Inflow.

        - Ultraviolet Light.

        - LED Lighting.

        - Additional Service Valves for Gas, Air, Vacuum.

        - Additional Duplex Outlet.

        - IV Bar with 6 Stainless Steel Hooks.

        - Exhaust Transition Canopy.

        - Base Support Stand (available as telescoping for work surface heights of 30 to 36 inches) (762 to 914mm) with or without Storage Shelves.

        - Adjustable Control for Support Stand or Storage Cabinet.

        - Sinks with Hot/Cold or DI Water Faucets.

        - Storage Pull Out Trays.

        - Lay in Sorbent Exhaust Filter.

        - Decorative Side Panels (hides plumbing fixture connections).

        - HEPA Filters 99.999% at 0.3 Micron.

        - Arm Rest (Stainless Steel).

        - GFI Outlets.

        - Provide custom solutions as indicated or required.

      - Product: LABGARD ES Energy Saver NU-540E Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-540E-300. Interior: 34-3/8 inches (873 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540E-400. Interior: 46-3/8 inches (1178 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540E-500. Interior: 58-3/8 inches (1483 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-540E-600. Interior: 70-3/8 inches (1788 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - 8 inch (203mm) Access Opening at 105fpm (.53 m/s) Inflow.

        - Ultraviolet Light.

        - LED Lighting.

        - Additional Service Valves for Gas, Air, Vacuum.

        - Additional Duplex Outlet.

        - IV Bar with 6 Stainless Steel Hooks.

        - Exhaust Transition Canopy.

        - Base Support Stand (available as telescoping for work surface heights of 30 to 36 inches) (762 to 914mm) with or without Storage Shelves.

        - Adjustable Control for Support Stand or Storage Cabinet.

        - Sinks with Hot/Cold or DI Water Faucets.

        - Storage Pull Out Trays.

        - Lay in Sorbent Exhaust Filter.

        - Decorative Side Panels (hides plumbing fixture connections).

        - HEPA Filters 99.999% at 0.3 Micron.

        - Arm Rest (Stainless Steel).

        - GFI Outlets.

        - Provide custom solutions as indicated or required.

** NOTE TO SPECIFIER **The LABGARD ES Energy Saver NU-543 Laminar Airflow Biological Safety Cabinet. Offers the very best in personnel, product, and environmental protection through the use of airflow probe to monitor and maintain real time airflow and displays airflow speeds on the OLED interface. The ability to choose up to three different access opening heights allows our customers the option to select a larger access opening to improve technician movement and reach; select a smaller access opening to lower your total cost of ownership; or select somewhere in-between to get the best of both heights.

      - Product: LABGARD ES Energy Saver NU-543 Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-543-300. Interior: 34-3/8 inches (873 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543-400. Interior: 46-3/8 inches (1178 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543-500. Interior: 58-3/8 inches (1483 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543-600. Interior: 70-3/8 inches (1788 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - 8 inch (203mm) Access Opening at 105fpm (.53 m/s) Inflow.

        - 12 inch (305mm) Access Opening at 105fpm (.53mls) (400, 500, 600).

        - Ultraviolet Light.

        - LED Lighting.

        - Additional Service Valves for Gas, Air, Vacuum.

        - Additional Duplex Outlet.

        - IV Bar with 6 Stainless Steel Hooks.

        - Exhaust Transition Canopy.

        - Base Support Stand (available as telescoping for work surface heights of 30 to 36 inches) (762 or 914mm) with or without Storage Shelves.

        - Adjustable Control for Support Stand or Storage Cabinet.

        - Sinks with Hot/Cold or DI Water Faucets.

        - Storage PullOut Trays.

        - Lay in Sorbent Exhaust Filter.

        - Decorative Side Panels (hides plumbing fixture connections).

        - HEPA Filters 99.999% at 0.3 Micron.

        - Arm Rest (Stainless Steel).

        - GFI Outlets.

        - Provide custom solutions as indicated or required.

      - Product: LABGARD ES Energy Saver NU-543E Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-543E-300. Interior: 34-3/8 inches (873 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543E-400. Interior: 46-3/8 inches (1178 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543E-500. Interior: 58-3/8 inches (1483 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-543E-600. Interior: 70-3/8 inches (1788 mm) W by 25-3/4 inches (654 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - 8 inch (203mm) Access Opening at 105fpm (.53 m/s) Inflow.

        - 12 inch (305mm) Access Opening at 105fpm (.53mls) (400, 500, 600).

        - Ultraviolet Light.

        - LED Lighting.

        - Additional Service Valves for Gas, Air, Vacuum.

        - Additional Duplex Outlet.

        - IV Bar with 6 Stainless Steel Hooks.

        - Exhaust Transition Canopy.

        - Base Support Stand (available as telescoping for work surface heights of 30 to 36 inches) (762 or 914mm) with or without Storage Shelves.

        - Adjustable Control for Support Stand or Storage Cabinet.

        - Sinks with Hot/Cold or DI Water Faucets.

        - Storage PullOut Trays.

        - Lay in Sorbent Exhaust Filter.

        - Decorative Side Panels (hides plumbing fixture connections).

        - HEPA Filters 99.999% at 0.3 Micron.

        - Arm Rest (Stainless Steel).

        - GFI Outlets.

        - Provide custom solutions as indicated or required.

** NOTE TO SPECIFIER **The LABGARD ES Energy Saver NU-545 Laminar Airflow Biological Safety Cabinet offers the very best in personnel, product, and environmental protection through the use of airflow probe to monitor and maintain real time airflow and displays airflow speeds on the OLED interface.

      - Product: LABGARD ES Energy Saver NU-545 Class II, Type A2 Biosafety Cabinet.

** NOTE TO SPECIFIER ** Delete model not required.

        - Model: NU-545-300. Interior: 34-3/8 inches (873 mm) W by 26 inches (660 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-545-400. Interior: 46-3/8 inches (1178 mm) W by 26 inches (660 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-545-500. Interior: 58-3/8 inches (1483 mm) W by 26 inches (660 mm) D by 28-1/2 inches (724 mm) H.

        - Model: NU-545-600. Interior: 70-3/8 inches (1788 mm) W by    26inches (660 mm) D by 28-1/2 inches (724 mm) H.

** NOTE TO SPECIFIER ** Delete options not required.

        - 8 inch (203mm) Access Opening @ 105 (.53 m/s) Inflow

        - Ultraviolet Light

        - Additional Service Valves for Gas, Air, Vacuum

        - Additional Duplex Outlet

        - IV Bar with 6 Stainless Steel Hooks

        - Exhaust Transitions - Canopy

        - Base Support Stand (available in standard working    surface heights of 30 or 36 inches) (762 or 914mm) with or without Storage Shelves

        - Adjustable Control for Support Stand or Storage Cabinet

        - Sinks with Hot/Cold or DI Water Faucets

        - Storage Pull-Out Trays

        - Lay in Sorbent Exhaust Filter

        - Decorative Side Panels (hides plumbing fixture connections)

        - HEPA Filters 99.999% @ 0.3 Micron

        - Arm Rest (Stainless Steel)

        - Provide custom solutions as indicated or required.

** NOTE TO SPECIFIER ** Personnel, Product, and Environmental protection provided by a NSF/ANSI listed Class II Biological Safety Cabinet. A large 12 inch (305 mm) access opening accommodates the largest mouse and rat cages. NSF Listed, Three sizes available [4 ft. (1.2m), 5 ft. (1.5m), 6 ft. (1.8m)], Energy Saver Technology.

      - Product: LabGard ES (Energy Saver) NU-677 Class II, Type A2 Animal Handling Biological Safety Cabinet.

** NOTE TO SPECIFIER ** NuAire Class II, Type B1 Biological Safety Cabinets exhausts 70% of airflow. The B1 Biosafety Cabinet may be used for work treated with minute quantities of volatile toxic chemicals and trace amounts of radionuclides required as an adjunct to microbiological studies if work is done in the direct exhausted portion of the cabinet, or if the chemicals or radionuclides will not interfere with the work when recirculated in the downflow air. Delete if not required.

    - Biological Safety Cabinets Class II Type B1 as manufactured by NuAire Laboratory Equipment Supply.

      - Product: LabGard ES (Energy Saver) NU-427 Class II, Type B1 Biological Safety Cabinet.

** NOTE TO SPECIFIER **NuAire Class II, Type B2 Biological Safety Cabinets feature downflow air drawn from within the laboratory or outside air. No downflow air is drawn from the cabinet exhaust air. All downflow and inflow air is exhausted through a HEPA filter without recirculation within the cabinet. All contaminated ducts and plenums are maintained at negative pressure. The Type B2 Biological Safety Cabinet may be used for work with volatile toxic chemicals and radionuclides as required as adjuncts to microbiological studies.

    - Biological Safety Cabinets Class II Type B2 as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER **The LabGard ES 430 Total Exhaust Biological Safety Cabinet may be used for work with non-flammable or explosive volatile chemicals and radionuclides by providing personnel, product, and environmental protection by exhausting 100% of contaminated air.

      - Product: LabGard ES (Energy Saver) NU-430 Class II, Type B2 Biosafety Cabinet.

** NOTE TO SPECIFIER **The LabGard ES 435 Biological Safety Fume Hood is designed to standard NFPA-45 and UL 1805. Exterior outlets, valves, and fire rated HEPA filters ensure proper environment for use of flammable liquids, chemicals, isotopes and radionuclides.

      - Product: LabGard ES (Energy Saver) NU-435 Class II, Type B2 Biosafety Fume Hood.

** NOTE TO SPECIFIER ** NuAire Class III Biosafety Cabinet provides maximum protection of the environment and user when working with highly infectious microbiological agents. Delete if not required.

    - Biological Safety Cabinets Class III as manufactured by NuAire Laboratory Equipment Supply.

** NOTE TO SPECIFIER **The LabGard 704 Class III Glove Box is designed for maximum personnel protection against high-risk pathogenic agents for research applications in BSL-3 and BSL-4 laboratories by creating a vacuum relative to the room, preventing contamination.

      - Product: LabGard ES (Energy Saver) NU-704 Class III Biological Safety Cabinet.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - INSTALLATION

    - Clean surfaces thoroughly prior to installation.

    - Install in accordance with manufacturer's instructions and approved submittals.

    - Test for proper operation and adjust until proper operation is achieved.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION