SECTION 06 11 00

GREEN DOUGLAS-FIR

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2022 -    2022 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Humboldt Sawmill; Douglas-fir Lumber and Timbers
This section is based on the products of Humboldt Sawmill, which is located at:<BR />169 Main St.<BR />Scotia, CA 95565<BR />Tel: 707-764-4472 <BR />Fax: 707-764-4444<BR />Email: request info (marketing@mendoco.com)
Web: http://www.getredwood.com    
 [ Click Here ] for additional information.
Humboldt Sawmill is the leading supplier of Forest Stewardship Council (FSC® C013133) certified redwood and Douglas-fir lumber products; logs are sourced from 440,000 acres of company-owned timberlands in California. An additional 400,000 acres are available under a long-term log purchase agreement. Products manufactured at company-owned mills include redwood decking, timbers, and uppers, as well as Douglas-fir joists and planks, posts. and beams. All products available FSC certified at dealers and distributors throughout the Western U.S. and beyond.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Green Douglas-fir products including the following:

      - Green Douglas-fir, S4S.

      - Green Douglas-fir, Rough.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 06 10 00 - Rough Carpentry.

    - Section 34 11 33.16 - Timber Track Cross Ties.

  - REFERENCES

    - PS 20 - American Softwood Lumber Standard - US Department of Commerce, National Institute of Standards and Technology.

    - ASTM E84: Standard Test Method for Surface Burning Characteristics of Building Materials.

    - California Building Code Chapter 7A: Materials and Construction Methods for Exterior Wildfire Exposure.

    - Forest Stewardship Council (FSC) Certification: Humboldt Sawmill.

      - FSC 100 percent; FSC Mix, FSC Controlled Wood.

        - Certificate Code: SCS-COC-000180 Trademark License Code: FSC C013133.

      - Well Managed Multiple Natural Forest Management Units Certification.

        - Certificate Code: SCS-FM/COC-00120N Trademark License Code: FSC C031337.

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

** NOTE TO SPECIFIER ** Delete the following if LEED certification is not applicable.

    - LEED v4 Submittals: Provide documentation of how the requirements shall be met for the following credits:

      - Building product disclosure and optimization - sourcing of raw materials.

        - Option 2. Leadership extraction practices (1 point). Wood products. Wood products must be certified by the Forest Stewardship Council (FSC) or USGBC-approved equivalent. Products meeting wood products criteria are valued at 100 percent of their cost for the purposes of credit achievement calculation.

** NOTE TO SPECIFIER ** Delete the following if SITES certification is not applicable.

    - SITES v2 Submittals: Provide documentation of how the requirements shall be met for the following credits:

      - Credit 5.7: Support responsible extraction of raw materials.

        - Option 3. Support raw material suppliers and/or manufacturers that meet or exceed standards for raw material extraction (5 points). New Wood Products: Be certified by the Forest Stewardship Council (FSC) (or local equivalent for products outside the United States).

** NOTE TO SPECIFIER ** Delete the following if FSC certification is not applicable.

    - FSC Submittals: Provide documentation indicating manufacturer is FSC Chain-of-Custody certified.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver and store products in manufacturer's unopened packaging bearing the brand name and manufacturer's identification until ready for installation.

    - Handle materials to avoid damage.

    - Store on a level surface with blocking to keep boards off the ground and provide uniform and adequate support.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - SEQUENCING

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Humboldt Sawmill, which is located at:<BR />169 Main St.<BR />Scotia, CA 95565<BR />Tel: 707-764-4472 <BR />Fax: 707-764-4444<BR />Email: request info (marketing@mendoco.com);Web: http://www.getredwood.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Delete article if not required.

  - GREEN DOUGLAS-FIR, S4S

    - Cross Section Size: 2x4.

      - Standard and Better HCTR.

        - Lengths: 8 to 20 ft.

      - Standard and Better.

        - Lengths: 8 to 20 ft.

      - Utility/Economy.

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 2x6.

      - Grade No. 2 and Better HCTR.

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better.

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy.

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 2x8.

      - Grade No. 2 and Better HCTR.

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better.

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy.

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 2x10.

      - Grade No. 2 and Better HCTR.

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better.

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy.

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 2x12.

      - Grade No. 2 and Better HCTR:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 3x6.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft .

    - Cross Section Size: 4x4.

      - Standard and Better, HCTR, Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Standard and Better, Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better, HCTR, Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better, Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Utility / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 4x6.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; HCTR, Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 4x8.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 4x10.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 4x12.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 3 / Economy:

        - Lengths: 6 to 16 ft.

    - Cross Section Size: 6x6.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x8.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x10.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x12.

      - Grade No. 1 and Better; Free of Heart Center, 70 Percent:

        - Lengths: 8 to 20 ft.

      - Grade No. 2 and Better:

        - Lengths: 8 to 20 ft.

** NOTE TO SPECIFIER ** Delete article if not required. Lengths of 22 to 24 ft (6.705 to 7.315 m) available via special order, rough stock only. Delete article if not required.

  - GREEN DOUGLAS-FIR, ROUGH

    - Cross Section Size: 4x6.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 4x8.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x6.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x8.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x10.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 6x12.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 8x8.

      - Grade No. 1 and Better; Free of Heart Center, Full Sawn:

        - Lengths: 8 to 20 ft.

    - Cross Section Size: 1x4.

      - Mill Run. Lengths: 6 to 16 ft.

    - Cross Section Size: 1x6.

      - Mill Run. Lengths: 6 to 16 ft.

    - Cross Section Size: 2x3.

      - Mill Run. Lengths: 6 to 16 ft.

    - Cross Section Size: 2x4.

      - Mill Run. Length: 6 ft.

    - Cross Section Size: 2x6.

      - Mill Run. Length: 6 ft.

  - FASTENERS

** NOTE TO SPECIFIER ** Delete type of fastener not required.

    - Material: Stainless steel.

    - Material: Hot-dip galvanized steel.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Finish products in accordance with manufacturer's instructions.

    - Use only corrosion resistant fasteners. Acceptable are stainless steel or hot-dipped galvanized nails.

    - Joints shall fall over framing lumber and shall be double nailed. Do not nail any less than 1/2 inch from any edge and fasten at a minimum of every 24 inches on center.

    - Drive nails perpendicular to the framing lumber and the wood product; drive nails flush with the product's surface. Nails shall penetrate at least 1-1/4 inches into the structural framing.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION