SECTION 07 25 00

WEATHER BARRIERS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2019 -    2019 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Wolf Home Products; vapor retarders.
This section is based on the products of Wolf Home Products, which is located at:<BR />20 W. Market St.<BR />York, PA 17401<BR />Toll Free Tel: 800-388-9653<BR />Email: request info (JDFoura@wolfhomeproducts.com)
Web: https://www.wolfhomeproducts.com    
 [ Click Here ] for additional information.
Wolf Home Products was founded in 1843 as a single store selling dry goods and lumber on the banks of Pennsylvania's Susquehanna River. During the first half of the 1900s, the company expanded into a mid-Atlantic wholesale and retail lumber operation with 30 locations.
By the early 2000s, Wolf transitioned its successful lumber operation into a sourcing organization, offering kitchen, bath, and building products exclusively to independent building materials dealers.
Wolf Home Products continued its growth with the introduction of Wolf-branded product lines in 2010. Today, the company is committed to product and geographic expansion, servicing 3,500+ dealers nationally and in Canada.

- GENERAL

  - SECTION INCLUDES

    - Vapor retarders.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 06 10 00 - Rough Carpentry.

    - Section 06 15 00 - Wood Decking.

    - Section 07 42 00 - Wall Panels.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM International (ASTM):

      - ASTM D779 - Standard Test Method for Water Resistance of Paper, Paperboard and Other Sheet Materials by the Dry Indicator Method.

      - ASTM D5034 - Standard Test Method for Breaking Strength and Elongation of Textile Fabrics (Grab Test).

      - ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

      - ASTM E96 - Standard Test Methods for Water Vapor Transmission of Materials.

      - ASTM E2178 - Standard Test Method for Air Permeance of Building Materials.

      - ASTM E2273 - Standard Test Method for Determining the Drainage Efficiency of Exterior Insulation and Finish Systems (EIFS) Clad Wall Assemblies.

    - International Code Council (ICC):

      - ICC AC38 - Water Resistive Barriers.

    - Oregon Residential Specialty Code:

      - Section R703.1 for drainage housewrap products.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data:

      - Manufacturer's data sheets on each product to be used.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

** NOTE TO SPECIFIER ** Delete if not applicable to product type.

    - Verification Samples: Two representative units of each type, size, pattern and color.

    - Shop Drawings: Include details of materials, construction and finish. Include relationship with adjacent construction.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up on might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - Intent of mock-up is to demonstrate quality of workmanship and visual appearance.

      - If mock-up is not acceptable, rebuild mock-up until satisfactory results are achieved.

      - Retain mock-up during construction as a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

    - Protect from damage due to weather, excessive temperature, and construction operations.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturer's Warranty: Provide manufacturer's standard limited warranty.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Supplier: Wolf Home Products, which is located at: 20 West Market Street; York, PA 17401; ASD Toll Free: 800-388-9653; Email: _____; Web: www.wolfhomeproducts.com.

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

  - PERFORMANCE REQUIREMENTS

    - Standards Compliance:

      - ICC AC38: ICC-ESR-3641 comply with:

        - UV exposure: Pass.

        - Accelerated aging: Pass.

        - Low temperature bend/pliability: Pass.

      - ASTM E96 for Water Vapor Transmission.

      - ASTM E2178 for Air Permeance.

    - Water Resistance per ASTM D779: Pass 60 minutes as is and aged.

    - Tensile Strength per ASTM D5034:

      - Machine direction: 50 pounds-force (222.4 N).

      - Cross direction: 40 pounds-force (178 N).

    - Drainage Efficiency per ASTM D2273 and Oregon Residential Specialty Code Section R703.1: 98 percent.

    - Surface Burning Characteristics per ASTM E84:

      - Flame spread: Class A.

      - Smoke developed: Class A.

  - VAPOR RETARDERS

    - Basis of Design: BLOCK-IT House Wrap; as supplied by Wolf Home Products.

      - Materials: Microporous film laminated to spunbound polypropylene fabric composed of small denier fibers, including olefin.

      - Overall Weight per ASTM D3776: 3.24 ounce per square yard (110 grams per square meter).

** NOTE TO SPECIFIER ** Contact manufacturer's representative to determine availability of roll sizes suitable for project in your area. Delete options not required.

      - Roll Width: 3 feet (914 mm).

      - Roll Width: 10 feet (3048 mm).

      - Roll Width: _____.

      - Roll Length: 100 feet (30.48 m).

      - Roll Length: 150 feet (45.72 m).

      - Roll Length: _____.

  - ACCESSORIES

    - Mechanical Fasteners: Cap nails or staples in accordance with vapor retarder manufacturer's written recommendations.

    - Sealing Tape: Self-adhesive acrylic seam sealing tape.

    - Flashing Tape: Flashing tape with acrylic or butyl adhesive in accordance with manufacturer's written recommendations.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly constructed and prepared.

    - Verify that substrate and adjacent materials are dry and ready to receive vapor retarder.

    - If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

    - Do not install vapor retarder over saturated substrate.

    - Start at corner and install vapor retarder in shingle fashion with fabric side facing exterior.

      - Ensure vapor retarder continues beyond band joist and extends 2 inches (51 mm) minimum below frame to foundation intersection.

      - Fasten in place using cap nails or staples at 24 inches (610 mm) on center maximum.

    - Ensure fasteners are 4inches minimum away from material edges and 12 inches (305 mm) minimum away from top of doors and windows.

    - Ensure 6 inches minimum overlap at horizontal joint and 12 inches (305 mm) minimum overlap at vertical joints.

    - Install vapor retarder beyond attic and other unoccupied spaces to roofline and under trim.

      - Ensure vapor retarder integrates correctly with flashing.

      - Trim vapor retarder at intersection of wall and roof.

    - Overlap edges 24 inches (610 mm) minimum beyond corner if seams fall within 16 inches (406 mm) of exterior corner of structure.

    - Cut vapor retarder at windows, doors and other penetrations in accordance with manufacturer's written recommendations.

    - Flash around penetrations with flashing tape. Apply flashing tape to vertical edges of vapor retarder flap in accordance with manufacturer's written recommendations.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate field inspection in accordance with appropriate sections in Division 01.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturer's recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

      - Repair small tears less than 6 inches (152 mm) long in vapor retarder with sealing tape.

      - Repair tears longer than 6 inches (152 mm) by cutting flap in vapor retarder 2 inches (51 mm) minimum beyond bottom and sides of tear.

        - Insert wider piece of vapor retarder under flap, shingle style and seal edges.

        - Do not cover repairs with other work before receipt of approval from Architect.

END OF SECTION