SECTION 05 51 00

FIXED METAL LADDERS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2003 -    2003 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER **    Alaco Ladder Company; fixed and folding aluminum wall ladders and ship's ladders.
 .
 This section is based on the products of Alaco Ladder Company, which is located at:
        5167 G Street
        Chino, CA    91710-5143
        Tel: 888-310-7040
        Fax: 909-591-7565
        Email: sales@alacoladder.com
        [ Additional Product Data and Info ]
 .
 Alaco is a manufacturer of special purpose and custom designed ladders to fit a variety of residential, commercial, and industrial uses. Established over 100 years ago, Alaco engineers and manufactures a wide array of ladders for numerous applications. Ladders are Alaco's only business, and they are the experts in the field.
 .

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER **    Delete items below not required for project.

    - Fixed aluminum wall ladders.

    - Fixed aluminum ship's ladders.

    - Folding aluminum wall ladders.

    - Retractable aluminum wall ladders.

    - Fasteners and installation accessories.

  - RELATED SECTIONS

** NOTE TO SPECIFIER **    Delete any sections below not relevant to this project; add others as required.

    - Section 05 50 00 - Metal Fabrications: Catwalks and fire escapes.

    - Section 06 10 00 - Rough Carpentry: Blocking in metal wall studs and partitions for anchorage of access ladders

    - Section 06 43 50 - Rolling Library Ladders.

  - REFERENCES

** NOTE TO SPECIFIER **    Delete references from the list below that are not actually required by the text of the edited section.

    - ANSI A14.3 - American National Standard for Ladders -- Fixed -- Safety Requirements; 1992.

    - ASTM B 209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate; 2001.

    - ASTM B 209M - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate (Metric); 2001.

    - ASTM B 210 - Standard Specification for Aluminum and Aluminum-Alloy Drawn Seamless Tubes; 2002.

    - ASTM B 221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes; 2000.

    - ASTM B 221M - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes (Metric); 2000.

    - ASTM B 308 - Standard Specification for Aluminum - Alloy T6061-T6 Standard Structural; 2002

    - ASTM B 308M - Standard Specification for Aluminum - Alloy T6061-T6 Standard Structural; 2002

    - OSHA 29 CFR Standard 1910.27 - Fixed ladders;    Occupational Safety and Health Standards; current edition

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

    - Product Data:    Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Detailed drawings showing complete dimensions, all materials, mounting attachments, and fabrication details.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in the engineering and manufacturing of metal ladders, with not less than fifty years of experience.

  - WARRANTY

    - See Section 01 78 00 - Closeout Submittals, for additional warranty requirements.

    - Provide manufacturer's standard limited five-year warranty against defects in materials and workmanship.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: ALACO Ladder Co., which is located at:<BR />5167 G St.<BR />Chino, CA 91710-5143<BR />Toll Free Tel: 888-310-7040<BR />Tel: 909-591-7561<BR />Fax: 909-591-7565<BR />Email: request info (sales@alacoladder.com);Web: http://alacoladder.com

** NOTE TO SPECIFIER **    Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions:    Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00.

  - MATERIALS

    - Extruded Aluminum Profiles: ASTM B 221/B 221M, ASTM B 210, ASTM B 308/B 308M, Alloy 6061-T6; standard mill finish.

    - Aluminum Sheet and Plate: ASTM B 209/B 209M, Alloy 6061-T6; standard mill finish.

    - Fasteners: Aluminum solid aircraft rivets rated at 300 lbs (1335 N) shear strength.

    - Cast fittings, connectors and rung ends: Cast Aluminum alloy 356

  - LADDERS

    - Ladders - General: Comply with ANSI A14.3 and OSHA regulations.

    - Fixed Wall Ladders: Extruded aluminum; serrated rungs 1-1/8 inches (29 mm) in diameter, connected to 2-7/8 inch (73 mm) side rail channels with cast aluminum rung connectors, each secured to rails by means of four solid aircraft rivets.

** NOTE TO SPECIFIER **    Delete the following paragraph if not required.

      - Walk-Through and Parapet Railings: Aluminum extrusions; extend not less than 42 inches (1,067 mm) above landing, 24 in (610 mm) between side rails at step through.

** NOTE TO SPECIFIER **    The following paragraph is for ladders over 20 feet in height.    Delete if not required.

      - Cage: Aluminum hoops 1/4 by 2 inches (6 by 51 mm) and seven aluminum vertical bars 3/16 by 1-1/2 inches (5 by 38 mm) riveted rigidly together.

** NOTE TO SPECIFIER **    The following paragraph is for ladders over 30 feet in height; platforms are required at 30 ft intervals.    Delete if not required.

      - Rest Platforms: Aluminum Grip Strut grating and aluminum toe boards 0.63 in (1.6 mm) thick by 4 inches (102 mm) wide; railings of 1-1/4 inch (32 mm) round serrated tube with cast aluminum railing fittings.

** NOTE TO SPECIFIER **    The following paragraph is for roof access ladders in locations that are accessible by the public.    Delete if not required.

      - Security Doors: Form from aluminum sheet 0.188 inch (4.8 mm) in thickness; secure with stainless steel piano hinges and hasps.

** NOTE TO SPECIFIER **    The following paragraph is for roof hatch access ladders.    Delete if not required.

      - Hatch Access:    Provide ladder side rail with rubber end caps and end with sufficient clearance for hatch to properly close.

    - Ship's Ladders: 24 inches (610 mm) wide.

** NOTE TO SPECIFIER **    Delete one of the following two capacity paragraphs.

      - Capacity: 500 lbs (225 kg).

** NOTE TO SPECIFIER **    Delete two of the following three paragraphs.

        - Slope: 60 degrees; 6-inch (152 mm) wide aluminum steps mounted on 10-13/32 inches (264 mm) centers to 5-1/4 inch (133 mm) aluminum side rails.

        - Slope: 70 degrees;

** NOTE TO SPECIFIER **    Delete one of the following two paragraphs.

          - For Ladders Less Than 10 Feet: 3-inch (76 mm) wide aluminum steps mounted on 12 inch (305 mm) centers to 2-7/8 inch (73 mm) aluminum side rails.

          - For Ladders 10 Feet and Greater: 4-inch (102 mm) wide aluminum steps mounted on 12 inch (305 mm) centers to 4-1/8 inch (105 mm) aluminum side rails.

        - Slope: 80 degrees; 3-inch (76 mm) wide aluminum steps mounted on 12 inch (305 mm) centers to 2-7/8 inch (73 mm) aluminum side rails.

      - Capacity: 1000 lbs (450 kg).

** NOTE TO SPECIFIER **    Delete three of the following four paragraphs.

        - Slope: 60 degrees; 6-inch (152 mm) wide aluminum steps mounted on 12 inch (264 mm) centers to 5-5/8 inch (133 mm) aluminum side rails.

        - Slope: 65 degrees; 6-inch (152 mm) wide aluminum steps mounted on 12 inch (264 mm) centers to 5-5/8 inch (133 mm) aluminum side rails.

        - Slope: 70 degrees; 6-inch (152 mm) wide aluminum steps mounted on 12 inch (264 mm) centers to 5-5/8 inch (133 mm) aluminum side rails.

        - Slope: 75 degrees; 6-inch (152 mm) wide aluminum steps mounted on 12 inch (264 mm) centers to 5-5/8 inch (133 mm) aluminum side rails.

      - Fasten each step with not less than eight aluminum aircraft rivets.

      - Provide handrails of 1-1/4 in (32 mm) round serrated aluminum tubing with cast or extruded aluminum fittings.

      - Furnish ladder with four mounting brackets.

    - Folding Wall Ladders: 24 inches (610 mm) wide.

** NOTE TO SPECIFIER **    Delete three of the following four paragraphs.

      - Slope: 60 degrees; 6-inch (152 mm) wide aluminum nonslip steps mounted on 10-13/32 inches (264 mm) centers to 5-1/4 inch (133 mm) aluminum side rails.

      - Slope for Ladders Less than 10 Feet: 70 degrees; 3-inch (76 mm) wide aluminum steps mounted on 12-inch (305 mm) centers to 2-7/8 inch (73 mm) aluminum side rails.

      - Slope for Ladders 10 feet and Greater: 70 degrees; 4-inch (102 mm) wide aluminum steps mounted on 12 inch (305 mm) centers to 4-1/8 in (105 mm) aluminum side rails.

      - Slope: 80 degrees; 3-inch (76 mm) wide aluminum steps mounted on 12 inch (305 mm) centers to 2-7/8 inch (73 mm) aluminum side rails.

** NOTE TO SPECIFIER **    Handrails are optional for folding ladders; delete the following paragraph if not required.

      - Provide handrails of 1-1/4 in (32 mm) round serrated aluminum tubing with cast or extruded aluminum fittings and four mounting brackets.

      - Fasten each step with not less than eight aluminum aircraft rivets.

      - Furnish with mounting bracket-slide assemblies to allow flat storage against wall and non-marking solid rubber feet for secure installation.

    - Retractable Wall Ladders:

      - Extruded aluminum profiles.

      - Serrated rungs 1-1/8 inches (29 mm) in diameter, connected to 2-78 inch (73 mm) side rail channels with cast aluminum rung connectors, each secured to rails by means of four solid aircraft rivets.

      - Furnish with retractable mounting bracket assemblies for flat storage against the wall, non-marking solid rubber feet, and rubber plugs to provide closed end protection at top of side rails.

  - FINISHES

** NOTE TO SPECIFIER **    Mill finish aluminum is standard. Custom coatings and surface treatments are available by special order.

    - Provide all aluminum in standard mill finish.

    - Factory finish all aluminum surfaces with manufacturer's standard powder coating system.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions and approved shop drawings, and in compliance with ANSI A14.3 and OSHA 1910.27.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION