SECTION 12 24 00

WINDOW SHADES

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2008 -    2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Draper Inc.; window shade products.
This section is based on the products of Draper Inc., which is located at:<BR />411 S. Pearl Street, P. O. Box 425<BR />Spiceland, IN 47385-0425<BR />Toll Free Tel: 800-238-7999<BR />Tel: 765-987-7999<BR />Fax: 866-637-5611<BR />Email: request info (drapercontract@draperinc.com)
Web: http://www.draperinc.com    
 [ Click Here ] for additional information.
Draper is a leader in the manufacture of window shades, projection screens, and gymnasium equipment. Founded in 1902, Draper maintains a tradition of quality and service. Draper products include FlexShade roller-operated and fixed window shades.
This section can be used for specifying both manually operated and motorized Draper Flexshade window shades and LightBloc FlexShade opaque fabric window shades for complete blackout of window opening
Chain and Clutch Operated FlexShade
Clutch-Operated FlexShade NEXD
Bottom-Up FlexShade
Dual Roller FlexShade
Motorized FlexShade
LightBloc FlexShade

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Manually operated, roll-up fabric interior window shades including mounting and operating hardware.

    - Motorized, roll-up fabric interior window shades including motor operator, controls, and mounting hardware.

    - Opaque window shade system: Manually operated, roll-up fabric opaque window shade system for complete blackout of window opening including side and bottom channels, headbox, manual operator, and mounting hardware.

    - Opaque window shade system: Motorized, roll-up fabric opaque window shade system for complete blackout of window opening including side and bottom channels, headbox, motorized operator, controls, and mounting hardware.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 06 10 00 - Rough Carpentry.

    - Section 07 90 00 - Joint Sealers.

    - Section 09 21 16.33 - Gypsum Board Area Separation Wall Assemblies.

    - Section 09 51 23 - Acoustical Tile Ceilings.

    - Section 11 52 13.50 - Motorized Projection Screens.

    - Section 11 52 13.50 - Motorized Projector Lifts.

    - Division 16 [26] - Electrical.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - NFPA 70 - National Electrical Code.

    - NFPA 701-99 - Fire Tests for Flame-Resistant Textiles and Films.

    - GREENGUARD Environmental Institute Gold.

    - US Green Building Council.

    - ANSI/WCMA A100.1-2018

  - SUBMITTALS

    - Submit under provisions of Section 01 33 13 - Certificates [01 33 26] - Submittal Procedures:

    - Product Data: Manufacturer's data sheets on each product specified, including:

      - Preparation instructions and recommendations.

      - Installation and maintenance instructions.

      - Styles, material descriptions, dimensions of individual components, profiles, features, finishes and operating instructions.

      - NFPA Flame-Test: Passes NFPA 701 including certification from manufacturer that the fabrics sourced for this project comply with the test data provided.

      - Storage and handling requirements and recommendations.

      - Mounting details and installation methods.

** NOTE TO SPECIFIER ** Delete the following paragraph for manual operation shades.

      - Typical wiring diagrams including integration of motor controllers with building management system, audiovisual and lighting control systems as applicable.

** NOTE TO SPECIFIER ** Include the following paragraph for projects with complex requirements for window treatment. Delete if not required.

    - Shop Drawings: Plans, elevations, sections, product details, installation details, operational clearances, wiring diagrams and relationship to adjacent work.

** NOTE TO SPECIFIER ** Delete the following paragraphs if LEED is not applicable.

    - LEED Submittals: Provide documentation of how the requirements of Credit will be met.

    - Window Treatment Schedule: For all roller shades. Use same room designations as indicated on the Drawings, field verified window dimensions, quantities, type of shade, controls, fabric, and color, and include opening sizes and key to typical mounting details.

** NOTE TO SPECIFIER ** Delete Selection Samples paragraph if colors and shade fabrics have already been selected.

    - Selection Samples: For each finish product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

    - Verification Samples: For each finish product specified, two complete sets of shade components, unassembled, demonstrating compliance with specified requirements. Shade fabric sample and aluminum finish sample as selected, representing actual product, color, and patterns. Mark face of material to indicate interior faces.

    - Maintenance Data: Methods for maintaining roller shades, precautions regarding cleaning materials and methods, instructions for operating hardware and controls.

    - Manufacturer's Certificates: Certify products meet or exceed specified requirements.

    - Warranty: Properly executed manufacturers warranty.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Obtain roller shades through one source from a single manufacturer with a minimum of twenty years documented experience in manufacturing products comparable to those specified in this section. Manufacturers that do not meet the required experience requirements must submit life cycle test data showing minimum 2000 complete operational cycles for each year of warranty showing no failure and that shade remains fit for use as a operable shade).

** NOTE TO SPECIFIER ** Include the following paragraph for shades requiring UL Listing. Delete if not applicable.

    - NFPA Flame-Test: Passes NFPA 701. Materials tested shall be identical to products proposed for use.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Provide a mock-up of one of each type roller shade assembly specified for evaluation of mounting, appearance and accessories.

      - Locate mock-up in window(s) designated by Architect.

      - Do not proceed with remaining work until mock-up is accepted by Architect.

  - DELIVERY, STORAGE, AND HANDLING

    - Do not deliver window shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

    - Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

    - Label containers and shades according to Window Shade Schedule.

    - Store products in manufacturer's unopened packaging until ready for installation.

  - SEQUENCING

    - Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

  - PROJECT CONDITIONS

    - Install roller shades after finish work and ambient temperature, humidity and ventilation conditions are maintained at levels recommended for project upon completion.

  - WARRANTY

    - Hardware and Shade Fabric: Draper standard twenty-five year limited warranty.

** NOTE TO SPECIFIER ** Delete the following paragraph for manual operation.

    - Motors and Controls: Draper standard five year limited warranty.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Draper Inc., which is located at:<BR />411 S. Pearl Street, P. O. Box 425<BR />Spiceland, IN 47385-0425<BR />Toll Free Tel: 800-238-7999<BR />Tel: 765-987-7999<BR />Fax: 866-637-5611<BR />Email: request info (drapercontract@draperinc.com);Web: http://www.draperinc.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements (01 60 00].

** NOTE TO SPECIFIER ** Select the shades required from the following paragraphs. For each Draper window shade there are options for size, fabric, operator, operator location, controls, method of installation, hardware, and hardware finish. Refer to Draper product literature for availability of options and selection guidelines. Various options are listed in this specification section, but not all options apply to all window shades. The specifier will need to select the appropriate options, verify their availability, and delete non-applicable models and options.

  - MANUALLY OPERATED WINDOW SHADES

** NOTE TO SPECIFIER ** Edit the following paragraphs to specify manually operated Draper Chain and Clutch Operated FlexShade CL, Draper Chain and Clutch Operated FlexShade NEXD, Bottom-Up Clutch-Operated FlexShade NEXD, and Dual Roller Clutch-Operated FlexShade NEXD. Delete those not required.

    - Manually Operated Window Shades with Independent Control: Manually operated, vertical roll-up, fabric window shade with components necessary for complete installation; Clutch-Operated FlexShade CLN as manufactured by Draper, Inc.

** NOTE TO SPECIFIER ** Select Bead Chain or Clutch operation type for window shade from the following options, delete the one not applicable.

      - Operation: Bead chain and clutch operating mechanism allowing shade to stop when chain is released. Designed never to need adjustment or lubrication. Provide limit stops to prevent shade from being raised or lowered too far.

        - Clutch mechanism: Fabricated from high carbon steel and molded fiberglass reinforced polyester or injected molded nylon. Minimum 20 lb. lifting capacity. White or Black color as selected by Architect.

        - Spring-Assist Clutch: Adjustment-free system includes spring-assist components to reduce lifting forces required to raise the shade. Manufacturer shall provide estimated torque for shade unit. Spring-assist is recommended on estimated non-spring-assist torque above 6 lb-in; required on shades with an estimated torque higher than 15 lb-in.

** NOTE TO SPECIFIER ** Select stainless steel (standard) or optional plastic bead chain from the following two paragraphs and delete the one not required.

        - Bead chain loop: Stainless steel bead chain hanging at side of window.

        - Bead chain loop: Polyester bead chain hanging at side of window, Ivory, Grey, White, Bronze or Black color as selected by Architect.

** NOTE TO SPECIFIER ** Select Bead Chain Hold Down required from the following two paragraphs.

        - Bead Chain Hold Down: P-Clip (standard).

        - Bead Chain Hold Down: Spring-Loaded Tensioner complies with ANSI/WCMA A100.1-2018 safety standard.

        - Idler Assembly: Provide roller idler assembly of molded nylon with adjustable or spring-loaded length idler pin to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER ** Select the mounting type required for window shades from the following options, delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section. Manufacturer chooses hardware size based on fabric size, weight, and thickness and window opening size.

      - Single Roller Configuration:

        - Mounting:

          - Universal Mounting brackets.

          - Endcaps only.

          - Endcaps and fascia.

          - Ceiling/Wall Style Headbox.

          - Recessed Pocket Headbox.

** NOTE TO SPECIFIER ** Shades provided without endcaps will require mounting brackets. Brackets are not used with Fascia or Headbox. Delete if not required.

        - Brackets: Plated stamped steel. Provide size compatible with roller size.

** NOTE TO SPECIFIER ** Select bracket mounting type and finish required from the following and delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section.

          - Mounted to ceiling.

          - Mounted to wall.

          - Mounted to jamb.

          - Finish: White.

          - Finish: Black.

** NOTE TO SPECIFIER ** Include endcaps if required and delete if not required. Endcaps are optional for Chain and Clutch Operated FlexShades. Endcaps are also required for attachment of fascia and headbox.

        - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

          - Endcap covers: To match fascia or headbox color.

** NOTE TO SPECIFIER ** Draper manually operated shades can be provided with aluminum fascia concealing shade roller and hardware. Fascia can be mounted inside window opening extending from jamb to jamb or mounted outside and overlapping window opening. Fascia is attached to endcaps. Fascia can be provided in lengths up to 16 feet and either 3-7/8 or 4-5/8 inches high. Fascia available for reverse roll applications. Include the following paragraph if fascia is required. Delete if not required.

        - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

          - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands. No notching is required.

          - Shape: Square Fascia Panel.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Draper manually operated shades can be provided with aluminum headbox designed for exposed surface or recessed ceiling installation. Ceiling/Wall style headbox consists of removable closure, back and top, and endcaps. Pocket style headbox consists of removable closure, U-Shaped pocket and endcaps. Headbox is provided in two sizes. Refer to Draper product literature for required sizes based on window and roller size. Include one of the following paragraphs if headbox is required. Delete if not required.

        - Headbox Ceiling/Wall style: Aluminum fabrication with removable closure, endcaps, and back and top cover piece:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Headbox, Pocket style: Aluminum fabrication with removable closure, endcaps, and U-shaped pocket:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range

          - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Chain and Clutch Operated FlexShades can be provided with and installed in fabricated pockets. Type D Pockets are designed to be installed prior to or as part of ceiling system installation. Window shade assembly can be site installed later after construction operations that might damage shade are complete. Include the appropriate following paragraph if Type D Pockets are required. Delete if not required.

        - Type D Shade pocket: Rectangular pocket and endcaps designed for recessed ceiling installation of window shades with ceiling tile lip.

          - Material: Extruded aluminum alloy or steel with white finish.

          - Size: 5 inches (127 mm) wide by 5-3/8 inches (137 mm) high.

          - Closure Panel:

** NOTE TO SPECIFIER ** Closure panel is optional. Delete if not required.

            - 1-1/2 (38 mm) closure dimension, recommended for bead chain operation.

** NOTE TO SPECIFIER ** Endcaps kit with acoustical ceiling panel support lip is optional. Select if required or delete if not required.

          - Pocket End Cap Kit: Metal endcaps with 7/8 inch (22 mm) lip for support of acoustical ceiling panel.

** NOTE TO SPECIFIER ** Type D FlexShade Pocket can be provided with corner sections where window shade recess continues around two walls. Pocket ends can also be provided. Choose all that apply.

          - Corners: Welded one-piece aluminum sections connecting to and matching pockets to allow continuous shade recess at ceiling corners.

** NOTE TO SPECIFIER ** Draper manually operated window shades can be installed in a ceiling recess site constructed from light gage metal framing and gypsum board or other materials. For this condition, use the following paragraph to specify aluminum removable closure panel to be installed leaving a minimal slot for passage of shade. Delete if not required.

        - Wall Clip with Closure Panel: For site constructed ceiling recesses, provide removable closure panel to minimize slot for shade passage but allowing access to shade for maintenance.

          - Material: Aluminum alloy with white epoxy paint finish.

** NOTE TO SPECIFIER ** Delete the following paragraph if tile lip option is not required.

          - Tile Lip: Provide wall clip with 7/8 inch tile lip (22 mm).

** NOTE TO SPECIFIER ** Select closure width required from the following paragraphs.

          - Closure width: 1-1/2 inches (38 mm).

          - Closure width: 3 inches (76 mm).

          - Closure Width: 5 inches (127 mm).

          - Provide continuous wall clip, 1-3/4 (44 mm) by 3/16 inch (5 mm), for snap-in attachment of closure panel without fasteners.

** NOTE TO SPECIFIER **    Include the following paragraphs for Draper Motorized FlexShades installed with light reduction channels. Delete if not required.

        - Aluminum Light Gap Reduction Channels:

** NOTE TO SPECIFIER **    Select channel required from the following

          - L Angle - 3/4 inch by 1 inch (19 mm by 25 mm).

          - L Angle - 1 inch by 2-3/4 inches (25 mm by 70 mm).

          - U Channel - 1 inch by 2-1/2 inches (25 mm by 64 mm).

          - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

        - Vinyl Light Gap Reduction Channels:

          - L Angle - 1-1/2 inches 3/4 inches (38 mm by 19 mm).

        - Interior Cable Guide Kit:

          - Interior cable hem bar finial.

          - Cable mount.

          - Cable.

          - Tension Stop.

** NOTE TO SPECIFIER ** Select one of the following mounting paragraphs and delete the paragraphs not required. . If multiple types are required indicate the location of each in the schedule at the end of this section. Manufacturer chooses hardware size based on fabric size, weight, and thickness and window opening size

      - Dual Roller Configuration / Mounting:

        - Dual roller endcaps only. Endcaps for surface or recessed mounting of dual roller window shades. 1028 steel stamping.

        - Dual roller fascia. Endcaps with fascia designed for surface mounting of dual roller window shades.

          - Endcaps: 1028 steel stamping.

          - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

          - Size: Determined by manufacturer based on fabric size, weight, and thickness and window opening size.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Dual roller with pocket headbox. Pocket and endcaps designed for recessed ceiling installation of dual roller window shades.

          - Material: 18 gauge steel.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: White Inside/Grey Outside with white ceiling tile flange.

          - Finish: Grey Inside/White Outside with white ceiling tile flange.

          - Closure Panel Aluminum, without exposed fasteners:

** NOTE TO SPECIFIER ** Select the flange required from the following paragraphs and delete those not required.

            - Front Tile Flange.

            - Left Tile Flange.

            - Right Tile Flange.

            - Pre-drilled Mounting Holes.

            - Pre-drilled Ventilation holes.

        - Dual roller with ceiling/wall headbox. Fascia, L-shaped top/back, and endcaps designed for wall or ceiling installation of dual roller window shades.

          - Endcaps: 1028 steel stamping.

          - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

          - Finish: Powder coat.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

            - Selected from Manufacturers standard range.

            - Custom powder coat as selected by Design Professional

        - Light Gap Reduction Channels.

** NOTE TO SPECIFIER **    Select channel size required from the following paragraphs.

          - Aluminum L Angle - 3/4 inch (19 mm) by 1 inch (25 mm).

          - Aluminum L Angle -1 inch (25 mm) by 2-3/4 inches (70 mm).

          - Vinyl L Angle-1-1/2 inches (38 mm) by 3/4 inch (19 mm).

          - U Channel -1 inch (25 mm) by 2-1/2 inches (64 mm)

          - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

      - Roller Tube: Fabricated from extruded aluminum, galvanized steel, or enameled steel. Diameter, wall thickness, and material selected by manufacturer to accommodate shade type and size. Minimum roller diameter 1.5 inches. Tube diameters less than 1.5 inches shall not be acceptable unless manufacturer provides deflection analysis showing deflection limited to <= width (inches) /700 at 1.5X design load.

      - Fabric connected to the roller tube with LSE (low surface energy) double sided adhesive specifically developed to attach coated textiles to metal. Adhesive attachment to eliminate horizontal impressions in fabric.

      - Shade slat:

** NOTE TO SPECIFIER ** Select the shade slat required from the following paragraphs and delete those not required

        - Closed pocket elliptical slat: 1 inch (25 mm) aluminum elliptical slat inside of a 1-5/8 inch (41 mm) pocket with heat sealed ends.

        - Open pocket elliptical slat: 1 inch (25mm) aluminum elliptical slat with plastic ends inside of a 1-5/8 inch (41 mm) pocket.

        - Flat exposed hem bar: Small: 7/8 inch by 5/16 inch (22 mm by 8 mm) and Large 1-1/2 inch by 5/16 inch (38 mm by 8 mm) aluminum rectangular hem bar with plastic end caps.

        - Round exposed hem bar: Small 5/8 inch (16 mm) and Large 13/16 inch (21 mm) aluminum with plastic end caps.

    - Heavy-Duty Manually Operated Window Shades with Independent Control: Manually operated, vertical roll-up, fabric window shade with components necessary for complete installation; Clutch-Operated FlexShade NEXD as manufactured by Draper, Inc.

      - Operation: Bead chain and clutch operating mechanism allowing shade to stop when chain is released. Designed never to need adjustment or lubrication. Provide limit stops to prevent shade from being raised or lowered too far.

        - Spring-Assist Clutch: Adjustment-free system includes spring-assist components to reduce lifting forces required to raise the shade. Manufacturer shall provide estimated torque for shade unit. Spring-assist is recommended on estimated non-spring-assist torque above 6 lb-in; required on shades with an estimated torque higher than 15 lb-in.

** NOTE TO SPECIFIER ** Select stainless steel or optional plastic bead chain from the following two paragraphs and delete the one not required.

        - Bead chain loop: Stainless steel bead chain.

        - Bead chain loop: Polyester bead chain, Ivory, Grey, White, Bronze or Black color as selected by Architect.

** NOTE TO SPECIFIER ** Select Bead Chain Hold Down required from the following paragraphs. Select Spring Loaded Tensioner is shades are specified to be ANSI/WCMA A100.1 compliant.

        - Bead Chain Hold Down: P-Clip.

        - Bead Chain Hold Down: Spring-Loaded Tensioner complying with ANSI/WCMA A100.1-2018 safety standard.

        - Idler end: Height adjustable idler end allows fine leveling adjustments after installation-min plus or minus 1/8 inch without shimming brackets. Contains at least two entry points for the idler end. Safety engagement feature requires idler end pin to have a minimum engagement in bracket, ensuring that the idler end cannot fall out of the bracket due to lack of pin engagement.

** NOTE TO SPECIFIER ** FlexShade NEXD is available in single roller, dual roller, Bottom-Up single roller, and Bottom-Up dual roller configurations. Select configuration required, and delete those not required.

      - Single Roller Configuration:

** NOTE TO SPECIFIER ** Select the mounting type required from the following options, delete those not applicable. . If multiple types are required indicate the location of each in the schedule at the end of this section. Manufacturer chooses hardware size based on fabric size, weight, and thickness and window opening size.

        - Mounting:

          - Endcaps only.

          - Endcaps and fascia.

          - Endcaps and headbox.

          - Cassette Headbox.

            - Complete Cassette.

            - Back only.

            - Top only.

            - Top and back only

          - Cassette Style with Fascia.

            - Back with Fascia.

            - Top with Fascia.

            - Top and back with Fascia.

        - Endcaps: 1018 stamped steel. Suitable for mounting to ceiling, wall, and jamb. Field adjustable from ceiling to wall. No "L" angle required for wall mounting. Contains at least two entry points for the idler end.

** NOTE TO SPECIFIER ** Select endcap mounting type required from the following and delete those not applicable.

          - Endcap covers: To match fascia or headbox color.

          - Mounted to ceiling.

          - Mounted to wall.

          - Mounted to jamb.

** NOTE TO SPECIFIER ** Draper Bottom-Up NEXD shades can be provided with aluminum fascia concealing shade roller and hardware. Fascia can be mounted inside window opening extending from jamb to jamb or mounted outside and overlapping window opening. Fascia is attached to endcaps. Fascia can be provided in lengths up to 16 feet and either 3-7/8 or 4-5/8 inches high. Fascia available for reverse roll applications. Include the following paragraph if fascia is required. Delete if not required.

        - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

          - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands. Notching of fascia to provide for chain clearance is NOT acceptable. Fascia height to match throughout space unless specifically approved in advance by the Architect

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Draper NEXD manually operated shades can be provided with aluminum headbox designed for exposed surface or recessed ceiling installation. Ceiling/Wall style headbox consists of removable closure, back and top, and endcaps. Pocket style headbox consists of removable closure, U-Shaped pocket and endcaps. Headbox is provided in two sizes. Refer to Draper product literature for required sizes based on window and roller size. Include one of the following paragraphs if headbox is required. Delete if not required.

        - Headbox Ceiling/Wall style: Aluminum fabrication with a top/back cover and fascia with endcaps for mounting:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Headbox, Pocket style: Aluminum fabrication with removable closure, endcaps, and U-shaped pocket:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Chain and Clutch Operated FlexShades can be provided with and installed in fabricated pockets. Window shade assembly can be site installed later after construction operations that might damage shade are complete. Delete if not required.

        - Type D Shade pocket: Rectangular pocket with ceiling tile lip designed for recessed ceiling installation of window shades.

          - Material: Extruded aluminum alloy with white epoxy paint finish.

          - Size: 5 inches (127 mm) by 5-3/8 inches (137 mm) high.

          - Closure Panel:

** NOTE TO SPECIFIER ** Closure panel is optional select the one required or delete if not required. Please note the 3 inch closure requires notching before installation if used with FlexShade NEXD.

            - 1-1/2 (38 mm) closure dimension.

            - 3 inch (78 mm)

** NOTE TO SPECIFIER ** Endcaps for Type D FlexShade Pocket is provided with an acoustical ceiling panel support lip.

          - Pocket Endcap Kit: Endcaps with 7/8 inch (22 mm) lip for support of acoustical ceiling panel.

** NOTE TO SPECIFIER ** Type D FlexShade Pocket can be provided with corner sections where window shade recess continues around two walls. Pocket ends can also be provided. Choose all that apply.

          - Corners: Welded one-piece aluminum sections connecting to and matching pockets to allow continuous shade recess at ceiling corners.

          - Pocket ends: Welded one-piece aluminum sections connecting to and matching pockets.

** NOTE TO SPECIFIER ** Draper Motorized FlexShades can be installed in ceiling recess site constructed from light gauge metal framing and gypsum board or other materials. For this condition, use the following paragraph and subparagraphs to specify aluminum removable closure panel to be installed leaving a minimal slot for passage of shade. Delete if not required.

        - Wall Clip with Closure panel: For site constructed ceiling recesses, provide removable closure panel to minimize slot for shade passage but allowing access to shade for maintenance.

          - Material: Aluminum alloy with white epoxy paint finish.

** NOTE TO SPECIFIER ** Delete the following paragraph if tile lip option is not required.

          - Tile Lip: Provide wall clip with 7/8 inch tile lip (22 mm).

** NOTE TO SPECIFIER ** Select closure width required from the following paragraphs.

          - Closure width: 1-1/2 inches (38 mm).

          - Closure width: 3 inches (75 mm).

          - Closure Width: 5 inches (127 mm).

          - Provide continuous wall clip, 1-3/4 (44 mm) by 3/16 inch (5 mm), for snap-in attachment of closure panel without fasteners.

        - Cassette Headbox Fully assembled. Standard consists of fascia and two interlocking extrusions that form a back/top cover. Suitable for pocket installation. Size: 3-3/8 inch by 3-3/8 inch (86 mm by 86 mm):

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Cassette Style with Fascia. Standard consists of fascia and two interlocking extrusions that form a back/top cover. Size: 3-3/8 inch by 3-3/8 inch (86 mm by 86 mm):

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Roller Tube: Fabricated from extruded aluminum, galvanized steel, or enameled steel. Diameter, wall thickness, and material selected by manufacturer to accommodate shade type and size. Minimum roller diameter 1.5 inches. Tube diameters less than 1.5 inches shall not be acceptable unless manufacturer provides deflection analysis showing deflection limited to <= width (inches) /700 at 1.5X design load.

          - Fabric to tube attachments: Spline fabric/roller attachment system to allow shade fabric to be removed from roller without having to remove roller from brackets.

        - Shade slat:

** NOTE TO SPECIFIER ** Select the shade slat required from the following paragraphs and delete those not required

          - Closed pocket elliptical slat: 1 inch (25 mm) aluminum elliptical slat inside of a 1-5/8 inch (41 mm) pocket with heat sealed ends.

          - Open pocket elliptical slat: 1 inch (25mm) aluminum elliptical slat with plastic ends inside of a 1-5/8 inch (41 mm) pocket.

          - Flat exposed hem bar: Small: 7/8 inch by 5/16 inch (22 mm by 8 mm) and Large 1-1/2 inch by 5/16 inch (38 mm by 8 mm) aluminum rectangular hem bar with plastic end caps.

          - Round exposed hem bar: Small 5/8 inch (16 mm) and Large 13/16 inch (21 mm) aluminum with plastic end caps.

** NOTE TO SPECIFIER ** Include the following paragraphs for Draper Motorized FlexShades installed with light reduction channels. Delete if not required.

        - Aluminum Light Gap Reduction Channels

** NOTE TO SPECIFIER ** Select channel required from the following

          - L Angle - 3/4 inch by 1 inch (19 mm by 25 mm).

          - L Angle - 1 inch by 2-3/4 inches (25 mm by 70 mm).

          - U Channel - 1 inch by 2-1/2 inches (25 mm by 64 mm).

          - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

        - Vinyl Light Gap Reduction Channels

          - L Angle - 1-1/2 inches 3/4 inches (38 mm by 19 mm).

        - Interior cable guide kit

          - Interior cable hem bar finial.

          - Cable mount.

          - Cable.

          - Tension Stop

** NOTE TO SPECIFIER ** Select one of the following configuration mounting paragraphs as required and delete the paragraphs not required. . If multiple types are required indicate the location of each in the schedule at the end of this section. Manufacturer chooses hardware size based on fabric size, weight, and thickness and window opening size.

      - Dual Roller Configuration / Mounting:

        - Dual roller endcaps only. Endcaps for surface or recessed mounting of dual roller window shades. 1028 steel stamping.

        - Dual roller fascia. Endcaps with fascia designed for surface mounting of dual roller window shades.

          - Endcaps: 1028 steel stamping.

          - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

          - Size: 4-3/4 inches deep x 7 inches high x length required by window opening.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Dual roller with pocket headbox. Pocket and endcaps designed for recessed ceiling installation of dual roller window shades.

          - Material: 18 gauge steel.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: White Inside/Grey Outside with white ceiling tile flange.

          - Finish: Grey Inside/White Outside with white ceiling tile flange.

          - Closure Panel Aluminum, without exposed fasteners:

** NOTE TO SPECIFIER ** Select the flange required from the following paragraphs and delete those not required.

            - Front Tile Flange.

            - Left Tile Flange.

            - Right Tile Flange.

            - Pre-drilled Mounting Holes.

            - Pre-drilled Ventilation holes.

        - Dual roller with ceiling/wall headbox. Fascia, L-shaped top/back, and endcaps designed for wall or ceiling installation of dual roller window shades.

          - Endcaps: 1028 steel stamping.

          - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

          - Finish: Powder coat.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

            - Selected from Manufacturers standard range.

            - Custom powder coat as selected by Design Professional.

        - Shade slat:

** NOTE TO SPECIFIER ** Select the shade slat required from the following paragraphs and delete those not required

          - Closed pocket elliptical slat: 1 inch (25 mm) aluminum elliptical slat inside of a 1-5/8 inch (41 mm) pocket with heat sealed ends.

          - Open pocket elliptical slat: 1 inch (25mm) aluminum elliptical slat with plastic ends inside of a 1-5/8 inch (41 mm) pocket.

          - Flat exposed hem bar: Small: 7/8 inch x 5/16 inch (22 mm x 8 mm) and Large 1-1/2 inch x 5/16 inch (38 mm x 8 mm) aluminum rectangular hem bar with plastic end caps.

          - Round exposed hem bar: Small 5/8 inch (16 mm) and Large 13/16 inch (21 mm) aluminum with plastic end caps.

        - Light Gap Reduction Channels.

** NOTE TO SPECIFIER ** Select channel size required from the following paragraphs.

          - Aluminum L Angle - 3/4 inch (19 mm) by 1 inch (25 mm).

          - Aluminum L Angle -1 inch (25 mm) by 2-3/4 inches (70mm).

          - Vinyl L Angle-1-1/2 inches (38 mm) by 3/4 inch (19 mm).

          - U Channel -1 inch (25 mm) by 2-1/2 inches (64 mm).

          - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

      - Bottom-Up Single Roller Configuration. Bottom-Up Single Roller Clutch-Operated FlexShade NEXD: Manually operated, vertical roll-up, fabric window shade with components necessary for complete installation, as manufactured by Draper, Inc.

        - Brackets: Plated stamped steel. Provide size compatible with roller size.

** NOTE TO SPECIFIER ** Select the mount required from the following paragraphs and delete those not required.

          - Mounted to ceiling.

          - Mounted to wall.

          - Mounted to jamb.

        - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

          - Endcap covers: To match fascia or headbox color.

        - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

          - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands. No notching is required.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range

          - Finish: Custom powder coat as selected by the Architect.

        - Headbox Ceiling/Wall style: Aluminum fabrication with removable closure, endcaps, and back and top cover piece:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

      - Rollers: Extruded aluminum roller tube of appropriate diameter to support shade fabric with minimal deflection.

        - Minimum Roller Tube Diameter: 1.5 inches (32 mm). Tube diameters less than 1.5 inches shall not be acceptable unless manufacturer provides deflection analysis showing deflection limited to <= width (inches) /700 at 1.5X design load.

        - Fabric Connection to Roller Tube: Spline fabric/roller attachment system to allow shade fabric to be removed from roller without having to remove roller from brackets.

        - Fabric Length: 6 inches (152 mm) greater than window height minimum.

** NOTE TO SPECIFIER ** Select orientation required from the following paragraphs.

        - Orientation: Regular from back of roller.

        - Orientation: Reverse from front of roller.

        - Hembar: Extruded aluminum, finished to match fascia.

** NOTE TO SPECIFIER ** Use the following paragraphs to specify Draper Motorized FlexShades. Select the motor and control types desired from the following paragraphs. Delete if not required.

  - MOTORIZED WINDOW SHADES

    - Type: Motorized vertical roll-up, fabric, window shade with motors, controls, mounting brackets, and other components necessary for complete installation; Motorized FlexShade as manufactured by Draper, Inc.

** NOTE TO SPECIFIER ** Select the mounting type required from the following options, delete those not applicable. Brackets are not used with Headbox. If multiple types are required indicate the location of each in the schedule at the end of this section.

      - Mounting brackets.

      - Endcaps and fascia.

      - Endcaps and headbox.

      - Ceiling pocket.

    - Shade Motor and Control System

** NOTE TO SPECIFIER ** Select one of the following motor option paragraphs and delete those not required. General descriptions of each Motor and Control system are included with each option below. Contact Draper for more information concerning motor limitations.

      - Motorized FlexShade I/O AC UL Listed if single roller configuration.

        - IntelliFlex I/O 6Nm AC Motor: 120V AC, single phase, 60 HZ, 3-wire, 6nm or 15nm, at the manufacturer's discretion, instantly reversible, lifetime lubricated smart motor. Tubular motor concealed inside each shade roller tube. Equipped with internal thermal overload protector, and electric brake. Thermal protection: 4 minutes.

          - Rated current: 1.0 Amps. Motors drawing greater than 1.0 amps shall not be acceptable without prior approval by Design Professional responsible for electrical design of project.

          - Speed: 28RPM. All motors within a discrete control zone must operate at same speed.

          - Sound level: 42dBA.

          - UL Listing: UL325 & CSA C22.2 No. 247. Motors not listed as part of an entire shade assembly and control system shall not be acceptable.

          - Encoded motors with two-way communication to network control devices. Motor/control networks that do not support two-way communication shall not be acceptable.

          - Limits: Intermediate stopping points and presets.

** NOTE TO SPECIFIER ** Most IntelliFlex® I/O motors, interface, and input devices automatically include a Network Device Connector (NDC. NDC is required to connect a device to the network and distribute power and communications between devices.

        - IntelliFlex I/O Network Devices.

          - Network Device Connector. Connects lntelliFlex I/O devices into a network. Distributes power and communications between devices. One provided with each IntelliFlex I/O motor and network device. Approved for use in plenum spaces. LED Indicators indicate network status. Includes built-in terminating resistor, and network bypass switch for troubleshooting.

** NOTE TO SPECIFIER ** Edit the following paragraphs for network devices that are required on your project and remove those not required. Note that a IntelliFlex I/O Wireless Network Gateway is required to connect wireless devices to the wired network. Delete if not required.

          - Wireless Network Gateway. Allows wireless devices to control shades on the wired network. Required when using Radio Frequency handheld remote. LEDs indicate the status of both the wired and wireless networks. Allows wireless control of the entire network from a single receiving point.

          - AV Gateway. Used to integrate with third party control systems. Single entry point must support RS-232 and RS-485 and contain at least 6 contact closure inputs, low voltage trigger, and IR receiver input. Approved for use in plenum spaces. LCD display allows for commissioning of serial groups.

          - Contact Closure Interface. Allows up to 6 contact closure inputs to be connected to the network. These are grouped into 3 sets of Up/Down signals, each of which can control any number of shades on an lntelliFlex I/O network. Configuration must be done using onboard programming buttons, without rewiring or the use of external configuration devices. Power is provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication.

          - Sensor/Schedule Interface. Allows up to 4 sensors to be added to an IntelliFlex I/O network. Must include built-in support for brightness, wind, and precipitation sensors, and pyranometers. Must support up to 8 scheduled override events. Configuration must be done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. External power input for sensors provides support for any sensor power requirements.

            - SSI Transformer. To be used when adding third party sensors to the network.

          - Sensor/Schedule Interface with Automated Glare Control. The IntelliFlex I/O Sensor / Scheduler Interface with automated glare control allows up to 4 sensors to be added to an IntelliFlex I/O network. Includes built-in support for brightness, temperature, wind, and precipitation sensors as well as pyranometers. Also provides support for up to 8 scheduled override events. Configuration is done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. Configuration using built-in buttons and LCD display. Scheduled events can be configured to run on any combination of days of the week. External power input for sensors provides support for any sensor power requirements. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4-7/8 inches by 3 inches by 1 inch (124 by 83 by 32 mm). Mounting hardware provided.

          - Central Network Controller. Touchscreen device to add advanced functionality to a network. It also acts as a router between the control network and a building IP network. Graphical user interfaces allow for the monitoring and override of motors and network devices. Automated glare control and scheduling. BACnet/IP integration. Interface to monitor and override network devices. Allows logging of shade movements, sensor readings, and user overrides. Input power: Micro USB. Operating temperature: ambient. Dimensions: 9-7/8 inches by 5-1/4 inches by 2 inches (251 by 146 by 51mm). Rough cutout: 8 inches by 5 inches by 1-3/4 inches (203 by 127 by 44 mm). Mounting hardware provided.

          - IP Gateway. For connecting a network of IntelliFlex I/O devices to an IP network. Built-in two port network switch to allow direct connection to another IP Gateway creating a standalone network for access to all connected IntelliFlex I/O devices. Each IPG allows for an additional 120 devices to be added to the network. Configuration is done using the built-in buttons and display. Material: ABS plastic. Operating temperature: ambient. Dimensions: 4-7/8 inches by 3 inches by 1 inch (124 by 83 by 32 mm). Mounting hardware provided.

        - IntelliFlex I/O Input Devices:

          - Single Zone Wall Switch. Allows user to move roller shades to any position with a single touch. Swipe gestures must be recognized to move shades to fully open or closed positions. Power provided through lntelliFlex I/O network using a single Ethernet cable for power and communication. Switch is capable of controlling any number of shades on an lntelliFlex I/O network using onboard programming buttons without rewiring or the use of external configuration devices. Must allow unlimited number of preset alignment positions. Push button switches not permitted.

          - Dual Zone Wall Switch. Allows user to control two independent zones of roller shades, moving them to any position with a single touch. Swipe gestures must also be recognized to move shades to fully open or closed positions. Power for switch must be provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication. Each zone must be configured to control any number of shades on an lntelliFlex I/O network using the onboard programming buttons, without rewiring or the use of external configuration devices. Unlimited number of preset alignment positions. Push button switches not permitted.

          - Radio Frequency Handheld Remote Control. Display must allow for any number of channels, up to 32 channels. Touch control to position the shades anywhere with one touch. Must be capable of serving as a simple commissioning tool. lntelliFlex I/O Wireless Network Gateway is required with remote. Range is 40 feet from closest IntelliFlex I/O device. Push button remotes not permitted.

          - Infrared (IR) Remote Control Kit. Includes handheld remote transmitter and IR receiver. For use with AV Gateway.

** NOTE TO SPECIFIER ** Select required optional sensors and delete the ones not required. IntelliFlex I/O Sensor/Schedule Interface required for sensors.

        - IntelliFlex I/O Sensors.

          - External Brightness Sensor. For mounting on the exterior of the building. For use with Sensor/Schedule Interface.

          - Wind Speed Sensor. For use with Sensor/Schedule Interface.

          - Internal Brightness Sensor. To be mounted on the interior side of the window with adhesive or with provided mounting bracket. For use with Sensor/Schedule Interface.

** NOTE TO SPECIFIER ** Standard Motor: 120V AC tubular motor with 4-wire pigtail. All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces. Only those controls listed in this section will operate this motor

      - Motorized FlexShade AC UL Listed if single roller configuration

        - Standard Motor: 120V AC, single phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Quiet Standard Motor: Motor operates at 44 Db. Motor noise is approximately 30 percent less than standard tubular motors. Ultra Quiet operates at 38 Db. All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

        - Quiet 120V AC motor operates at 44 Db measured 3 feet from the motor. Tubular motor concealed inside each shade roller tube.

        - Ultra Quiet 120V AC motor operates at 38 Db measured 3 feet from the motor. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select one of the following control option paragraphs and delete the ones not required. For each control selected select the control device(s) required and delete those not required.

        - Individual Control:

          - Wall Switch - Toggle three position wall switch.

          - Wall Switch - Key operated three position wall switch.

        - Group Control:

          - 1 Motor Isolating Relay (1ISO) - One ISO relay per motor. Allows 110-120V group switching. Allows for up to 12 motors to be operated from a single 15 amp double throw switch, determined by motor size and circuit rating. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps.

          - 1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 2-3/4 inched by 3-1/2 inches by 2-1/8 inches (70 by 90 by 54 mm). Enclosure: 4 inches by 4 inches by 4 inches (102 by 102 by 102 mm).

          - 4 Motor Isolating Relay (GC4) - Panelized version with four ISO relays. Up to three GC4s can be linked together for control of up to 12 motors from any single 15 amp double throw wall switch, determined by motor size and circuit rating. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps. Installed in a 8 inch by 8 inch by 4 inch (203 by 203 by 102 mm) box.

          - 4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 3-1.2 inches by 7 inches by 1-7/8 inches (89 by 178 by48 mm). Enclosure: 6 inches by 4 inches by 4 inches (152 by 102 by 102 mm).

        - Individual and Group Control: IGC4N1. Individual and Group control of up to four motors with up to three intermediate positions per motor. Each motor output can be individually addressed and mapped to any of the 5 wall switch input ports using onboard dip switches

** NOTE TO SPECIFIER ** FlexShade Recharge Battery Motor: Rechargeable battery powered motor with Ion Lithium batteries in roller and built-in radio receiver. No wiring required. Only those controls listed will operate this motor

      - FlexShade Recharge. Lithium-Ion battery powered motor with built-in radio receiver. Includes Ion Lithium batteries. No external wiring required. Tubular motor and batteries concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the Battery Charging options and delete those not required.

        - 12V Power Adapter.

        - Solar Recharging Panel.

        - Recharge Motor Cable Extender - 6 inches.

        - Recharge Motor Cable Extender - 48 inches.

        - Recharge Motor Cable Extender - 96 inches.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld radio frequency transmitter.

          - Five channel wireless handheld radio frequency transmitter.

          - 15-Channel wireless handheld radio frequency transmitter.

          - Single channel wireless wall switch for radio motor control.

          - Five channel wireless wall switch for radio motor control.

          - Fifteen channel wireless wall switch for radio motor control.

          - Single channel dry contact interface.

          - Screwless switch cover plate.

          - Recharge Sun Sensor.

          - Recharge Arc Wi-Fi to 433 2 way RF bridge kit

** NOTE TO SPECIFIER ** Radio Technology Motor: Motor with built-in radio receiver. 3-wire pigtail allows for attachment of optional three prong plug for plug-an-play operation. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor. Quiet Radio Technology Motor: Quiet motor with built-in radio receiver. Operates at 44 Db. Motor noise is approximately 30 percent less than standard tubular motor. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

      - Motorized FlexShade AC RTS. UL Listed if single roller configuration

** NOTE TO SPECIFIER ** Select the motor required and delete the one not required.

        - Standard Motor: 120V AC motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

        - Quiet 120V AC RTS Motorized FlexShade. 120V AC quiet motor with built-in radio receiver. Operates at 44 dBA. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld radio frequency transmitter.

          - Single channel wireless handheld transmitter-Black.

          - Four channel wireless handheld transmitter-White.

          - Four channel wireless handheld transmitter-Black.

          - 16-Channel wireless handheld transmitter-White.

          - 16-Channel wireless handheld transmitter-Silver.

          - Single channel wireless wall switch for radio motor control-White.

          - Single channel wireless wall switch for radio motor control-Ivory.

          - Four channel wireless wall switch for radio motor control-White.

          - Four channel wireless wall switch for radio motor control-Ivory.

          - RS-232/Dry Contact - Allows for full control of radio motor shade via RS-232/dry contact interface.

          - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

          - Wireless Timer - Allows operation of one group of radio motors per timer.

          - Repeater, 1 per project to extend range of transmitters.

          - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

** NOTE TO SPECIFIER ** 24V DC Quiet motors with built-in radio receiver. Operate at 44 DB. Requires 24V transformer. Low voltage wiring does not require an electrician. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate these motors.

      - FlexShade ST30 DC RTS. 24V DC quiet motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld transmitter-White.

          - Single channel wireless handheld transmitter-Black.

          - Four channel wireless handheld transmitter-White.

          - Four channel wireless handheld transmitter-Black.

          - 16-Channel wireless handheld transmitter-White.

          - 16-Channel wireless handheld transmitter-Silver.

          - Single channel wireless wall switch for radio motor control-White.

          - Single channel wireless wall switch for radio motor control-Ivory.

          - Four channel wireless wall switch for radio motor control-White.

          - Four channel wireless wall switch for radio motor control-Ivory

          - Dry Contact - Allows for full control of radio motor shade via dry contact interface.

          - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

          - Wireless Timer - Allows operation of one group of radio motors per timer.

          - Repeater, 1 per project to extend range of transmitters

          - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

        - Transformer (24V DC ONLY):

** NOTE TO SPECIFIER ** Select the transformer type and motor quantity required and delete those not required.

          - Plug-In:

            - 1 Motor - Wall Type.

            - 1 Motor - Floor Type.

          - Power Panel:

            - 5 Motors.

            - 10 Motors.

            - 15 Motors.

            - 20 Motors.

      - Motorized FlexShade 50 RTS DC. 24V DC, UL Listed if single roller configuration quiet motor Quiet motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube. 4 Nm of torque. Operates at 38 dBA.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld transmitter-White.

          - Single channel wireless handheld transmitter-Silver.

          - Four channel wireless handheld transmitter-White.

          - Four channel wireless handheld transmitter-Silver

          - 16-Channel wireless handheld transmitter-White.

          - 16-Channel wireless handheld transmitter-Silver.

          - Single channel wireless wall switch for radio motor control-White.

          - Single channel wireless wall switch for radio motor control-Ivory.

          - Four channel wireless wall switch for radio motor control-White.

          - Four channel wireless wall switch for radio motor control-Ivory

          - Dry Contact - Allows for full control of radio motor shade via dry contact interface.

          - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

          - Wireless Timer - Allows operation of one group of radio motors per timer.

          - Repeater, 1 per project to extend range of transmitters

          - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

        - Transformer (24V DC ONLY):

** NOTE TO SPECIFIER ** Select the transformer type and motor quantity required and delete those not required.

          - Plug-In:

            - 1 Motor - Wall Type.

            - 1 Motor - Floor Type.

          - Power Panel:

            - 10 Motors.

** NOTE TO SPECIFIER ** 24V DC Dry Contact Motor: 24V DC Operates at 44 DB. Requires 24V transformer. Low voltage wiring does not require an electrician. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

      - FlexShade ST30 DC Dry Contact - 24V DC quiet motor with dry contact for connection to third party control systems. Tubular motor concealed inside each shade roller tube. 2 Nm of torque.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control, and Individual and Group Control:

          - 3-button dry contact switch (white) for up/stop/down control, limit setting, and motor speed control of a shade. Third party controllers can be connected to a secondary 4 wire terminal block located on the switch.

          - 3-button dry contact switch (ivory) for up/stop/down control, limit setting, and motor speed control of a shade. Third party controllers can be connected to a secondary 4 wire terminal block located on the switch.

          - Dry contact motor limit setting tool.

** NOTE TO SPECIFIER ** Note that the IP interface requires a connection the building server to pull time data and an IP address for access to the interface.

        - Transformer:

** NOTE TO SPECIFIER ** Select the transformer type and motor quantity required and delete those not required.

          - Plug-In:

            - 1 Motor - Wall Type.

            - 1 Motor - Floor Type.

          - Power Panel:

            - 5 Motors.

            - 10 Motors.

            - 15 Motors.

            - 20 Motors.

** NOTE TO SPECIFIER ** Intelligent Technology Motor: Motor with built-in low voltage controller with 3-wire pigtail and data cable. No external motor control interface required. Best for integration into building automation systems. Available with optional three prong plug for plug-and-play operation. Only those controls listed will operate this motor. Quiet Intelligent Technology Motor: Motor with built-in low voltage controller with 3-wire pigtail and data cable. Operates at 44 Db. No external motor control interface required. Best for integration into building automation systems. Available with optional three prong plug for plug-and-play operation. Only those controls listed will operate this motor.

      - Motorized FlexShade AC 485. UL Listed if single roller configuration

        - 120V AC Intelligent Technology motor with built-in low voltage controller with 3-wire pig tail and data cable. No external motor controls are required. Available with optional three prong plug. Tubular motor concealed inside each shade roller tube.

        - Quiet 120V AC Intelligent Technology Motor - 120V AC quiet motor with built-in low voltage controller with 3-wire pig tail and data cable. Motor operates at 44 dBA measured 3 feet from the motor. Makes no audible clicks when motor stops or starts. Available with optional three prong plug. Tubular motor concealed inside each shade roller tube.Ultra

        - Ultra Quiet 120V AC Intelligent Technology Motor - 120V AC quiet motor with built-in low voltage controller with 3-wire pig tail and data cable. Motor operates at 38 dBA measured 3 feet from the motor. Makes no audible clicks when motor stops or starts. Available with optional three prong plug. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control & Individual and Group Control:

          - 6 button RS 485 switch- for 2 groups up/stop/down or 1 group up/stop/down and 3 presets.

          - 8 button RS 485 switch- for 5 groups up/stop/down or 1 group up/stop/down and 5 presets.

          - Somfy Digital Network. SDN system of window shades with intelligent motors linked to an IP interface box and other input devices for automated glare control.

          - Bus Distribution

            - Data Hub for 485 Motor

            - 5-Way RJ45 Splitter

            - Building Controllers

            - SDN Data Panel

          - User Interfaces

            - Somfy Connect BMS Interface

            - Somfy Connect LTI

            - Z Wave Digital Motor Interface (ZDMI)

            - Zigbee to RS485 (ZBDMI)

            - RTS Receiver for 485

            - SDN 6 button wall switch (Preprogrammed for master control)

            - SDN 8 button wall switch

            - Somfy Connect UAI Plus

          - Accessories

            - Bus Power Supply, 24V, 1A

          - 32 point BACnet interface- Control of up to 32 groups with feedback via BACnet over IP.

** NOTE TO SPECIFIER ** 24V DC RS 485 Motor: 24V DC Quiet motor with built-in intelligence for RS 485 control. Operates at 44 DB. Requires 24V transformer. Low voltage wiring does not require an electrician. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

      - Model: FlexShade ST30 DC RS 485 Motor - 24V DC quiet motor with built-in intelligence for RS 485 control. Tubular motor concealed inside each shade roller tube. 4 Nm of torque.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - 6 button RS 485 switch- for 2 groups up/stop/down or 1 group up/stop/down and 3 presets.

          - 8 button RS 485 switch- for 5 groups up/stop/down or 1 group up/stop/down and 5 presets

          - Somfy Digital Network. SDN system of window shades with intelligent motors linked to an IP interface box and other input devices for automated glare control.

          - Bus Distribution

            - Data Hub for 485 Motor

          - 5-Way RJ45 Splitter

            - Building Controllers

            - SDN Data Panel

          - User Interfaces

            - Somfy Connect BMS Interface

            - Somfy Connect LTI

            - Z Wave Digital Motor Interface (ZDMI)

            - Zigbee to RS485 (ZBDMI)

            - RTS Receiver for 485

            - SDN 6 button wall switch (Preprogrammed for master control)

            - SDN 8 button wall switch

            - Somfy Connect UAI Plus

          - Accessories

            - Bus Power Supply, 24V, 1A

          - 32-point BACnet interface-Control of up to 32 groups with feedback via BACnet over IP

        - Transformer (24V DC ONLY):

** NOTE TO SPECIFIER ** Select the transformer type and motor quantity required and delete those not required.

          - Plug-In:

            - 1 Motor - Wall Type.

            - 1 Motor - Floor Type.

          - Power Panel:

            - 10 Motors.

      - Motorized FlexShade 50 DC RS 485 UL Listed if single roller configuration 24V DC quiet motor with built-in intelligence for RS 485 control. Tubular motor concealed inside each shade roller tube. 4 Nm of torque.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - 6 button RS 485 switch- for 2 groups up/stop/down or 1 group up/stop/down and 3 presets.

          - 8 button RS 485 switch- for 5 groups up/stop/down or 1 group up/stop/down and 5 presets

          - Somfy Digital Network. SDN system of window shades with intelligent motors linked to an IP interface box and other input devices for automated glare control.

          - Bus Distribution

            - Data Hub for 485 Motor

          - 5-Way RJ45 Splitter

            - Building Controllers

            - SDN Data Panel

          - User Interfaces

            - Somfy Connect BMS Interface

            - Somfy Connect LTI

            - Z Wave Digital Motor Interface (ZDMI)

              - Zigbee to RS485 (ZBDMI)

            - RTS Receiver for 485

            - SDN 6 button wall switch (Preprogrammed for master control)

            - SDN 8 button wall switch

            - Somfy Connect UAI Plus

          - Accessories

            - Bus Power Supply, 24V, 1A

          - 32-point BACnet interface-Control of up to 32 groups with feedback via BACnet over IP.

        - Transformer:

** NOTE TO SPECIFIER ** Select the transformer type and motor quantity required and delete those not required.

          - Plug-In:

            - 1 Motor - Wall Type.

            - 1 Motor - Floor Type.

          - Power Panel:

            - 10 Motors.

NOTE TO SPECIFIER ** Select the mounting type required for window shade from the following options, delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section. Manufacturer chooses hardware size based on fabric size, weight, and thickness and window opening size.

    - Configuration:

      - Single Roller.

        - Mounting.

** NOTE TO SPECIFIER ** Endcaps are required for attachment of fascia or use with headbox. Delete if not required.

          - Endcaps: 1028 stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size. Endcap covers to match fascia/headbox finish.

** NOTE TO SPECIFIER ** Shades provided without endcaps will require mounting brackets. Delete if not required.

          - Brackets: 1018 stamped steel, black powder coat, idler height adjuster, field adjustable to wall or ceiling mount.

** NOTE TO SPECIFIER ** Select bracket mounting type required from the following and delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section.

            - Mounted to ceiling.

            - Mounted to wall.

            - Mounted to jamb.

** NOTE TO SPECIFIER ** Draper Motorized FlexShades can be provided with aluminum fascia concealing shade roller and hardware. Fascia can be mounted inside window opening extending from jamb to jamb or mounted outside and overlapping window opening. Fascia is attached to endcaps. Fascia can be provided in lengths up to 16 feet and either 3-7/8 or 4-5/8 inches high. Fascia available for reverse roll applications. If fascia is required, include the following paragraph. Delete if not required.

          - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

            - Attachment: Snaps onto end caps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

            - Finish: Selected from Manufacturers standard range.

            - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Draper Motorized FlexShades can be provided with aluminum headbox designed for exposed surface or recessed ceiling installation. Ceiling/Wall style headbox consists of removable closure, back and top, and endcaps. Pocket style headbox consists of removable closure, U-shaped pocket and endcaps. Headbox is provided in two sizes. Refer to Draper product literature for required sizes based on window and roller size. If headbox is required, include one of the following paragraphs. Delete if not required.

          - Headbox Ceiling/Wall Style: Aluminum fabrication with removable closure, endcaps, and back and top cover piece:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

            - Selected from Manufacturers standard range.

            - Finish: Custom powder coat as selected by the Architect.

          - Headbox, Pocket Style: Aluminum fabrication with removable closure, endcaps, and U-shaped pocket:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

            - Finish: Selected from Manufacturers standard range.

            - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Select one of the following two paragraphs for Shade Pockets or Wall Clip with Closure Panel as required and delete the paragraph not required.
Motorized FlexShades can be provided with and installed in fabricated pockets. Pockets are designed to be installed prior to or as part of ceiling system installation. Window shade assembly can be site installed later after construction operations that might damage shade are complete. Include the following paragraph if aluminum pockets are required. Delete if not required.

          - Type D Shade pocket: Rectangular pocket with ceiling tile lip designed for recessed ceiling installation of window shades.

            - Material: Extruded aluminum alloy with white epoxy paint finish.

            - Size: 5 inches (127 mm) by 5-3/8 inches (137 mm) high.

            - Closure Panel:

** NOTE TO SPECIFIER ** Closure panel is optional select the one required or delete if not required.

              - 1-1/2 (38 mm) closure dimension.

              - 3 inch (78 mm)

** NOTE TO SPECIFIER ** Endcaps for Type D FlexShade Pocket is provided with an acoustical ceiling panel support lip.

            - Pocket Endcap Kit: Endcaps with 7/8 inch (22 mm) lip for support of acoustical ceiling panel.

** NOTE TO SPECIFIER ** Type D FlexShade Pocket can be provided with corner sections where window shade recess continues around two walls. Pocket ends can also be provided. Choose all that apply.

            - Corners: Welded one-piece aluminum sections connecting to and matching pockets to allow continuous shade recess at ceiling corners.

            - Pocket ends: Welded one-piece aluminum sections connecting to and matching pockets.

** NOTE TO SPECIFIER ** Draper Motorized FlexShades can be installed in ceiling recess site constructed from light gauge metal framing and gypsum board or other materials. For this condition, use the following paragraph and subparagraphs to specify aluminum removable closure panel to be installed leaving a minimal slot for passage of shade. Delete if not required.

          - Wall Clip with Closure panel: For site constructed ceiling recesses, provide removable closure panel to minimize slot for shade passage but allowing access to shade for maintenance.

            - Material: Aluminum alloy with white epoxy paint finish.

** NOTE TO SPECIFIER ** Delete the following paragraph if tile lip option is not required.

            - Tile Lip: Provide wall clip with 7/8 inch tile lip (22 mm).

** NOTE TO SPECIFIER ** Select closure width required from the following paragraphs.

            - Closure width: 1-1/2 inches (38 mm).

            - Closure width: 3 inches (75 mm).

            - Closure Width: 5 inches (127 mm).

            - Provide continuous wall clip, 1-3/4 (44 mm) by 3/16 inch (5 mm), for snap-in attachment of closure panel without fasteners.

** NOTE TO SPECIFIER ** Draper Motorized FlexShades can be installed with light reduction channels. For this requirement, use the following paragraph and subparagraphs. Delete if not required.

          - Aluminum Light Gap Reduction Channels

** NOTE TO SPECIFIER ** Select channel required from the following.

            - L Angle - 3/4 inch by 1 inch (19 mm by 25 mm).

            - L Angle - 1 inch by 2-3/4 inches (25 mm by 70 mm).

            - U Channel - 1 inch by 2-1/2 inches (25 mm by 64 mm).

            - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

          - Vinyl Light Gap Reduction Channels

            - L Angle - 1-1/2 inches 3/4 inches (38 mm by 19 mm).

          - Interior cable guide kit

            - Interior cable hem bar finial.

            - Cable mount.

            - Cable.

            - Tension Stop.

          - Shade slat:

** NOTE TO SPECIFIER ** Select the shade slat required from the following paragraphs and delete those not required

            - Closed pocket elliptical slat: 1 inch (25 mm) aluminum elliptical slat inside of a 1-5/8 inch (41 mm) pocket with heat sealed ends.

            - Open pocket elliptical slat: 1 inch (25mm) aluminum elliptical slat with plastic ends inside of a 1-5/8 inch (41 mm) pocket.

            - Flat exposed hem bar: Small: 7/8 inch x 5/16 inch (22 mm x 8 mm) and Large 1-1/2 inch x 5/16 inch (38 mm x 8 mm) aluminum rectangular hem bar with plastic end caps.

            - Round exposed hem bar: Small 5/8 inch (16 mm) and Large 13/16 inch (21 mm) aluminum with plastic end caps.

      - Dual Roller.

        - Mounting.

          - Dual roller endcaps. Endcaps for surface or recessed mounting of dual roller window shades. 1028 steel stamping.

          - Dual roller fascia. Endcaps with fascia designed for surface mounting of dual roller window shades.

            - Endcaps: 1028 steel stamping.

            - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

            - Finish: Powder coat.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

              - Finish: Selected from Manufacturers standard range

              - Finish: Custom powder coat as selected by the Architect.

          - Dual roller with pocket headbox. Pocket and endcaps designed for recessed ceiling installation of dual roller window shades.

            - Material: 18 gauge steel.

            - Finish:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

              - White Inside/Grey Outside with white ceiling flange.

              - Grey Inside/White Outside with white ceiling tile flange.

            - Closure Panel Aluminum, without exposed fasteners:

** NOTE TO SPECIFIER ** Dual Roller Pocket Headbox can be ordered with tile flanges, pre-drilled mounting holes, and pre-driller ventilation holes. Select required paragraphs below and delete the others.

              - Front Tile Flange.

              - Left Tile Flange.

              - Right Tile Flange.

              - Pre-drilled Mounting Holes.

              - Pre-drilled Ventilation holes.

          - Dual roller with ceiling/wall headbox. Fascia, L-shaped top/back, and endcaps designed for wall or ceiling installation of dual roller window shades.

            - Endcaps: 1028 steel stamping.

            - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

            - Finish: Powder coat.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

              - Selected from Manufacturers standard range.

              - Custom powder coat as selected by Design Professional.

        - Aluminum Light Gap Reduction Channels

** NOTE TO SPECIFIER ** "L", "U", or "H" channels are available on the rear roller for light gap reduction. Select the desired paragraph and deleted the others.

          - L Angle - 3/4 inch by 1 inch (19 mm by 25 mm).

          - L Angle - 1 inch by 2-3/4 inches (25 mm by 70 mm).

          - U Channel - 1 inch by 2-1/2 inches (25 mm by 64 mm).

          - H Channel - 1 inch (25 mm) by 5 inches (127 mm).

        - Vinyl Light Gap Reduction Channels

          - L Angle - 1-1/2 inches 3/4 inches (38 mm by 19 mm).

        - Shade slat:

** NOTE TO SPECIFIER ** Select the shade slat required from the following paragraphs and delete those not required

          - Closed pocket elliptical slat: 1 inch (25 mm) aluminum elliptical slat inside of a 1-5/8 inch (41 mm) pocket with heat sealed ends.

          - Open pocket elliptical slat: 1 inch (25mm) aluminum elliptical slat with plastic ends inside of a 1-5/8 inch (41 mm) pocket.

          - Flat exposed hem bar: Small: 7/8 inch x 5/16 inch (22 mm x 8 mm) and Large 1-1/2 inch x 5/16 inch (38 mm x 8 mm) aluminum rectangular hem bar with plastic end caps.

          - Round exposed hem bar: Small 5/8 inch (16 mm) and Large 13/16 inch (21 mm) aluminum with plastic end caps.

      - Bottom-Up Single Roller. Motorized Bottom-Up FlexShade.

** NOTE TO SPECIFIER ** Shades provided without endcaps will require mounting brackets. Brackets are not used with Fascia or Headbox. Delete if not required.

        - Brackets: Plated stamped steel. Provide size compatible with roller size.

** NOTE TO SPECIFIER ** Select bracket mounting type required from the following and delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section.

          - Mounted to ceiling.

          - Mounted to wall.

          - Mounted to jamb.

** NOTE TO SPECIFIER ** Include endcaps if required and delete if not required. Endcaps are optional for Chain and Clutch Operated FlexShades. Endcaps are also required for attachment of fascia and headbox.

        - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

          - Endcap covers: To match fascia or headbox color.

** NOTE TO SPECIFIER ** Draper Bottom-Up FlexShades can be provided with aluminum fascia concealing shade roller and hardware. Fascia can be mounted inside window opening extending from jamb to jamb or mounted outside and overlapping window opening. Fascia is attached to endcaps. Fascia can be provided in lengths up to 16 feet and either 3-7/8 or 4-5/8 inches high. Fascia available for reverse roll applications. Include the following paragraph if fascia is required. Delete if not required.

        - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

          - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands. No notching is required.

          - Shape: Square Fascia Panel.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER ** Ceiling/Wall style headbox consists of removable closure, back and top, and endcaps. Headbox is provided in two sizes. Refer to Draper product literature for required sizes based on window and roller size. Include the following paragraph if headbox is required. Delete if not required.

        - Headbox Ceiling/Wall style: Aluminum fabrication with removable closure, endcaps, and back and top cover piece:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

          - Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Hembar: Machine extruded aluminum, finished to match fascia.

      - Bottom-Up Dual Roller. Motorized Bottom-Up Dual Roller FlexShade. Endcaps with fascia designed for surface mounting of dual roller window shades.

        - Endcaps: 1028 steel alloy stamping.

        - Fascia: L-shaped cover of extruded aluminum, .060 wall. Assembly snaps onto endcaps without exposed fasteners.

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

        - Selected from Manufacturers standard range.

        - Finish: Custom powder coat as selected by the Architect.

        - Hembar: Extruded aluminum, finished to match fascia.

    - Roller: Fabricated from extruded aluminum or steel. Wall thickness and material selected by manufacturer to accommodate shade size. Provide with roller idler assembly of molded nylon and zinc-plated steel pin. Sliding pin to allow easy installation and removal of roller. Fabric connected to the roller tube with LSE (low surface energy) double sided adhesive specifically developed to attach coated textiles to metal. Adhesive attachment to eliminate horizontal impressions in fabric.

** NOTE TO SPECIFIER ** Up to five Draper Motorized FlexShades can be operated by a single motor by coupling the rollers with Draper FlexShade Coupling System. Shades can be different widths (up to 120 inches (305 cm) per panel) but all rollers must be the same diameter. Maximum group width is 30 feet (9.1 m). System consists of male and female couplings and wall or ceiling brackets. Motor can be installed in either the far left or far right shades, with no more than five panels on one motor. PLEASE NOTE: ST30 DC MOTORS AND BOTTOM-UP SHADES CANNOT BE COUPLED. Delete if not required.

    - Coupling system: Couplings to join motorized shade rollers to allow operation by single motor. FlexShade Coupling System as manufactured by Draper, Inc. Provide endcaps to receive couplers and support multiple shades. Coupling system must be capable of up to 30-degree angles.

** NOTE TO SPECIFIER ** Use the following paragraphs to specify Intermediate size Draper Motorized FlexShades. Select the motor and control types desired from the following paragraphs. Delete if not required.

  - INTERMEDIATE SIZE MOTORIZED WINDOW SHADES

    - Type: Motorized vertical roll-up, fabric, window shade with motors, controls, mounting brackets, and other components necessary for complete installation; Motorized FlexShade 2 as manufactured by Draper, Inc.

      - Mounting brackets.

      - Endcaps and fascia.

      - Endcaps and ceiling/wall headbox.

      - Endcaps with pocket headbox

    - Shade Motor and Control System

** NOTE TO SPECIFIER ** Select one of the following motor option paragraphs and delete those not required. Descriptions of each Motor and Control system are included with each option below. Contact Draper for more information concerning motor limitations. Up to 20' wide and 30' high, depending on fabric.
All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces. Only those controls listed in this section will operate this motor.
Quiet Standard Motor: Motor operates at 44 Db. Motor noise is approximately 30 percent less than standard tubular motors. All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

      - Motorized FlexShade 2 AC

        - FlexShade 2 120V AC motor. Single phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Tubular motor concealed inside each shade roller tube.

        - FlexShade 2 Quiet 120V AC motor. Operates at 44 Db measured 3 feet from the motor. Makes no audible clicks when motor stops or starts. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select one of the following control option paragraphs and delete the ones not required. For each control selected select the control device(s) required and delete those not required.

        - Individual Control:

          - Wall Switch - Toggle three position wall switch.

          - Wall Switch - Key operated three position wall switch.

        - Group Control:

          - 1 Motor Isolating Relay (GC4) - One ISO relay per motor. Allows 110-120V group switching. Allows for up to 12 motors to be operated from on one a single 15 amp double throw switch, determined by motor size and circuit rating. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps.

          - 1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 2-3/4 inched by 3-1/2 inches by 2-1/8 inches (70 by 90 by 54 mm). Enclosure: 4 inches by 4 inches by 4 inches (102 by 102 by 102 mm).4 Motor Isolating Relay (GC4) - Panelized version with four ISO relays. Up to three 4ISOs can be linked together for control of up to 12 motors from any single 15 amp double throw wall switch, determined by motor size and circuit rating. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps. Installed in a 8 inch by 8 inch by 4 inch (203 by 203 by 102 mm) box.

          - 4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 3-1.2 inches by 7 inches by 1-7/8 inches (89 by 178 by48 mm). Enclosure: 6 inches by 4 inches by 4 inches (152 by 102 by 102 mm).

          - IGC4N1. Individual and Group control of up to four motors with up to three intermediate positions per motor. Each motor output can be individually addressed and mapped to any of the 5 wall switch input ports using onboard dip switches.

** NOTE TO SPECIFIER ** Standard Motor: 110V AC tubular smart motor with 3-wire pigtail. All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces. Only those controls listed in this section will operate this motor.

      - FlexShade 2 I/O AC.

        - IntelliFlex I/O AC smart motor: 120V AC, single phase, 60 HZ, 3-wire, 6nm or 15nm (at manufacturer's discretion), instantly reversible, lifetime lubricated smart motor. Equipped with internal thermal overload protector, and electric brake.

          - Thermal protection: 4 minutes.

          - Rated current: 1.0 Amps. Motors drawing greater than 1.0 amps shall not be acceptable without prior approval by Design Professional responsible for electrical design of project.

          - Speed: 28RPM. All motors within a discrete control zone must operate at same speed.

          - Sound level: 42dBA.

          - UL Listing: UL325 & CSA C22.2 No. 247. Motors not listed as part of an entire shade assembly and control system shall not be acceptable.

          - Encoded motors with two-way communication to network control devices. Motor/control networks that do not support two-way communication shall not be acceptable.

          - Limits: Intermediate stopping points and presets.

** NOTE TO SPECIFIER ** Most IntelliFlex I/O motors, interface, and input devices automatically include a Network Device Connector. NDC is required to connect a device to the network and distribute power and communications between devices. Network devices allow input from external sources such as sensors and third party devices. Delete those not required. 1 Network Device Connector is supplied with each input device.

        - IntelliFlex I/O Network Devices

          - Network Device Connector. Connects lntelliFlex I/O devices into a network. Distributes power and communications between devices. One provided with each IntelliFlex I/O motor and network device. Approved for use in plenum spaces. LED Indicators indicate network status. Includes built-in terminating resistor, and network bypass switch for troubleshooting. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 3-3/8 inches by 2 inches by 1-1/16 inches (90 by 31 by 27 mm). Mounting hardware provided. Indoor use only.

NOTE TO SPECIFIER ** Select the network devices required from the following paragraphs and delete those that are not required. Note that a IntelliFlex I/O Wireless Network Gateway is required to connect wireless devices to the wired network.

          - Wireless Network Gateway. Allows wireless devices to control shades on the wired network. Required when using Radio Frequency handheld remote. LEDs indicate the status of both the wired and wireless networks. Allows wireless control of the entire network from a single receiving point. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 3-3/8 inches by 2 inches by 1-1/16 inches (85 by 52 by 27 mm). Mounting hardware provided. Indoor use only.

          - AV Gateway. Used to integrate with third party control systems. Single entry point must support RS-232 and RS-485 and contain at least 6 contact closure inputs, low voltage trigger, and IR receiver input. Approved for use in plenum spaces. LCD display allows for commissioning of serial groups. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4-7/8 inches by 3 inches by 1 inch (124 by 83 by 32 m). Mounting hardware provided. Indoor use only.

          - Contact Closure Interface. Allows up to 6 contact closure inputs to be connected to the network. These are grouped into 3 sets of Up/Down signals, each of which can control any number of shades on an lntelliFlex I/O network. Configuration must be done using onboard programming buttons, without rewiring or the use of external configuration devices. Power is provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 3 inches by 3 inches by 1 inch (76 by 76 by 25 mm). Mounting hardware provided. Indoor use only.

          - Sensor/Schedule Interface. Allows up to 4 sensors to be added to an IntelliFlex I/O network. Must include built-in support for brightness, temperature, wind, and precipitation sensors, and pyranometers. Must support up to 8 scheduled override events. Configuration must be done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. External power input for sensors provides support for any sensor power requirements. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4-7/8 inches by 3-1/4 inches by 1-1.4 inched (124 by 83 by 32 m). Mounting hardware provided. Indoor use only.

            - SSI Transformer. To be used when adding third party sensors to the network.

          - Sensor/Schedule Interface with Automated Glare Control. The IntelliFlex I/O Sensor / Scheduler Interface with automated glare control allows up to 4 sensors to be added to an IntelliFlex I/O network. Includes built-in support for brightness, temperature, wind, and precipitation sensors as well as pyranometers. Also provided support for up to 8 scheduled override events. Configuration is done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. Configuration using built-in buttons and LCD display. Scheduled events can be configured to run on any combination of days of the week. External power input for sensors provides support for any sensor power requirements. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4-7/8 inches by 3 inches by 1 inch (124 by 83 by 32 m). Mounting hardware provided.

          - Central Network Controller. Touchscreen device to add advanced functionality to a network. It also acts as a router between the control network and a building IP network. Graphical user interfaces on local touchscreen and remote access web interfaces allow for the monitoring and override of motors and network devices. Automated glare control and scheduling. BACnet/IP integration. Interface to monitor and override network devices. Allows logging of shade movements, sensor readings, and user overrides. Input power: Micro USB. Operating temperature: ambient. Dimensions: 9-7/8 inches by 5-1/4 inches by 2 inches (251 x 146 x 51 mm). Rough cutout: 8 inches by 5 inches by x 1-3/4 inches (203 by 127 by 44 mm). Mounting hardware provided.

          - IP Gateway. For connecting a network of IntelliFlex I/O devices to an IP network. Built-in two port network switch to allow direct connection to another IP Gateway creating a standalone network for access to all connected IntelliFlex I/O devices. Each IPG is a BACnet/IP device and allows for an additional 120 devices to be added to the network. Configuration is done using the built-in buttons and display. Material: ABS plastic. Operating temperature: ambient. Dimensions: 4-7/8 inches by 3 inches by 1 inch (124 by 83 by 32 mm). Mounting hardware provided.

** NOTE TO SPECIFIER ** Select required input device option paragraphs and delete the ones not required.

        - IntelliFlex I/O User Input Devices.

          - Single Zone Wall Switch. Allows user to move roller shades to any position with a single touch. Swipe gestures must be recognized to move shades to fully open or closed positions. Power provided through lntelliFlex I/O network using a single Ethernet cable for power and communication. Switch shall be capable of controlling any number of shades on an lntelliFlex I/O network using onboard programming buttons without rewiring or the use of external configuration devices. Must allow unlimited number of preset alignment positions. Push button switches not permitted.

          - Dual Zone Wall Switch. Allows user to control two independent zones of roller shades, moving them to any position with a single touch. Swipe gestures must also be recognized to move shades to fully open or closed positions. Power for switch must be provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication. Each zone must be configured to control any number of shades on an lntelliFlex I/O network using the onboard programming buttons, without rewiring or the use of external configuration devices. Unlimited number of preset alignment positions. Push button switches not permitted.

** NOTE TO SPECIFIER ** IntelliFlex I/O Wireless Gateway is required with this device.

          - Radio Frequency Handheld Remote Control. Display must allow for any number of channels, up to 32 channels. Touch control to position the shades anywhere with one touch. Must be capable of serving as a simple commissioning tool. lntelliFlex I/O Wireless Network Gateway is required with remote. Range is 40 feet from closest IntelliFlex I/O device. Push button remotes not permitted.

** NOTE TO SPECIFIER ** IntelliFlex I/O AV Gateway is required with this device.

          - Infrared (IR) Remote Control Kit. Includes handheld remote transmitter and IR receiver. For use with AV Gateway.

** NOTE TO SPECIFIER ** Select required optional sensors and delete the ones not required. IntelliFlex I/O Sensor/Schedule Interface required for sensors.

        - IntelliFlex I/O Sensors.

          - External Brightness Sensor. For mounting on the exterior side of the window. For use with Sensor/Schedule Interface.

          - Wind Speed Sensor. For use with Sensor/Schedule Interface.

          - Internal Brightness Sensor. To be mounted on the interior side of the window with adhesive or with provided mounting bracket. For use with Sensor/Schedule Interface.

** NOTE TO SPECIFIER ** Radio Technology Motor: Motor with built-in radio receiver. 3-wire pigtail allows for attachment of optional three prong plug for plug-an-play operation. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor.

      - FlexShade 2 RTS AC.

        - Standard Motor: 120V AC motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld transmitter-White.

          - Single channel wireless handheld transmitter-Black.

          - Four channel wireless handheld transmitter-White.

          - Four channel wireless handheld transmitter-Black.

          - 16-Channel wireless handheld transmitter-White.

          - 16-Channel wireless handheld transmitter-Silver.

          - Single channel wireless wall switch for radio motor control-White.

          - Single channel wireless wall switch for radio motor control-Ivory.

          - Four channel wireless wall switch for radio motor control-White.

          - Four channel wireless wall switch for radio motor control-Ivory.

          - RS-232/Dry Contact - Allows for full control of radio motor shade via RS-232/dry contact interface.

          - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

          - Wireless Timer - Allows operation of one group of radio motors per timer.

          - Repeater, 1 per project to extend range of transmitters.

          - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

      - FlexShade 2 485 AC.

        - 120V AC motor with built-in low voltage controller with 3-wire pig tail and data cable. No external motor controls are required. Available with optional three prong plug. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select the control device(s) required and delete those not required.

        - Individual Control, Group Control & Individual and Group Control:

          - 6 button RS 485 switch- for 2 groups up/stop/down or 1 group up/stop/down and 3 presets.

          - 8 button RS 485 switch- for 5 groups up/stop/down or 1 group up/stop/down and 5 presets.

          - Somfy Digital Network. SDN system of window shades with intelligent motors linked to an IP interface box and other input devices for automated glare control.

          - Bus Distribution

            - Data Hub for 485 Motor

            - 5-Way RJ45 Splitter

            - Building Controllers

            - SDN Data Panel

          - User Interfaces

            - Somfy Connect BMS Interface

            - Somfy Connect LTI

            - Z Wave Digital Motor Interface (ZDMI)

            - Zigbee to RS485 (ZBDMI)

            - RTS Receiver for 485

            - SDN 6 button wall switch (Preprogrammed for master control)

            - SDN 8 button wall switch

            - Somfy Connect UAI Plus

          - Accessories

            - Bus Power Supply, 24V, 1A

          - 32 point BACnet interface- Control of up to 32 groups with feedback via BACnet over IP.

      - Roller: Size 4 inch (102 mm) or 5 inch (127 mm) O.D. steel tube. Size at manufacturer's discretion.

      - Roller Idler Assembly: Zinc-plated steel shaft supported by roller bearings in an aluminum housing.

      - Slat: 13/16 inch aluminum dowel, encased in heat seamed hem.

      - Mounting

        - Endcaps: 1028 steel stamping. Complete with adapter roller bracket. Installs to wall, ceiling, or jamb. Accepts fascia or headbox.

        - Brackets: 1018 plated steel stamping, suitable for mounting to wall, ceiling, or jamb. Brackets do not require additional adapters.

        - Fascia: 6-1/2 inch by 6-1/2 inch (165 by 165 mm) square cover of extruded aluminum, 0.125 inch (3.175 mm) wall.

          - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind.

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by Design Professional.

        - Headbox Ceiling/Wall Style: 6-5/8 inches by 6-5/8 inches (168 by 168 mm) square fascia and L-shaped back/top cover to be fabricated of extruded aluminum:

          - Finish: Selected from Manufacturers standard range.

          - Finish: Custom powder coat as selected by the Architect.

        - Pocket headbox constructed of extruded aluminum. Bottom of headbox fully enclosed except for slot allowing fabric passage. Bottom closure panel shall be extruded aluminum, finished white, and may be removed manually for access to roller and drive unit. Aluminum endplates with integrated steel brackets for suspended mounting shall be fastened to extruded headbox with thread-cutting screws.

          - Ceiling Tile Flange: 12 foot (366 cm) extruded aluminum, finished white. Attaches to pocket headbox. Hardware for attachment to headbox included.

          - Endcap Ceiling Tile Flange: Extruded aluminum, finished white. Available on one or both endcaps.

** NOTE TO SPECIFIER ** Use the following paragraphs to specify large Draper Motorized FlexShades up to 25 feet (7.62 m) wide

  - LARGE MOTORIZED WINDOW SHADES

    - Type: FlexShade XL Motorized Roller shading systems: Electrically operated, motorized, vertical roll-up, fabric window shade. Assembly to include motor operator, controls, mounting hardware and other components necessary for complete installation;

    - Roller: 2.364 inch (60 mm) outside diameter aluminum. Roller assembly is integrated with floating bracketry

      - Roller Idler Assembly: Plastic idle end insert into roller will fit over steel pin and rotate on pin with use of sealed bearing. Steel pin connects to bearing mounted to bracket mounted inside the headbox. Systems not using sealed bearings are not acceptable.

      - Fabric Connection to Roller Tube: Fabric panel to incorporate spline attachment at the top and hem pocket at the bottom to allow connection to the hem bar.

      - Slat: 13/16 inch aluminum dowel, encased in heat seamed hem.

      - Cradle System: Extrusion supports entire roller using cradle system to reduce roller deflection

    - Headbox: Cassette-style extruded aluminum sections

      - Size: 5 inches high by 5 inches deep by length required for shade being provided.

** NOTE TO SPECIFIER ** Select the shape and finsh required from the following paragraphs and delete those not required.

      - Shape: Angled

      - Shape: Curved.

      - Finish: Selected from Manufacturers standard range.

      - Finish: Custom powder coat as selected by the Architect

    - Electric Operator: Tubular motor concealed inside shade roller tube;

** NOTE TO SPECIFIER ** Select radio technology or standard wired motor from the following paragraphs and delete those not reuired. Coordinate motor selection with controls specified below

      - Standard Motor: Sonesse Radio Technology Motor: 110-120V AC, single-phase, 60 HZ, instantly reversible, lifetime lubricated motor with built-in radio receiver, equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. (Compatible with Wireless Radio Technology Controls.)

      - Wired Motor: Sonesse 110-120V AC, single-phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Compatible with Intelliflex Controls.

      - Location:

** NOTE TO SPECIFIER ** Indicate operator location as seen from inside the room facing the window. Right hand motor location is standard

        - Right hand

        - Left hand

        - As scheduled

** NOTE TO SPECIFIER ** Select Standard Motor Controls below if standard motor is not specified as electric operator in motorized roller window shades article above. delete if not applicable.

      - RTS Motor Controls:

        - Individual Control, Group Control and Individual and Group Control:

        - Single channel wireless handheld radio frequency transmitter.

        - Single channel wireless handheld transmitter-Black.

        - Four channel wireless handheld transmitter-White.

        - Four channel wireless handheld transmitter-Black.

        - 16-Channel wireless handheld transmitter-White.

        - 16-Channel wireless handheld transmitter-Silver.

        - Single channel wireless wall switch for radio motor control-White.

        - Single channel wireless wall switch for radio motor control-Ivory.

        - Four channel wireless wall switch for radio motor control-White.

        - Four channel wireless wall switch for radio motor control-Ivory.

        - RS-232/Dry Contact - Allows for full control of radio motor shade via RS-232/dry contact interface.

        - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

        - Wireless Timer - Allows operation of one group of radio motors per timer.

        - Repeater, 1 per project to extend range of transmitters.

        - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

** NOTE TO SPECIFIER ** Delete RTS Motor Controls article above if standard motor is not specified as electric operator in motorized roller window shades article above

      - Wired Motor Controls:

        - Individual Control:

          - Wall Switch - Toggle three position wall switch.

          - Wall Switch - Key operated three position wall switch.

        - Group Control:

          - 1 Motor Isolating Relay (1ISO) - One ISO relay per motor. Allows 110-120V group switching. Allows for up to 12 motors on one double throw switch. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps.

          - 1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 2-3/4 inched by 3-1/2 inches by 2-1/8 inches (70 by 90 by 54 mm). Enclosure: 4 inches by 4 inches by 4 inches (102 by 102 by 102 mm).

          - 4 Motor Isolating Relay (GC4) - Panelized version with four ISO relays. Up to three 4ISOs can be linked together for control of up to 12 motors from any single 15 amp double throw wall switch. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps. Installed in a 8 inch by 8 inch by 4 inch (203 by 203 by 102 mm) box.

          - 4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 3-1.2 inches by 7 inches by 1-7/8 inches (89 by 178 by48 mm). Enclosure: 6 inches by 4 inches by 4 inches (152 by 102 by 102 mm).

        - Individual and Group Control: IGC4N1. Individual and Group control of up to four motors with up to three intermediate positions per motor.

    - Fabric:

** NOTE TO SPECIFIER **    Fabrics for Motorized FlexShade XL are limited. Select the shape and fabric type required from the following paragraphs and delete those not required.

      - 1%

      - 3%

      - 5%

      - Soltis 92

** NOTE TO SPECIFIER ** Draper LightBloc FlexShade System can be manually or motor operated. Manual shades are provided in two operating styles: chain and clutch, and crank. Refer to Draper product literature for selection guidelines. Select operation type from the following. Delete those not required. If multiple types are required indicate the location of each in the schedule at the end of this section.

  - OPAQUE WINDOW SHADE SYSTEM

    - Operation Type: Motorized, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

    - Operation Type: Bead chain and clutch operated, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

    - Operation Type: Crank operated, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

** NOTE TO SPECIFIER ** For Chain and Clutch Operated LightBloc FlexShade System include the following paragraph. Delete if not required

    - Operation: Bead chain and clutch operating mechanism allowing shade to stop when chain is released. Designed never to need adjustment or lubrication. Provide limit stops to prevent shade from being raised or lowered too far.

      - Clutch mechanism: Fabricated from high carbon steel and molded fiberglass reinforced polyester.

** NOTE TO SPECIFIER ** Select stainless steel or plastic bead chain from the following. Delete the one not required.

      - Bead chain loop: Stainless steel bead chain hanging at side of window.

      - Bead chain loop: Plastic bead chain hanging at side of window, Ivory, Grey, or Black color as selected by Architect.

      - Idler Assembly: Provide roller idler assembly of molded nylon with adjustable length idler pin to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER **For Crank Operated LightBloc FlexShade System include the following paragraph. Delete if not required.

    - Operation: Crank and gear box operated shades.

      - Gear box: Die cast aluminum and steel construction. Injection molded nylon and steel crank insert installed in roller to engage gear box.

      - Crank handle:

** NOTE TO SPECIFIER ** Crank handle can be detachable or permanently mounted, select one of the following paragraphs.

        - Detachable anodized aluminum and chrome plated steel handle.

        - Permanently mounted anodized aluminum and chrome plated steel handle.

** NOTE TO SPECIFIER ** Crank handle can be provided in lengths up to 10 feet., select one of the following paragraphs.

        - Size: 4 feet (122 cm).

        - Size: 6 feet (183 cm).

        - Size: 10 feet (305 cm).

      - Idler assembly: Provide roller idler assembly of molded nylon with adjustable length idler pin or spring-loaded idler to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER ** For Motorized LightBloc FlexShade System include and edit the following paragraphs as required. Delete if not required.

    - Shade Motor and Control System:

** NOTE TO SPECIFIER ** Select one of the following motor option paragraphs and delete the ones not required. Contact Draper for more information concerning motor limitations.

      - Standard Motor: 110 VAC, single phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Tubular motor concealed inside each shade roller tube.

      - Quiet Standard Motor - 110 VAC motor operates at 38 dBA measured 3 feet from the motor. Makes no audible clicks when motor stops or starts. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select one of the following control option paragraphs and delete the ones not required. For each control selected select the control device(s) required and delete those not required.

        - Individual Control:

          - Wall Switch - Toggle three position wall switch.

          - Wall Switch - Key operated three position wall switch.

        - Group Control:

          - Motor Isolating Relay (1ISO) - One ISO relay per motor. Allows 110-120V group switching. Allows for up to 12 motors on one double throw switch. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps.

          - 1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 2-3/4 inched by 3-1/2 inches by 2-1/8 inches (70 by 90 by 54 mm). Enclosure: 4 inches by 4 inches by 4 inches (102 by 102 by 102 mm).

          - 4 Motor Isolating Relay (GC4) - Panelized version with four ISO relays. Up to three GC4s can be linked together for control of up to 12 motors from any single 15 amp double throw wall switch. Input Voltage: 120V AC/60Hz. Output Voltage: 120V AC/60Hz. Max Output Current: 4.0 Amps. Installed in a 8 inch by 8 inch by 4 inch (203 by 203 by 102 mm) box.

        - 4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 degree F (0 - 60 degree C). Dimensions: 3-1.2 inches by 7 inches by 1-7/8 inches (89 by 178 by48 mm). Enclosure: 6 inches by 4 inches by 4 inches (152 by 102 by 102 mm)

          - IGC4N1. Individual and Group control of up to four motors with up to three intermediate positions per motor.

      - Radio Technology Motor- 110 VAC motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

      - Quiet Radio Technology Motor - 110 VAC motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

** NOTE TO SPECIFIER ** Select one of the following control option paragraphs and delete the ones not required. For each control selected select the control device(s) required and delete those not required.

        - Individual Control, Group Control and Individual and Group Control:

          - Single channel wireless handheld radio frequency transmitter.

          - Single channel wireless handheld transmitter-Black.

          - Four channel wireless handheld transmitter-White.

          - Four channel wireless handheld transmitter-Black.

          - 16-Channel wireless handheld transmitter-White.

          - 16-Channel wireless handheld transmitter-Silver.

          - Single channel wireless wall switch for radio motor control-White.

          - Single channel wireless wall switch for radio motor control-Ivory.

          - Four channel wireless wall switch for radio motor control-White.

          - Four channel wireless wall switch for radio motor control-Ivory.

          - RS-232/Dry Contact - Allows for full control of radio motor shade via RS-232/dry contact interface.

          - Wireless Sun Sensor - Allows operation of one group of radio motors per sensor.

          - Wireless Timer - Allows operation of one group of radio motors per timer.

          - Repeater, 1 per project to extend range of transmitters.

          - (URTSI II) Universal Interface - Allows for full control of radio motor shade via RS-232/485 or IR signals.

    - Roller Tube: Fabricated from extruded aluminum, galvanized steel, or enameled steel. Diameter, wall thickness, and material selected by manufacturer to accommodate shade type and size. Fabric connected to the roller tube with LSE (low surface energy) double sided adhesive specifically developed to attach coated textiles to metal. Adhesive attachment to eliminate horizontal impressions in fabric.

** NOTE TO SPECIFIER ** Draper LightBloc FlexShade System is provided in two configurations: one for exposed surface installation and the other for recessed pocket installation. Delete the one not required.

    - Headbox: Consists of extruded aluminum sections with endcaps and opacity plates.

      - Size: 4-1/8 inches (105 mm) high by 3-1/2 inches (89 mm) wide by length required for shade being provided.

** NOTE TO SPECIFIER ** Include the following paragraph for exposed headbox. Delete if not required.

      - L-shaped removable front face and bottom cover and L-shaped back and top.

** NOTE TO SPECIFIER ** Include the following paragraph for recessed headbox. Delete if not required.

      - U-shaped front, back, and top and removable bottom.

    - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

    - Side Channels: Double chamber fabricated from 0.06 inch (1.5 mm) thick extruded aluminum sections. One chamber accepts fabric and contains groove for fabric retainer. Other chamber accepts fabric guide and channel locator.

    - Sill channel: 0.06 inch (1.5 mm) thick extruded aluminum channel to receive slat bar and prevent light leakage.

    - Slat bar: Extruded aluminum bar attached to bottom of shade. Bar does not retract into headbox.

    - Channel locator: Injected molded nylon insert to align side and sill channels with headbox.

    - Fabric guide: Plated steel transition for fabric rolling into side channel.

** NOTE TO SPECIFIER ** LightBloc FlexShades with SB9000 and SB9100 fabric use fabric retainers, that includes a patented system which enables shade to withstand reasonable air pressure differentials. However, extreme air currents or physical interference cannot be resisted by fabric retention system and may cause physical damage. Refer to Draper product literature for selection guidelines. (Available with SB9000 and SB9100 fabrics only).

    - Fabric retainer: System designed to prevent disengagement of fabric from side channels due to normal variations of air pressure caused by doors opening, HVAC systems, and temperature differences between room and window well. System consists of horizontal steel stays installed in shade, covered with fabric, and spaced at regular intervals. Grommets installed through stays are held within groove of side channel chamber.

    - Exposed aluminum finish:

** NOTE TO SPECIFIER ** Select the finish required from the following paragraphs and delete those not required.

      - Finish: Selected from Manufacturers standard range.

      - Custom powder coat as selected by the Architect.

  - FABRIC

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabrics and selection guidelines. Select fabric(s), percent open and color as applicable from the following and delete those not applicable. Use the schedule at the end of this section if required to define location(s) of multiple fabrics. Note that for dual roller shades, choose two, and indicate which is for front roller and which is for back roller.

    - Light-Filtering Fabrics

      - PVC Coated Fiberglass

        - Basketweave

          - E Screen 1% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G21. 1 percent open, .020 inches thick, 13.3 oz/square yard.

          - E Screen 3% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G21. 3 percent open, .017 inches thick, 11.6 oz/square yard.

          - E Screen 5% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G21. 5 percent open, .016 inches thick, 10.7 oz/square yard.

          - E Screen 10% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 10 percent open, .020 inches thick, 10.3 oz/square yd.

          - E Screen 1% with KOOLBLACK by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .021 inches thick. Weight: 12.95 oz/square yard.

          - E Screen with KOOLBLACK. 3 percent open factor by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .020 inches thick, 11.3 oz. per square yard.

          - E Screen with KOOLBLACK. 5 percent open factor by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .019 inches thick, 10.6 oz/square yard.

          - E Screen Deco 1% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .022 inches thick. 12.30 oz/square yard.

          - E Screen Deco 3% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .019 inches thick. 11.50 oz/square yard.

          - E Screen Chroma by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave with metalized coating on outward side. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .020 inches thick. 11.92 oz/square yard.

          - Natte 5% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5% open. Thickness: 0.022 inches. Weight: 14.13 oz/square yard.

          - Natte 10% by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5% open. Thickness: 0.021 inches. Weight: 13.48 oz/square yard.

          - S Screen 1% by Mermet: PVC coated fiberglass wrapped in polyester thread in a plain weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .041 inches thick, 19.10 oz/square yard.

          - S Screen 4% by Mermet: PVC coated fiberglass wrapped in polyester thread in a plain weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. ASTM E 2180 and ASTM G 21. Environmental benefits: RoHS - lead free. 4 percent open, .037 inches thick, 18.9 oz/square yard.

          - S Screen Naturals 5% by Mermet: PVC coated fiberglass wrapped in polyester thread in a fancy weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .042 inches thick 15 oz/square yard.

          - SheerWeave Series SW2000 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2000 5 percent open.

          - SheerWeave Series SW2100 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2100 10 percent open.

          - SheerWeave Series SW2400 (2410) by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open .019 inches thick. 14.1 oz/square yard.

          - SheerWeave Series SW2500 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

          - SheerWeave Series SW2600 (2360) by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open, .017 inches thick. 10.4 oz/square yard.

          - SheerWeave Series SW2900 (2390) by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

          - SheerWeave Basic 3% by Phifer. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal ResistanceASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent Open. Average Fabric Thickness: .025 inch. Average Fabric Weight: 16.4 ounces per square yard.

          - SheerWeave Basic 5% by Phifer. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal ResistanceASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent Open. Average Fabric Thickness: 0.022 inch. Average Fabric Weight: 14.1 ounces per square yard.

          - SheerWeave Performance+ 2500 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

          - SheerWeave Performance+ 2410 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open .019 inches thick. 14.1 oz/square yard.

          - SheerWeave Performance+ 2390 by Phifer: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

        - Ribbed Weave

          - M Screen 1% by Mermet: PVC coated fiberglass in 1 by 2 ribbed weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .022 inches thick, 13.12 oz/square yard.

          - M Screen 3% by Mermet: PVC coated fiberglass in 1 by 2 ribbed weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .022 inches thick, 12.27 oz/square yard.

          - M Screen 5% by Mermet: PVC coated fiberglass in 1 by 2 ribbed weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .022 inches thick, 11.3 oz/square yard.

          - M Screen Deco 5% by Mermet: PVC coated fiberglass in 1 by 2 ribbed weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .020 inches thick, 11.3 oz/square yd.

          - SheerWeave Series SW1000 SunScreen by Phifer. PVC coated fiberglass in a ribbed weave. Fire Classification: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large scale). Bacteria and Fungal Resistance: ASTM E 2180, ASTM G21, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental: Certified to UL GREENGUARD and GREENGUARD Gold standards. Lead Free: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 25% Open.

          - SheerWeave Series SW3000 by Phifer: Vinyl coated fiberglass and vinyl coated polyester woven into 62 by 20 mesh. Fire rating: NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 13.46 oz/sq yd, .028 inches thick. 14 percent open.

          - SilverScreen 4% by Verosol: 36 percent fiberglass and 64 percent vinyl with an ultra-fine layer of aluminum on the backside. Fire rating: NFP 92503 specification M1, NFPA 701, B5 5867 Part 2 Type B, AS 1530. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - Lead Free, Formaldehyde free. 5 percent Open, 0.020 inches thick, 11.7974 oz/square yard.

        - Satin Weave

          - T Screen with KOOLBLACK 1% by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .024 inches thick, 13.54 oz/square yard.

          - T Screen with KOOLBLACK 3% by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .029 inches thick, 13.27 oz/square yard.

          - T Screen with KOOLBLACK. 5 percent open factor by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUAR® and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .027 inches thick, 12.48 oz/ square yard.

          - T Screen Deco 1% by Mermet: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .028 inches thick, 13.76 oz/square yd.

          - T Screen Deco 3% by Mermet: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .028 inches thick, 12.98 oz/square yard.

          - T Screen Deco 5% by Mermet: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 5 percent open, .028 inches thick, 11.89 oz/square yard.

        - Twill Weave

          - 9803 3% by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 3 percent open, .025 inches thick, 11.6 oz/square yard.

          - SheerWeave Series SW2701 by Phifer: Duplex basketweave fabric-light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open. .027 inches thick. 14.6 oz/square yard.

          - SheerWeave Series SW2703 by Phifer: Duplex basketweave fabric-light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, CAN/ULC S-102. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. 3 percent open. .028 inches thick. 14.0 oz/square yard.

          - SheerWeave Series SW2705 by Phifer: Duplex basketweave fabric-light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Gold. Manufacturer to supply GREENGUARD Gold certificate. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .025 inches thick. 11.8 oz/square yard.

          - SheerWeave Series SW2710 by Phifer: Duplex basketweave fabric-light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open. .023 inches thick. 10.4 oz/square yard.

          - T Screen 1% by Mermet: PVC coated fiberglass woven mesh. White, charcoal, or pearl backing color. Fire rating: NFPA 701-15 TM#1, California U.S. Title 19 CAN/ULC-S109-14 Small Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. 1 percent open, .028 inches thick, 13.83 oz/square yard.

      - Fiberglass

        - Mock Leno Weave

          - GreenScreen Nature 5%: GreenScreen Nature is an inherently fire-retardant fabric constructed of 100 percent fiberglass yarn in a mock leno weave. Additional fire retardancy chemicals are not required. Recyclable, halogen- and PVC-free. Red List compliant. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109 Small & Large Scale. Microbial Resistance, ASTM E 2180 and ASTM G 21: Pass. UL GREENGUARD, GREENGUARD Gold. 5 percent open. Average Fabric Thickness: 0.010 inch (.024 mm). Average Fabric Weight: 5.10 ounces per square yard (174 grams per square meter).

        - Ribbed Weave

          - Vizela < 1% by Mermet: 100 percent fiberglass with an EVA coating and PVC free. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E 2180, ASTM G 21. < 1% open, .013 inches thick, 8.7 oz/square yard.

      - PVC Coated Polyester

        - Basketweave

          - SunTex 90 by Phifer. Vinyl-coated polyester woven into a basketweave. For outdoor use. Fire Classification: NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), CFR 49V 571.302 (FMVSS 302), California Technical Bulletin 117, CAN/ULC S-109 (large scale) and ASTM E-662. Bacteria and Fungal Resistance: ASTM E 2180 and ASTM G21; includes Microban antimicrobial additives. Environmental: Certified to UL GREENGUARD and GREENGUARD Gold standards. Lead Free: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10% open. Thickness: 0.039 inches. Weight: 17.2 oz/square yard.

          - SunTex 90 Design by Phifer. Vinyl-coated polyester woven into a basketweave. For outdoor use. Fire Classification: NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), CFR 49V 571.302 (FMVSS 302), California Technical Bulletin 117, CAN/ULC S-109 (large scale) and ASTM E-662. Bacteria and Fungal Resistance: ASTM E 2180 and ASTM G 21; includes Microban antimicrobial additives. Environmental: Certified to UL GREENGUARD and GREENGUARD Gold standards. Lead Free: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10% open. Thickness: 0.039 inches. Weight: 17.2 oz/square yard.

          - SunTex 95 by Phifer. Vinyl-coated polyester woven into a basketweave. For outdoor use. Fire Classification: NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), CFR 49V 571.302 (FMVSS 302), California Technical Bulletin 117, CAN/ULC S-109 (large scale) and ASTM E-662. Bacteria and Fungal Resistance: ASTM E 2180 and ASTM G 21; includes Microban antimicrobial additives. Environmental: Certified to UL GREENGUARD and GREENGUARD Gold standards. Lead Free: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5% open. Thickness: 0.029 inches. Weight: 15.4 oz/square yard.

          - SunTex 97 by Phifer. Vinyl-coated polyester woven into a basketweave. For outdoor use. Fire Classification: NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), CFR 49V 571.302 (FMVSS 302), California Technical Bulletin 117, CAN/ULC S-109 (large scale) and ASTM E-662. Bacteria and Fungal Resistance: ASTM E 2180 and ASTM G 21; includes Microban antimicrobial additives. Environmental: Certified to UL GREENGUARD and GREENGUARD Gold standards. Lead Free: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3% open. Thickness: 0.033 inches. Weight: 16.0 oz/square yard.

          - SheerWeave Series PW3500 (4000) by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. 5 percent open, 19.2 oz/sq yd, .036 inches thick.

          - SheerWeave Series PW4100 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G 21, ASTM G 22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. 10 percent open, 17.5 oz/sq yd, .035 inches thick.

          - SheerWeave Series PW4400 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental Certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. 3 percent open, 20.7 oz/sq yd, .037 inches thick.

          - SheerWeave Series PW4500 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD UL Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open, 14.4 oz/sq yd, .024 inches thick.

          - SheerWeave Series PW4600 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open, 17.4 oz/sq yd, .030 inches thick.

          - Soltis Harmony 88 by Ferrari. PVC-coated polyester with pre-tensioning process for dimensional stability. Flame Retardancy: B-s2,d0 / EN 13501-1, M1/NFP92-507, B1/DIN 4102-1, BS 7837, BS 5867, Schwerbrennbar Q1-Tr1/ONORM, A3800-1, CAN/ULC-S 109, M1/UNE 23727-90, Classe 1/EN 13773, CSFM T19, Method 1 & 2/ NFPA 701, Class A/ASTM E84, 1530.3/AS NZS, Group 1/AS NZS 3837, G1/GOST.30244.94. Environment: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 8 percent open, .018 inches thick, 10.61 oz/square yard.

          - Soltis Horizon 86 by Ferrari. PVC-coated polyester with pre-tensioning process for dimensional stability. Flame Retardancy: B-s2,d0 / EN 13501-1, M1/NFP92-507, B1/DIN 4102-1, BS 7837, BS 5867, Schwerbrennbar Q1-Tr1/ONORM, A3800-1, CAN/ULC-S 109, M1/UNE 23727-90, Classe 1/EN 13773, CSFM T19, Method 1 & 2/ NFPA 701, Class A/ASTM E84, 1530.3/AS NZS, Group 1/AS NZS 3837, G1/GOST.30244.94. Environment: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 14 percent open, .018 inches thick, 11.2 oz/square yard.

          - Soltis Master 99 by Serge Ferrari: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1/NFPA 701, CSFM T19, Class A/ ASTM E 84. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 3 percent open, .0156 inches thick, 8.6 oz/ square yard.

          - Soltis Perform 92 by Serge Ferrari: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1/NFPA 701, CSFM T19, Class A/ ASTM E84. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 4 percent open, .018 inches thick, 12.4 oz/square yard.

        - Decorative Weave

          - SheerWeave Series SW5000 by Phifer: Polyester and vinyl coated polyester woven into a 42 by 20 mesh. Fire rating: California Technical Bulletin 117 Sect. E. Part 1/ /UBC (Class 1)/British Standard 5867. Varies depending on color. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, ASTM D3273, AATCC 30 Part 3, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 12.3 to14.5 oz/sq yd, .035 inches - .044 inches thick, depending on color. 5 to 10 percent open, depending on color.

        - Plain Weave

          - SheerWeave PW4800 by Phifer: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open. 18.5 oz/sq yd, .036 inches thick.

        - Twill Weave

          - SheerWeave Series PW4550 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal Resistance. ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open, 12.5 oz/sq yd, .025 inches thick.

          - SheerWeave Series PW4650 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal Resistance. ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open, 13.3 oz/sq yd, .026 inches thick.

          - SheerWeave PW4901 by Phifer: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. 1 percent open. 15.85 oz/sq yd, .028 inches thick.

          - SheerWeave PW4903 by Phifer: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, UL GREENGUARD Mold and Bacteria Standard ASTM 6329; includes Microban antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. 3 percent open. 14.9 oz/sq yd, .027 inches thick.

      - Polyester

        - Basketweave

          - SheerWeave Series SW8000 by Phifer: PVC-free polyester. Fire rating: California U.S. Title 19 (small scale), BS 5867 2008 Part 2 Type B Performance, IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, B1 (DIN 4102-1), NFPA 101 (Class A Rating). Bacteria and fungal resistance: ASTM E 2180, ASTM G21. Environmental certification: Cradle to Cradle Certified Bronze and certified to ul GREENGUARD and GREENGUARD Golld standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101and REACH (EC 1907/2006) compliant. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. 3% open. .019 inches thick. 7.35 oz/square yard.

        - Decorative Weave

          - SheerWeave Series SW7450 by Phifer: PVC-free polyester with acrylic coating. Fire rating: California Technical Bulletin 117, IBC Section 803.1.1 (Class A Rating), NFPA 101 (Class A Rating). Bacteria and Fungal Resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. .026 inches thick, 6.54 oz/square yard.

        - Fancy Weave

          - Verona Daylight by Mermet. 100% PVC-free polyester. Fire Classifications: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Bacterial Resistance: ASTM E2180, ASTM G21. Environment: RoHS - Lead Free, certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. 1% open. .021 inches thick, 5.90 oz/square yard.

        - Diamond Knit

          - GreenScreen Evolve 3%: GreenScreen Evolve is a fire retardant PVC-free fabric is constructed of 100 percent polyester yarn, with up to 78% Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, UL GREENGUARD, GREENGUARD Gold, PVC Free. 3 percent open. Average Fabric Thickness: 0.027 inch (.068 mm). Average Fabric Weight: 8.41 ounces per square yard. Microbial Resistance, ASTM E2180 and ASTM G21: Pass.

          - GreenScreen Revive 1%. GreenScreen Revive is a fire retardant PVC-free fabric constructed of 100 percent polyester yarn, with a minimum of 89 percent Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, UL GREENGUARD, GREENGUARD Gold. 1 percent open. Average Fabric Thickness: 0.019 inch (0.48 mm). Average Fabric Weight: 5.87 ounces per square yard.

          - GreenScreen Revive 5%. GreenScreen Revive is a fire retardant PVC-free fabric is constructed of 100 percent polyester yarn, with a minimum of 89 percent Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, UL GREENGUARD, GREENGUARD Gold, Green PVC Free. 5 percent open. Average Fabric Thickness: 0.017 inch (.043 mm). Average Fabric Weight: 5.01 ounces per square yard.

        - Linen Pattern

          - SheerWeave Series SW7650 by Phifer: PVC-free polyester. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. .014 inches thick, 6.2 oz/square yard.

      - Thermoplastic Olefin

        - Basketweave

          - SheerWeave Infinity2 1%: sustainable window treatment fabric eco-friendly basketweave. Core yarn and coating are PVC-free, lead-free and 100 percent recyclable. Fire classification: ASTM E-84 (Class I), NFPA 701-2004 TM#1 (small scale), NFPA 101 (Class A Rating) and CAN/ULC-S 109-03 Large, UL GREENGUARD, GREENGUARD Gold. 1 percent open. Average Fabric Thickness: .033 inch (.838 mm) Average Fabric Weight: 14.63 ounces per square yard.

          - SheerWeave Infinity2 3%: sustainable window treatment fabric eco-friendly basketweave. Core yarn and coating are PVC-free, lead-free and 100 percent recyclable. Fire classification: ASTM E-84 (Class I), NFPA 701-2004 TM#1 (small scale), NFPA 101 (Class A Rating) and CAN/ULC-S 109-03 Large, UL GREENGUARD, GREENGUARD Gold. 3 percent open. Average Fabric Thickness: .031 inch (.79 mm) Average Fabric Weight: 13.69 ounces per square yard.

          - SheerWeave Infinity2 5%: sustainable window treatment fabric eco-friendly basketweave. Core yarn and coating are PVC-free, lead-free and 100 percent recyclable. Fire classification: ASTM E-84 (Class I), NFPA 701-2004 TM#1 (small scale), NFPA 101 (Class A Rating) and CAN/ULC-S 109-03 Large, UL GREENGUARD, GREENGUARD Gold. 5 percent open. Average Fabric Thickness: .03 inch (.76 mm). Average Fabric Weight: 12.82 ounces per square yard.

    - Room Darkening Fabrics

      - Opaque

        - PVC Coated Fiberglass

          - SheerWeave Series SW7100 Blackout by Phifer: PVC-coated Fiberglass laminated with a 2-ply PVC film. Fire rating: NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 101 (Class B Rating), IBC Section 803.1.1 (Class B Rating). Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD Mold and Bacterial Standard ASTM 6329; Style 2000 face includes Microban® antimicrobial additives. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Opaque, .025 inches thick, 21.6 oz/square yard, opaque.

        - Fiberglass

          - Apagon Style III is a 3-ply opaque shade fabric consisting of vinyl, fiberglass, and a proprietary coating for maximum blackout. Thread Count: 59 threads per square inch. Fire rating: NFPA 701-2019 Test Method 1. Environmental Benefits: ASTM D3335 85a - Lead Testing. Bacterial and fungal resistance: ASTM Method G-21-15 - Resistance to Fungi. Opaque, .0131 inches thick, 12.8 oz/square yard.

          - Flocke by Mermet: Opaque, PVC-free fiberglass textile with acrylic flocked backing. Antistatic treated. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Draper shades made with this fabric are GreenSpec listed. Opaque, .020 inches thick, 15.93 oz/square yard.

          - SunBloc Series SB9000: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1, California U.S. Title 19. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. REACH and RoHS compliant - Lead Free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Washable and stain resistant. Same color both sides. Opaque, .015 inches thick, 12 oz/square yard.

          - SunBloc Series SB9100: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1, California U.S. Title 19. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. REACH and RoHS compliant - Lead Free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Washable and stain resistant. White exterior in all colors. Opaque, .013 inches thick, 12 oz/square yard.

        - PVC Coated Polyester.

          - Soltis Opaque B92 by Serge Ferrari. PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1 and 2/NFPA 701, Class A / ASTM E84. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Recyclable through TexyLoop. 0 percent open, .0312 inches thick, 19.1 oz/square yard.

        - Polyester

          - SheerWeave Series SW7000 Blackout by Phifer: PVC-free polyester with acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), BS 5867 Part 2 Type B Performance, IBC Section 803.1.1 (Class A Rating), NFPA 101 (Class A Rating), NFPA 701 TM#1 (small scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM G21, AATCC 174-1998 Part II and III. E/nvironmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content. Draper shades made with this fabric are GreenSpec listed. Opaque, .030 inches thick, 13.92 oz/square yard.

          - SheerWeave Series SW7400 by Phifer: PVC-free polyester with acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB 2-4. 162-M80, NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale). Bacteria and Fungal Resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental Certification: Certified to UL GREENGUARD and GRENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. Opaque, .029 inches thick, 12.53 oz/square yard.

          - SheerWeave Series SW7500 by Phifer: PVC-free polyester with an acrylic coating. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), IBC Section 803.1.1 (Class A Rating), CAN/CGSB 2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180 and ASTM G21. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content. Opaque, .016 inches thick, 12.14 oz/square yard.

          - SheerWeave Series SW7600 by Phifer: PVC-free polyester with an acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale). Bacterial and fungal resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental certification: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV protection: Seal of approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. Opaque, .020 inches thick, 11.8 oz/square yard.

          - Verona Twilight by Mermet. 50 percent Polyester and 50 percent acrylic with foam backing. Duraguard fabric protector and Sanitized Antimicrobial Protection. Plain weave that is 100% PVC free. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD and GREENGUARD Gold standards for low chemical emissions into indoor air during product usage. RoHS compliant - lead free. Bacterial and fungal resistance: ASTM E2180. 0% open, .024 inches thick, 11.65 oz/square yard.

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabric colors and patterns. Select fabric colors and patterns from the following, delete those not applicable.

    - Color and pattern: __________, No. _____.

    - Color and pattern: As indicated in Color Schedule on Drawings.

    - Color and pattern: As selected by Architect from manufacturer's standard range.

    - Custom Printed Graphics: As submitted to manufacturer in accordance with manufacturer's guidelines

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

** NOTE TO SPECIFIER ** Include one of the following paragraphs. Delete if not applicable.

    - Coordinate requirements for blocking and structural supports to ensure adequate means for installation of window shades.

    - Coordinate requirements for blocking, construction of shade pockets, and structural supports to ensure adequate means for installation of window shades.

** NOTE TO SPECIFIER ** Include one or more of the following paragraphs for window shades with headboxes recessed in ceilings. Delete those not applicable.

    - Coordinate installation of recessed shade pockets with construction of suspended acoustical panel ceilings specified in Section 09 51 23 - Acoustical Tile Ceilings.

    - Coordinate installation of recessed shade pockets with construction of suspended gypsum board ceilings specified in Section 09 21 16.33 - Gypsum Board Area Separation Wall Assemblies.

** NOTE TO SPECIFIER ** Include the following paragraph for motorized shades. Delete if not applicable.

    - Coordinate requirements for power supply conduit, and wiring required for window shade motors and controls.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install roller shades level, plumb, square, and true. Allow proper clearances for window operation hardware.

** NOTE TO SPECIFIER ** Draper Type D FlexShade Pockets are designed to be installed prior to or as part of ceiling system installation. Shade and operating mechanism can be site installed in pockets after construction operations that might damage shade are complete. If pockets are to be installed. Include the following paragraph as applicable and delete if not applicable.

    - Shade pockets:

      - Install shade pockets prior to installation of suspended ceiling system. Attach to supporting structure with screws through top of pocket at 24 inches (610 mm) minimum centers.

      - Install shade pockets in conjunction with installation of suspended ceiling system. Attach to supporting structure with screws through top of pocket at 24 inches (610 mm) minimum centers.

      - Install corner pieces securely and in alignment with pockets.

      - Install pocket ends securely and in alignment with pockets.

      - After interior construction is essentially complete, install shade and operating mechanism in pocket.

** NOTE TO SPECIFIER ** Include the following paragraph if fascia or separate closure for site constructed recess is being provided. Note fascia is not applicable if pockets are being provided. Delete if not applicable.

    - Install the following items to conceal roller and operating mechanism. Do not use exposed fasteners.

** NOTE TO SPECIFIER ** Select items to conceal roller and operating mechanism from the following as applicable. Delete if not applicable.

      - Fascias.

      - Closure panels.

      - Endcaps.

** NOTE TO SPECIFIER ** Include the following paragraphs for LightBloc shades. Delete if not applicable.

    - Install headbox, side channels, and sill channel with sealant specified in Section 07 90 00 - Joint Protection.

    - Position shades level, plumb, and at proper height relative to adjacent construction. Secure with fasteners recommended by manufacturer.

  - TESTING AND DEMONSTRATION

** NOTE TO SPECIFIER ** Include the following paragraph for motorized shades. Delete if not applicable.

    - Test motorized window shades to verify that controls, limit switches, interface to other building systems, and other operating components are functional. Correct deficiencies.

** NOTE TO SPECIFIER ** Include the following paragraph for LightBloc shades. Delete if not applicable.

    - Test window shades to verify that operating mechanism, fabric retainer, and other operating components are functional. Correct deficiencies.

** NOTE TO SPECIFIER ** Select one type operating mechanism from the list below. Delete those not applicable.

      - Crank.

      - Chain and clutch.

      - Motorized operating mechanism.

** NOTE TO SPECIFIER ** Include the following paragraph for LightBloc shades. Delete if not applicable.

    - During daylight hours, lower shades and turn off interior lights. Verify that there are no light leaks at perimeter or within shade assembly. Correct deficiencies.

    - Demonstrate operation of shades to Owner's designated representatives.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

  - SCHEDULES

** NOTE TO SPECIFIER ** Retain Paragraph below if required to suit project requirements. Identify products by name on the Drawings or use this paragraph to define the location of each shade type and mounting to be used using the "Type" designation required on the Drawings. The following are some examples of schedule references. Edit as required to suit project or delete and identify products on the Drawings.

    - Manually Operated Shades:

      - Shade Type SM-1 Single Roller

        - Mounting Type:

      - Shade Type SM-2 Dual Roller

        - Mounting Type:

    - Motorized Shades:

      - Shade Type MS-1 Single Roller

        - Configuration/Mounting:

      - Shade Type MS-2 Dual Roller

        - Configuration/Mounting:

END OF SECTION