SECTION 05 51 33

METAL LADDERS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2014 -    2024 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Kattsafe; Metal Ladders.
This section is based on the products of Kattsafe, which is located at:<BR />3921 Perry Blvd.<BR />Whitestown, IN 46075<BR />Toll Free Tel: 888-637-7872<BR />Tel: 317-769-7710<BR />Fax: 317-229 6451<BR />Email: request info (sales@kattsafe.com)
Web: https://www.kattsafe.com    
 [ Click Here ] for additional information.
FIXFAST USA - Providing maximum safety with minimum fuss to workers at height. Our extensive range of safe access products and fall protection systems are designed for the rooftop environment.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Fixed Ladder Systems:

      - Angled fixed ladders. (RL20 Series).

      - Standard fixed ladders. (RL30 Series).

      - Caged fixed ladders. (RL40 Series).

    - Ladder Accessories:

      - Ladder fall arrest system.

      - Ladder security door.

      - Cage security gate.

      - Self-closing gate.

    - Folding Ladder Systems:

      - Fold down ladders. (RL60 Series).

      - Fold down ladders suspension frames and ceiling ladder kits.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 05 12 13 - Architecturally-Exposed Structural Steel Framing.

    - Section 05 55 00 - Metal Stair Treads and Nosings.

    - Section 06 10 00 - Rough Carpentry.

    - Section 07 42 00 - Wall Panels.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section. OSHA 1910.23 is the standard recommended requirement.

    - Occupational Safety and Health Administration of the United States (OSHA):

      - OSHA 1910.23: Fixed Ladders.

      - OSHA 1910.29: Fall Protection systems and falling object protection.

    - American National Standards Institute (ANSI):

      - ANSI A14.3: Ladders - Fixed - Safety Requirements.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Provide plan, section, elevation and perspective view drawings as necessary to depict appropriate installation procedures including location, mounting, attachment, and penetration flashing as applicable.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: All primary products specified in this section will be supplied by a single manufacturer with a minimum of ten years experience.

    - Installer Qualifications: All products listed in this section are to be installed by a single installer with a minimum of five years demonstrated experience in installing products of the same type and scope as specified.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

      - Finish areas designated by Architect.

      - Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

      - Rebuild mock-up area as required to produce acceptable work.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation. Protect all components off the ground, away from standing water on a hard, level surface.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Warranty: At project closeout, submit an executed copy of the manufacturer's five year standard limited warranty against manufacturing defect, outlining its terms, conditions, and exclusions from coverage.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Kattsafe, which is located at:<BR />3921 Perry Blvd.<BR />Whitestown, IN 46075<BR />Toll Free Tel: 888-637-7872<BR />Tel: 317-769-7710<BR />Fax: 317-229 6451<BR />Email: request info (sales@kattsafe.com);Web: https://www.kattsafe.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Retain the next article only if Angled Fixed Ladders are required. Delete models not required.

  - RL20 SERIES MINI ACCESS LADDERS

    - Description: Modular Angled Aluminum Fixed Ladder for safe access between minimal elevation changes.

    - Model RL21: Angled Fixed Ladder with Grabrails.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL22: Angled fixed ladder with parapet platform.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL23: Angled fixed ladder with 8 ft (2438 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL25: Angled fixed ladder with 3 ft (914 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

** NOTE TO SPECIFIER ** Retain one of the following. Delete ladder mounting types not required.

    - Ladder Base Mounting and Walkway Mounting Kit:

      - Ladder Base Mount Option One: Non-penetrating 24 x 12 inches (610 x 305 mm) aluminum base plate for membrane and built-up roof systems.

      - Ladder Base Mount Option Two: Floor mount bracket set for finished floors.

      - LDWK.01: Non-penetrating weighted base kit for membrane roof systems for 8 ft (2438 mm) walkway.

      - LDWK.02: Non-penetrating clamp kit for standing seam roof systems for 8 ft (2438 mm) walkway.

      - LDWK.03: Fastening kit for metal roof systems for 8 ft (2438 mm) walkway.

      - LDWK.04: Non-penetrating weighted base kit for membrane roof systems for 3 ft (914 mm) walkway.

      - LDWK.05: Non-penetrating clamp kit for standing seam roof systems for 3 ft (914 mm) walkway.

      - LDWK.06: Fastening kit for metal roof systems for 3 ft (914 mm) walkway.

** NOTE TO SPECIFIER ** Retain the next article only if Standard Fixed Ladders are required.

  - RL30 SERIES STANDARD ACCESS LADDERS

    - Modular Vertical Aluminum Fixed Ladder for safe access to elevated areas. Ladder fall arrest system required on all heights 24ft (7320 mm) or greater.

** NOTE TO SPECIFIER ** Select Ladder Model. Choose one of the following five ladder kits.

    - Model RL31: Standard fixed ladder with grabrails.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm)

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL32: Standard fixed ladder with parapet platform.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL33: Standard fixed ladder with 8 ft (2438 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL34: Standard fixed ladder for roof hatch access.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm).

      - Retractable Stile Set: Stiles can be retracted to allow for safe access through the roof hatch.

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL35: Standard fixed ladder with 3 ft (914 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

** NOTE TO SPECIFIER ** Retain one of the following. Delete ladder mounting types not required.

    - Ladder Base Mounting and Walkway Mounting Kit:

      - Ladder Base Mount Option One: Non-penetrating 24 x 12 inches (610 x 305 mm) aluminum base plate for membrane and built-up roof systems.

      - Ladder Base Mount Option Two: Off-floor mount fixing bracket, heavy duty wall fixing bracket for ladders mounted to the wall only.

      - Ladder Base Mount Option Three: Floor mount bracket set for finished floors.

      - LDWK.01: Non-penetrating weighted base kit for membrane roof systems for 8 ft (2438 mm) walkway.

      - LDWK.02: Non-penetrating clamp kit for standing seam roof systems for 8 ft (2438 mm) walkway.

      - LDWK.03: Fastening kit for metal roof systems for 8 ft (2438 mm) walkway.

      - LDWK.04: Non-penetrating weighted base kit for membrane roof systems for 3 ft (914 mm) walkway.

      - LDWK.05: Non-penetrating clamp kit for standing seam roof systems for 3 ft (914 mm) walkway.

      - LDWK.06: Fastening kit for metal roof systems for 3 ft (914 mm) walkway.

** NOTE TO SPECIFIER ** Retain the next article only if Caged Fixed Ladders are required.

  - RL40 SERIES CAGED ACCESS LADDERS:

    - Modular Caged Aluminum Fixed Ladder for safe access to elevated areas. A Cage is considered a form of fall protection on ladders greater than 24 ft (7320 mm) until November 19th 2018.

** NOTE TO SPECIFIER ** Select Ladder Model. Choose one of the following five ladder kits.

    - Model RL41: Caged fixed ladder with grabrails.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Cage Weight: 18 lbs (8.16 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL42: Caged fixed ladder with parapet platform.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Cage Weight: 18 lbs (8.16 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL43: Caged fixed ladder with 8 ft (2438 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Cage Weight: 18 lbs (8.16 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL44: Caged fixed ladder for roof hatch access.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm).

      - Retractable stile set: Stiles can be retracted to allow for safe access through the roof hatch.

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Cage Weight: 18 lbs (8.16 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL45: Caged fixed ladder with 3 ft (914 mm) walkway.

      - Material: High tensile 6106-T6 aluminum, mill finish.

      - Ladder Height: Fall protection required over 24 ft (7320 mm). See ' Ladder Fall Arrest System' under ' Ladder Accessories.'

      - Ladder Width: 23-1/2 inches (597 mm). Walk-Through Width: 24 inches (610 mm).

      - Ladder Weight: 6.4 lbs (2.9 kg) per 40 inch (1016 mm) section.

      - Cage Weight: 18 lbs (8.16 kg) per 40 inch (1016 mm) section.

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

** NOTE TO SPECIFIER ** Retain one of the following. Delete ladder mounting types not required.

    - Ladder Base Mounting and Walkway Mounting Kit:

      - Ladder Base Mount Option One: Non-penetrating 24 x 12 inches (610 x 305 mm)aluminum base plate for membrane and built-up roof systems.

      - Ladder Base Mount Option Two: Off-floor mount fixing bracket, heavy duty wall fixing bracket for ladders mounted to the wall only.

      - Ladder Base Mount Option Three: Floor mount bracket set for finished floors.

      - LDWK.01: Non-penetrating weighted base kit for membrane roof systems for 8 ft (2438 mm) walkway.

      - LDWK.02: Non-penetrating clamp kit for standing seam roof systems for 8 ft (2438 mm) walkway.

      - LDWK.03: Fastening kit for metal roof systems for 8 ft (2438 mm) walkway.

      - LDWK.04: Non-penetrating weighted base kit for membrane roof systems for 3 ft (914 mm) walkway.

      - LDWK.05: Non-penetrating clamp kit for standing seam roof systems for 3 ft (914 mm) walkway.

      - LDWK.06: Fastening kit for metal roof systems for 3 ft (914 mm) walkway.

** NOTE TO SPECIFIER ** Select Ladder Accessories if required.

  - FIXED LADDER ACCESSORIES

    - Ladder Fall Arrest System:

** NOTE TO SPECIFIER ** Delete model options not required.

      - Model RL50SYS: Fall arrest kit includes fall arrest mounting brackets, tensioner, termination device, sign and stainless hardware.

      - Model SL228.10: Arresta Shuttle locking device with energy absorbing lanyard (1 required per user).

      - Model SL230F: Stainless steel (316) cable Specify wall height.

      - Capacity: For single person use; 1400 lbs (635 kg) rated.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.29 and ANSI A14.3.

    - Model LD426: Ladder Security Door. Aluminum security door to prevent unauthorized access. 84 x 25-1/2 inches (2134 x 648 mm).

    - Model LD425: Ladder Cage Gate. Aluminum gate to prevent unauthorized access. 37 x 30-5/8 inches (940 x 778 mm).

    - Model GW390: Self-Closing Gate. Aluminum gate for mounting at the end of the walkway kits or parapet platforms.

** NOTE TO SPECIFIER ** Retain the next article only if Fold Down Access Ladders are required.

  - RL60 SERIES FOLD DOWN ACCESS LADDERS AND SUSPENSION KITS

** NOTE TO SPECIFIER ** Select Ladder Model. Choose one of the following ladder kits

    - Model RL62: Vista Commercial Fold Down Ladder.

      - Material: High tensile aluminum, mill finish.

      - Opening Size: 48 x 31 inches (1219 x 787 mm).

      - Ladder Height Range: 7 to 10 ft (2134 to 3048 mm).

      - Ladder Width: 23-1/2 inches (597 mm).

      - Unit Weight: 88 lbs (40 kg).

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

    - Model RL63: Vista Maxi Fold Down Ladder.

      - Material: High tensile aluminum, mill finish.

      - Opening Size: 64 x 31 inches (1626 x 787 mm).

      - Ladder Height Range: 7 to 14 ft (2134 to 4267 mm).

      - Unit Weight: 88 lbs (40 kg).

      - Capacity: Unit shall support a 1000 lbs (453.6 kg) loading without failure, and individual treads shall withstand a 1000 lbs (453.6 kg) loading without failure.

      - Performance Standard: Units designed and manufactured to meet or exceed OSHA 1910.23.

** NOTE TO SPECIFIER ** Select Suspension frame and ceiling ladder kit. Choose one of the following kits and specify height

    - Ladder Suspension Frame and Ceiling Ladder Kit:

      - Model RL62: Height (in/mm): ______ between roof structure and suspended ceiling. Suspension kit can be cut to required height in the field.

      - Model RL63: Height (in/mm): ______ between roof structure and suspended ceiling. Suspension kit can be cut to required height in the field.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION