SECTION 08 53 13

VINYL WINDOWS AND PATIO DOORS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2022 -    2022 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** North Star Windows And Doors; vinyl windows and patio doors.
This section is based on the products of North Star Windows And Doors, which is located at:<BR />40684 Talbot Line<BR />St. Thomas, ON, Canada N5P 3T2<BR />Toll Free Tel: 800-265-5701<BR />Email: request info (null)
Web: https://www.northstarwindows.com    
 [ Click Here ] for additional information.
Quality Windows & Doors. Great Service. Exceptional Value.
Since 1985 North Star has been an industry leader in the manufacture of attractive and energy-efficient vinyl windows and patio doors. Whether you're building a new home or upgrading to today's standards, North Star has an extensive range of quality, energy-efficient and full-featured windows that are not only beautiful but offer exceptional value.
Take a tour of North Star and see how your Windows & Doors are made.
We are committed to best-in-class manufacturing technology and ensure all our products consistently exceed industry standards.
This is supported by a network of dealers with a reputation for providing quality workmanship and attentive customer service. For 36 years, North Star has been manufacturing quality windows and doors, delivering great service, and providing exceptional value.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Windows.

      - 1071 Picture window, no sash.

      - 1072 Picture window, sash.

      - Architectural shaped windows.

      - 1171 Casement windows.

      - 1173 Awning windows.

      - 1271 Double hung tilt windows with full screen.

      - 1272 Double slider tilt windows with full screen.

      - 1273 Double slider liftout windows with full screen.

      - 1371 Single hung tilt windows with full screen.

      - 1372 Single slider liftout windows, 1/2 screen. Left side operating, right side fixed.

      - 1373 Single slider liftout windows, 1/2 screen. Right side operating, left side fixed.

      - 1374 3-Lite (end vent) single slider liftout windows with full screen.

      - 1377 Picture window (singles frame).

      - Bay and Bow windows.

    - Sliding Patio Doors:

      - Single patio door sidelites.

      - Two-panel patio doors.

      - Three-panel doors (centre operable)

      - Four-panel doors (two centre panels operable)

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 05 40 00 - Cold-Formed Metal Framing0 - Cold-Formed Metal Framing.

    - Section 06 10 00 - Rough Carpentry0 - Rough Carpentry.

    - Section 07 91 23 - Backer Rods0 - Joint Sealants.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - National Standard of Canada (CAN/CSA):

      - CAN/CSA A440-00 - Windows/User Selection Guide to CSA Standard A440-00, Windows

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00 - Administrative Requirements.

    - Product Data:

      - Manufacturer's data sheets on each product to be used.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Typical installation methods.

** NOTE TO SPECIFIER ** Delete if not applicable to product type.

    - Verification Samples: Two representative units of each type, size, pattern, and color.

    - Shop Drawings: Include details of materials, construction, and finish. Include relationship with adjacent construction.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum five years documented experience.

    - Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

    - Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up on might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

      - Intent of mock-up is to demonstrate quality of workmanship and visual appearance.

      - If mock-up is not acceptable, rebuild mock-up until satisfactory results are achieved.

      - Retain mock-up during construction as a standard for comparison with completed work.

      - Do not alter or remove mock-up until work is completed or removal is authorized.

  - PRE-INSTALLATION CONFERENCE

    - Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

  - DELIVERY, STORAGE, AND HANDLING

    - Store and handle in strict compliance with manufacturer's written instructions and recommendations.

    - Protect from damage due to weather, excessive temperature, and construction operations.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - WARRANTY

    - Manufacturer's standard limited warranty unless indicated otherwise.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: North Star Windows And Doors, which is located at:<BR />40684 Talbot Line<BR />St. Thomas, ON, Canada N5P 3T2<BR />Toll Free Tel: 800-265-5701<BR />Email: request info (null);Web: https://www.northstarwindows.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Delete article in not required or delete basis of design paragraphs not required.

  - PICTURE WINDOWS

    - Basis of Design: 1071 Picture Window, No Sash, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

      - Air: (A) Fixed Rating.

        - Water: B7.

        - Structural: C5.

        - Forced Entry: N/A.

        - DP Rating: CW-PG70-FW.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: Fixed.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed: Minimum: 10 x 10 inches (254 x 254 mm). Maximum one way: 128 inches (3251 mm); 30 sq ft (2.787 sq m).

        - Triple Glazed: Maximum one way: 128 inches (3251 mm); 20 sq ft (1.858 sq m).

      - Frames: Combined frame and stop profiles extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Weather-Stripping: Co-extruded flexible vinyl fin. Seals against exterior and interior glass faces along profiled exterior frame edge and profiled interior glass stop.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak

          - Interior Wood Grains: Light Oak

          - Interior Wood Grains: Walnut

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso,

          - Interior Textured Color Finishes: Hickory

          - Interior Textured Color Finishes: Midnight Black.

          - Interior Color Finishes: Espresso

          - Interior Color Finishes: Midnight Black

          - Interior Color Finishes: Hickory

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1072 Picture Window with Sash, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: (A) Fixed Rating.

        - Water: B7.

        - Structural: C5.

        - Forced Entry: N/A.

        - DP Rating: CW-PG70-FW.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: Fixed.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed: Minimum: 12 x 12 inches (305 x 305 mm). Maximum one way: 128 inches (3251 mm); 40 sq ft (3.716 sq m).

        - Triple Glazed: Maximum one way: 128 inches (3251 mm); 30 sq ft (2.787 sq m).

      - Frames: Frame and sash profiles extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Weather-Stripping: Co-extruded flexible vinyl fin. Seals against exterior and interior glass faces along profiled exterior frame edge and profiled interior glass stop.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1071 Architectural Shaped Windows, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: (A) Fixed Rating.

        - Water: B7.

        - Structural: C5.

        - Forced Entry: N/A.

        - DP Rating: CW-PG70-FW.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: Fixed.

      - Frames: Combined frame and stop profiles extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes option not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Weather-Stripping: Co-extruded flexible vinyl fin. Seals against exterior and interior glass faces along profiled exterior frame edge and profiled interior glass stop.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak

          - Interior Wood Grains: Light Oak

          - Interior Wood Grains: Walnut

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso,

          - Interior Textured Color Finishes: Hickory

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

** NOTE TO SPECIFIER ** Delete geometric shapes option not required and shape options not required.

      - Non-Bent Geometric Shapes:

        - Triangle (WxH): ___ x ___ inches (___ x ___ mm).

        - Hexagon (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Trapezoid (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Octagon (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Pentagon (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

      - Bent Geometric Shapes:

        - Half Arch (WxH): ___ x ___ inches (___ x ___ mm).

        - Extended Quarter Arch (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Quarter Arch (WxH): ___ x ___ inches (___ x ___ mm).

        - Full Circle (Radius): ___ inches (___ mm).

        - Extended Arch (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Cord/Eyebrow (WxH): ___ x ___ inches (___ x ___ mm).

        - Cathedral (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Ellipse Top (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - True Ellipse (WxH): ___ x ___ inches (___ x ___ mm).

    - Basis of Design: 1171 Casement Windows, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A3.

        - Water: B7.

        - Structural: C4.

        - Screen: S2.

        - Forced Entry: F20.

        - DP Rating: R-PG65-C.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A3.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 14-3/4 x 17 inches (375 x 432 mm).

          - Maximum Width and Height: 36 x 72 inches (914 and 1828 mm) or 30 x 78 (762mm x 1981mm)

          - Maximum Width + Height: 108 united inches (2743 mm).

        - Triple Glazed:

          - Minimum: 14-3/4 x 17 inches (375mm x 432 mm).

          - Maximum Width + Height: 98 united inches (2489 mm)

          - Maximum Width 36 inches( 914mm). Maximum Height 78 inches (1981mm)

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Triple Weather-Stripping: Two flexible dual durometer bulb type compression gaskets located around perimeter of window frame ensuring air and watertight seal and energy efficiency. One water repellant pile weather-strip with stiff fin type vapor barrier located around perimeter of sash increasing energy efficiency and reducing dust and dirt from accumulating around sash cavity.

      - Hardware: Truth Maxim Hardware. Hinges: Concealed scissor.

** NOTE TO SPECIFIER ** Delete operation option not required.

        - Operation: Truth Maxim Heavy-Duty Roto Gear: Opens sash to 90 degrees and hold sash at any position. White with contour handle.

        - Operation: Truth Encore Heavy-Duty Roto Gear: Folding handle with roto gear nesting cover. Finish: White, back, brushed antique chrome, satin nickel, and oil rubbed bronze finish.

** NOTE TO SPECIFIER ** Limit sash opening hardware, safeguard casement window, or opening control devices are available. Delete sash lock not required.

        - Sash Lock: Pulls weather-stripped sash against dual compression bulb weather gasket on frame. White with contour handle.

        - Concealed Multipoint Sash Locks: For units more than 22 inches (559 mm) in height.

        - Corrosion Protection: Truth E-Gard coating for steel components.

        - Snubber Hardware on Frame and Sash: Ensures compression of bulb weather-strip opposite to the lock side.

** NOTE TO SPECIFIER ** Special egress hardware is available for casement windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Standard Hardware:

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77 sq/ft (0.35 m2)

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0 sq/ft (0.47 m2)

          - Canadian Min. Frame (WxH): 24-13/16 x 21 inches (630 x 533 mm).

          - USA Min. Frame (WxH): 30 x 29 inches (630 x 533 mm).

        - Egress Hardware:

          - Canadian Min. Frame (WxH): 22-1/4 x 19-3/4 inches (565 x 502 mm).

          - USA Min. Frame (WxH): 27-1/2 x 29 inches (699 x 737 mm).

      - Screens: Heavy Duty EZ-Screens.

        - Frame: Roll formed 0.025 inch aluminum. Mitered corners and spring loaded corner keys for removal and installation.

        - Screen: Antiglare fiberglass cloth.

        - Frame Finish: Painted white or black.

        - Frame Finish: Laminated. Color: As determined by the Architect from Manufacturer's standard range.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight black.

          - Exterior Textured Color Finish: Anthracite grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color determined by the Architect from manufacturers standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1173 Awning Windows, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A3.

        - Water: B7.

        - Structural: C4.

        - Screen: S2.

        - Forced Entry: F20.

        - DP Rating: R-PG60-AP.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A3.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 21 x 14-3/4 inches (533 x 375 mm).

          - Maximum Width and Height: 60 x 48 inches (1524 mm x 1219 mm) or 48 x 60 inches (1219 x 1524)

          - Maximum Width + Height: 108 united inches (2743 mm).

        - Triple Glazed:

          - Minimum: 21 x 14-3/4 inches (533 x 375 mm).

          - Maximum Width + Height: 96 united inches (2438 mm).

          - Maximum Width: 60 inches (1524 mm) Maximum Height: 60 inches (1524 mm)

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Triple Weather-Stripping: Two flexible dual durometer bulb type compression gaskets located around perimeter of window frame ensuring air and watertight seal and energy efficiency. One water repellant pile weather-strip with stiff fin type vapor barrier located around perimeter of sash increasing energy efficiency and reducing dust and dirt from accumulating around sash cavity.

      - Hardware: Truth Maxim Hardware. Hinges: Concealed scissor.

** NOTE TO SPECIFIER ** Delete operation option not required.

        - Operation: Truth Maxim Heavy-Duty Roto Gear: Opens sash to 90 degrees and hold sash at any position. White with contour handle.

        - Operation: Truth Encore Heavy-Duty Roto Gear: Folding handle with roto gear nesting cover. Finish: White, back, brushed antique chrome, satin nickel, and oil rubbed bronze finish.

** NOTE TO SPECIFIER ** Limit sash opening hardware, safeguard casement window, or opening control devices are available.

        - Sash Lock: Pulls weather-stripped sash against dual compression bulb weather gasket on frame. White with contour handle.

          - Dual sash locks located on window jamb.

        - Corrosion Protection: Truth E-Gard coating for steel components.

        - Snubber Hardware: On windows 42 inch (1067 mm) wide or larger applied to frame and sash to ensure compression of the bulb weather-strip opposite to sill hardware.

      - Screens: Heavy Duty EZ-Screens.

        - Frame: Roll formed 0.025 inch aluminum. Mitered corners and spring loaded corner keys for removal and installation.

        - Screen: Antiglare fiberglass cloth.

        - Frame Finish: Painted white or black.

        - Frame Finish: Laminated. Color: As determined by the Architect from Manufacturer's standard range.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1271 Double Hung Tilt Window with full screen, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2.

        - Water: B6.

        - Structural: C5.

        - Screen: S1.

        - Forced Entry: F10.

        - DP Rating: R-PG40-H.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A2.

      - Window Sizes:

        - Double Glazed:

          - Minimum: 12 x 22 inches (305 x 559 mm).

          - Maximum Width: 48 inches (1219 mm) Maximum Height: 78 inches (1981).

          - Maximum Width + Height: 120 united inches (3048 mm).

        - Triple Glazed:

          - Minimum: 12 x 22 inches (305 x 559 mm).

          - Maximum Width: 42 inches (1067 mm). Maximum Height: 78 inches (1981 mm).

          - Maximum Width + Height: 104 united inches (2642 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: Stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Pile Weather-Strip: Stiff fin type vapor barrier on inside of window frame at sill and jamb for additional protection.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Width Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - Two constant force coil spring balance hardware.

          - Recessed Finger Latches: Release top and bottom sash to tilt sash.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile.

            - Color: White.

            - Color: _______.

          - Self-latching locks, keepers, and tilt latches.

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for double hung tilt windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35m2)

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0sq/ft (0.47m2)

        - Canada: Min. Frame (WxH): 20-1/8 x 43-1/2 inches (511 x 1105 mm).

        - United States: Min. Frame (WxH): 25-1/4 x 61-1/2 inches (641 x 1562 mm).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak

          - Interior Wood Grains: Walnut

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard color range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by manufacturers standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1272 Double Slider Tilt Window with full screen, as manufactured by North Star. Both sash and slide tilt.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2.

        - Water: B2.

        - Structural: C5.

        - Screen: S1.

        - Forced Entry: F10.

        - DP Rating: R-PG30-HS.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A2.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 22 x 12 inches (559 x 305 mm).

          - Maximum Width 78 inches (1981 mm): Maximum Height 54 inches (1371)

          - Maximum Width + Height: 126 united inches (3200 mm).

        - Triple Glazed:

          - Minimum: 22 x 12 inches (559 x 305 mm).

          - Maximum Width 78 inches (1981 mm): Maximum Height 54 inches (1371).

          - Maximum Width + Height: 106 united inches (2692 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: Stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Pile Weather-Strip: Stiff fin type vapor barrier on inside of window frame at sill and jamb for additional protection.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Height Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - "Delrin" self-lubricating glide shoes on bottom of sash to allow for smooth opening and closing of sash.

          - Recessed Finger Latches: secured at top of sash to allow tilt in and lift out of sash for ease of cleaning.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile.

            - Color:    White.

            - Color:    _______.

          - Self-latching locks, keepers, and tilt latches.

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for double slider tilt windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35m2)

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0sq/ft (0.47m2)

        - Canada: Minimum Frame (WxH): 43 x 20-1/4 inches (1092 x 515 mm).

        - United States: Minimum Frame (WxH): 53 x 59-1/2 inches (1346 x 1511 mm).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso,

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range of colors.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1273 Double Slider Liftout Window with full screen, as manufactured by North Star. Both sash slide.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2.

        - Water: B3.

        - Structural: C3.

        - Screen: S1.

        - Forced Entry: F20.

        - DP Rating: R-PG45-HS.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A2.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 22 x 13 inches (559 x 330 mm).

          - Maximum Width 78 inches (1981 mm) Maximum Height: 60 inches (1524 mm).

          - Maximum Width + Height: 132 united inches (3353 mm).

        - Triple Glazed:

          - Minimum: 22 x 13 inches (559 x 330 mm).

          - Maximum Width 78 inches (1981 mm) Maximum Height: 60 inches (1524 mm).

          - Maximum Width + Height: 132 united inches (2896 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: Stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Pile Weather-Strip: Stiff fin type vapor barrier on inside of window frame at sill and jamb for additional protection.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Height Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - "Delrin" self-lubricating glide shoes on bottom of sash to allow for smooth opening and closing of sash.

          - Recessed Finger Latches: Secured at top of sash to allow tilt in and lift out of sash for ease of cleaning.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile

            - Color:    White.

            - Color:    _______.

          - Self-latching locks, keepers, and tilt latches.

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for single slider lift windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35m2).

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0sq/ft (0.47m2).

        - Canada: Minimum Frame (WxH): 43 x 20-1/4 inches (1092 x 515 mm).

        - United States: Minimum Frame (WxH): 53 x 59-1/2 inches (1346 x 1511 mm).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clip.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1371 Single Hung Tilt Window, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2/Fixed.

        - Water: B4.

        - Structural: C3.

        - Screen: S1.

        - Forced Entry: F20.

        - DP Rating: R-PG40-H.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A2.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 13 x 21 inches (330 x 533 mm).

          - Maximum Width 48 inches (1219 mm) Maximum Height: 78 inches (1981 mm).

          - Maximum Width + Height: 120 united inches (3048 mm).

        - Triple Glazed:

          - Minimum: 13 x 21 inches (330 x 533 mm).

          - Maximum Width 42 inches (1067 mm) Maximum Height: 78 inches (1981 mm).

          - Maximum Width + Height: 104 united inches (2642 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: Seven stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Pile Weather-Strips: Incorporated for added weatherability.

        - One Weather-Strip: Located on inside of window frame at sill and on the fixed upper sash where lower and upper sash overlap.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Width Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - Two constant force coil spring balance hardware.

          - Recessed Finger Latches: Release bottom sash to tilt sash.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile.

          - Color: White.

          - Color: _______.

          - Self-latching locks, keepers, and tilt latches

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for single hung tilt windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35m2).

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0sq/ft (0.47m2).

        - Canada: Min. Frame (WxH): 19-3/4 x 45 inches (502 x 1143 mm).

        - United States: Min. Frame (WxH): 26-3/4 x 58-3/4 inches (679 x 1492 mm).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

      - Shaped Top Single Hung:

        - Round Top (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Camber Top (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Cathedral (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Quarter Arch (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Pentagon (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

        - Trapezoid (WxHxLEG): ___ x ___ x ___ inches (___ x ___ x ___ mm).

    - Basis of Design: Single Slider Liftout Window with 1/2 screen, as manufactured by North Star.

** NOTE TO SPECIFIER ** Delete model not required.

      - Model: 1372. Left side operating and right side fixed.

      - Model: 1373. Right side operating and left side fixed.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2/Fixed.

        - Water: B4.

        - Structural: C3.

        - Screen: S1.

        - Forced Entry: F20.

        - DP Rating: R-PG40-H.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: A3.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 22 x 13 inches (559 x 330 mm).

          - Maximum Width 78 inches (1981 mm) Maximum Height: 60 inches (1524 mm).

          - Maximum Width + Height: 132 united inches (3353 mm).

        - Triple Glazed:

          - Minimum: 22 x 13 inches (559 x 330 mm).

          - Maximum Width 78 inches (1981) Maximum Height: 54 inches (1372 mm).

          - Maximum Width + Height: 114 united inches (2896 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: Eight stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Interior Side Pile Weather-Strips: Water repellant with stiff fin type vapor barrier are located on inside of window frame at sill and jamb location for additional protection against air and water penetration.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Width Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - "Delrin" self-lubricating glide shoes on bottom of sash to allow for smooth opening and closing of sash.

          - Recessed Finger Latches: Secured at top of sash to allow tilt in and lift out of sash for ease of cleaning.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile.

          - Color: White.

          - Color: _______.

          - Self-latching locks, keepers, and tilt latches.

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for single slider lift windows that enable narrow sizes to meet egress codes.

      - Egress: Consult local building codes. Verify window specified meets or exceeds requirements for Project location.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35m2).

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53m2). Grade floor 5.0sq/ft (0.47m2).

        - Canada: Minimum Frame (WxH): 40 x 19-3/4 inches (1016 x 502 mm).

        - United States: Minimum Frame (WxH): 50 x 29 inches (1270 x 737 mm).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1374 3-Lite (End Vent) Single Slider Lift-out Windows with full screen, as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: A2/Fixed.

        - Water: B3.

        - Structural: C3.

        - Screen: S1.

        - Forced Entry: F20.

        - DP Rating: R-PG55-HS.

        - Water Resistance: 330 psf (15.8 kPa).

        - Air Infiltration/Exfiltration: A3.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed:

          - Minimum: 48 x 13 inches (1219 x 330 mm).

          - Maximum Width: 120 inches (3048 mm) Maximum Height: 60 inches (1524 mm).

          - Maximum Width + Height: 168 united inches (4267 mm).

        - Triple Glazed:

          - Minimum: 48 x 13 inches (1219 x 330 mm).

          - Maximum Width: 108 inches (2743mm) Maximum Height: 54 inches (1372 mm).

          - Maximum Width + Height: 144 united inches (3658 mm).

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

        - Recessed Drain Covers: For frames. Reticulated foam to keep insects out.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Multiple Weather-Stripping:

        - Water Repellant Pile Weather-Strips: With stiff fin type vapor barrier around perimeter of each operable window sash ensuring air and watertight seal.

        - Two Pile Weather-Strips: With stiff fin type vapor barrier located on inside of window frame at sill and jamb for protection against air and water penetration.

      - Hardware: Two Die-Cast Cam Locks and Keeper: Assembles onto each operable sash for weatherability and security.

        - Window Width Less Than 27-1/4 inch (692 mm): One cam lock and keeper.

** NOTE TO SPECIFIER ** Limit sash opening features are available. Consult manufacturer for more information. Delete sash operation options not required.

        - Sash Operation:

          - "Delrin" self-lubricating glide shoes on bottom of sash to allow for smooth opening and closing of sash.

          - Recessed Finger Latches: Secured at top of sash to allow tilt in and lift out of sash for ease of cleaning.

          - Full Length Sash Pull Handles: Extruded into interior glazing stop profile.

          - Color: White.

          - Color: _______.

          - Self-latching locks, keepers, and tilt latches.

            - Color:    White.

            - Color:    Black.

** NOTE TO SPECIFIER ** Special egress hardware is not available for 3-lite end vent windows that enable narrow sizes to meet egress codes.

      - Egress: Verify windows meet local building code.

        - Canadian egress is minimum clear opening of 15 inches (381 mm) and a minimum sq/ft opening of 3.77sq/ft (0.35 m2).

        - USA egress is minimum clear opening (WxH) 20x24 and a minimum sq/ft opening of 5.7 sq/ft (0.53 m2). Grade floor 5.0 sq/ft (0.47 m2).

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clip.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: 1377 Picture Window (Singles Frame), as manufactured by North Star.

      - Standards Compliance: CAN/CSA A440-00.

        - Air: N/A.

        - Water: N/A.`

        - Structural: N/A.

        - Forced Entry: N/A.

        - DP Rating: CW-PG70-FW.

        - Water Resistance: 730 psf (35 kPa).

        - Air Infiltration/Exfiltration: Fixed.

      - Window Type and Sizes:

** NOTE TO SPECIFIER ** Delete window type not required.

        - Double Glazed: Minimum: 13 x 13 inches (330 x 330 mm). Maximum one way: 128 united inches (3251 mm); 40 sq ft (3.716 sq m).

        - Triple Glazed: Maximum one way: 128 inches (3251 mm); 20 sq ft (1.858 m).

      - Frame: Frame and stop profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Insulated Glass: Panes.

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Weather-Stripping: Profiled exterior frame edge and interior glass stop have co-extruded flexible vinyl fin weather-strip. Seals against exterior and interior glass faces.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

    - Basis of Design: Bay and Bow Windows, as manufactured by North Star.

      - Features:

        - Completely assembled with side jambs, including interior side capping to minimize trimming the cavity most common on bay and bow applications.

        - Exterior extruded vinyl capping on head and seat.

        - Head and seat with jambs include edge banding, which allows the option of a reveal when installing the casing trim.

        - Stretch-wrapped packaging on a shipping skid to avoid damage to the unit.

        - Air spaces behind window couplers are insulated with fiberglass batt insulation.

** NOTE TO SPECIFIER ** Bay or bow windows must be supported to avoid sagging. Failing to do so will void all warranty.

      - Cable Support System; Grip-Tite: Uses a straightline clamp to secure the cable to the window. The design has a "no-slip" function that locks the cable in place.

      - Frame and Sash: Profiles are extruded virgin PVC powder material.

        - Design: Multi-chamber for strength and energy efficiency.

        - Joints: Fusion welded for a water and airtight seal, strength, and squareness.

        - Frame Depth: 3-1/4 inch (83 mm).

      - Head and Seat:

** NOTE TO SPECIFIER ** Delete construction options not required.

        - Construction: Maintenance-free 3/4 inch (19 mm) white Formica.

        - Construction: Medium oak veneer (unfinished) head and seat

        - Construction: Laminated 2-1/8 inch (54 mm) insulated head and seat with exterior vinyl extruded capping system.

        - Front Edge of Head and Seat: Sealed against moisture with aluminum foil backed ice and water shield before extruded vinyl capping is applied and sealed.

      - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

        - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

        - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

        - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

        - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

        - Glass: Cardinal LoC3-366, soft coat.

        - Glass: Pilkington Activ Self-Cleaning.

        - Glass: Laminated. 3/8 inch (9.5 mm) airspace. Two 1/8 inch (3 mm) glass panes with a 0.030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

        - Air Space: Heavy inert argon or krypton gas.

      - Glazing:

        - Neoprene Setting Blocks: Around perimeter of sash.

        - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

        - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

        - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

      - Weather-Stripping: Combination of flexible vinyl bulb type and water repellant pile weather-strips with stiff fin type vapor barrier. Ensures maximum energy efficiency.

      - Hardware: Dependent on window types used in Bay and Bow configurations.

      - Screens:

        - Frame: Roll formed 0.021 inch (0.53 mm) aluminum. Nylon injection molded corner key with pull tab.

        - Screen Material: Anti-glare fiberglass cloth.

** NOTE TO SPECIFIER ** Screen frame is white or one of manufacturer's nine exterior colors

        - Color: As determined by the Architect or detailed on the Drawings.

      - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

        - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

        - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

          - Exterior Textured Color Finish: Ivory.

          - Exterior Textured Color Finish: Chestnut brown.

          - Exterior Textured Color Finish: Cocoa.

          - Exterior Textured Color Finish: Hickory.

          - Exterior Textured Color Finish: Sandalwood.

          - Exterior Textured Color Finish: Sable.

          - Exterior Textured Color Finish: Midnight Black.

          - Exterior Textured Color Finish: Anthracite Grey.

          - Exterior Textured Color Finish: Espresso.

          - Interior Wood Grains: Kolonial Oak.

          - Interior Wood Grains: Light Oak.

          - Interior Wood Grains: Walnut.

          - Interior Wood Grains: Stainable/Paintable Pine.

          - Interior Textured Color Finishes: Espresso.

          - Interior Textured Color Finishes: Hickory.

          - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

      - Grilles: Between glass panes.

        - Grille Width: 5/16 inch (8 mm) narrow. Finish: White, pewter and brass.

        - Grille Width: 5/8 inch (16 mm) flat rectangular.

          - Finish Interior: White or wood grain.

          - Finish Exterior: Color as determined by the Architect from manufacturer's standard range or as detailed on the Drawings.

        - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

          - Finish: Pewter all sides.

          - Finish: White all sides.

          - Finish: Black all sides.

        - Contoured 3/4 inch (19 mm) wide grilles.

          - Finish Interior: White.

          - Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

      - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

        - Grille Width: 7/8 inches (22 mm).

        - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

        - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

      - Accessories:

        - Nailing fin.

        - Installation Clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

        - Brick Molds: For bay and bow windows without head and seat.

        - Drywall/wood return.

        - Couplers.

        - Jamb extensions.

      - Standard Jamb: 6 inch (152 mm).

** NOTE TO SPECIFIER ** Delete bay window types and options not required.

      - Bay Window Type: 30 Degree Bay Window.

        - Bay Width: 48 to 144 inches (1219 to 3657 mm)

      - Bay Window Type: 45 Degree Bay Window.

        - Bay Width: 60 to 144 inches (1524 to 3657 mm).

      - Bay Window Type: 4 Lite Bow Window.

        - Bay Width: 60 to 144 inches (1524 to 3657 mm).

      - Bay Window Type: 5 Lite Bow Window.

        - Bay Width: 60 to 144 inches (1524 to 3657 mm).

** NOTE TO SPECIFIER ** Delete article if not required.

  - PATIO DOORS

    - Patio Doors:

** NOTE TO SPECIFIER ** Delete basis of design options not required.

      - Basis of Design: Model 3310. Patio Door, as manufactured by North Star.

        - Standards Compliance:    CAN/CSA A440-00.

          - Air:    A3.

          - Water:    B3.

          - Structural:    C3.

          - Forced Entry:    F20.

          - DP Rating:    CW-PG45-FW.

          - Water Resistance:    330 psf (15.8 kPa).

          - Air Infiltration/Exfiltration:    A3.

      - Basis of Design: Model 3320. Two-Panel Patio Doors, as manufactured by North Star.

        - Standards Compliance: CAN/CSA A440-00.

          - Air: A3.

          - Water: B3.

          - Structural: C3.

          - Forced Entry: F20.

          - DP Rating: CW-PG45-FW.

          - Water Resistance: 330 psf (15.8 kPa).

          - Air Infiltration/Exfiltration: A3.

      - Basis of Design: Model 3330. Three-Panel Patio Door, as manufactured by North Star.

        - Standards Compliance: CAN/CSA A440-00.

          - Air: A3.

          - Water: B3.

          - Structural: C2.

          - Forced Entry: F10.

          - DP Rating: CW-PG30-FW.

          - Water Resistance: 220 psf (10.5 Mpa).

          - Air Infiltration/Exfiltration: A3.

      - Basis of Design: Model 3340. Four-Panel Patio Door, as manufactured by North Star.

        - Standards Compliance: CAN/CSA A440-00.

          - Air: A2.

          - Water: B2.

          - Structural: C2.

          - Forced Entry: F10.

          - DP Rating: CW-PG30-FW.

          - Water Resistance: 260 psf (12.4 kPa).

          - Air Infiltration/Exfiltration: A2.

      - Basis of Design: Model 3300. Vinyl Patio Door with Reinforced Transom Assembly.

    - Frames and Sashes:

      - Profiles: Extruded virgin PVC powder material.

        - Design: Multi-chamber design for strength and energy efficiency.

        - Frame Depth: 5-5/8 inches (143 mm).

        - Corners: Mechanically fastened at corners and sealed with one-sided adhesive backed closed cell polyethylene gaskets.

        - Door Frame's Low Profile Sill: Fitted with an extruded anodized aluminum screen track.

        - Extruded Sash: Fusion welded ensuring a water and airtight seal. Metal reinforced providing superior rigidity.

    - Insulated Glass:

** NOTE TO SPECIFIER ** Delete panes options not required.

      - Number of Panes: Double. 1/8 inch (3 mm) tempered glass.

      - Number of Panes: Triple. 1/8 inch (3 mm) tempered glass.

      - Spacers: Edgetech silicone foam low conductive S-Class Super Spacer.

** NOTE TO SPECIFIER ** Delete glass options not required. Pilkington Energy Advantage is standard.

      - Glass: Pilkington Energy Advantage Low E (hard coat) glass.

      - Glass: Cardinal LoC3-366, soft coat.

      - Glass: Pilkington Activ Self-Cleaning.

      - Glass: Laminated. 1/8 inch (3 mm) thick, 3/8 inch (9.5 mm) airspace and two 1/8 inch (3 mm) glass panes with a .030 inch (0.76 mm) clear interior membrane in-between.

** NOTE TO SPECIFIER ** The remaining options are optional Delete options not required.

      - Air Space: Heavy inert argon or krypton gas.

      - White Mini Blinds: Raise, lower, and tilt.

    - Glazing:

      - Neoprene Setting Blocks: Around perimeter of sash.

      - Silicone Toe Bead Sealant: Interior glazing cavity to stabilize glass movement.

      - Insulated glass from interior side of sash against flexible vinyl fin weather-strip co-extruded to exterior edge of sash.

      - Interior glazing stops with flexible co-extruded vinyl fin weather-stripping against interior glass face.

    - Weather-Stripping:

      - Lock Side Jamb: Triple weather-strip comprised of two water repellant pile weather-strips with stiff fin type vapor barrier and one bulb type compression gasket.

      - Door Frame at Head and Bottom of Sliding Sash and at Interlocking Vertical Checkrail: Two water repellant pile weather-strips with stiff fin type vapor barrier.

      - At Frame Head: Anti-lift block with wool pile weather-strip.

      - Bottom of Lock Side Jamb: A closed cell foam pad is located at.

    - Hardware:

** NOTE TO SPECIFIER ** Delete hardware options not required.

      - Opening Panel: Equipped with two tandem roller assemblies with noise and friction reducing nylon rollers with stainless steel ball bearings.

      - Single point keyless lock and keeper:

        - Finish: White.

        - Finish: Black.

        - Finish: Deluxe Pewter.

        - Finish: Deluxe Brass.

      - Multi-point lock hardware.

      - Single point locks with key cylinder.

      - Multi-point locks with key cylinder.

      - Security bolt or security bar hardware.

** NOTE TO SPECIFIER ** Delete screens if not required.

    - Screens: Heavy duty extruded aluminum screen frame with adjustable steel rollers, screened with anti-glare fiberglass cloth.

      - Screen Frame Colors: White and manufacturer's nine exterior colors.

    - Exterior and Interior Finishes:

** NOTE TO SPECIFIER ** Delete finish options not required.

      - Virgin Vinyl Extrusion Finish: Smooth, clean, bright white.

      - Laminate Finishes: Applied PVC laminate film, 200im (microns) thick, with an acrylic overlay for exceptional UV protection.

        - Exterior Textured Color Finish: Ivory.

        - Exterior Textured Color Finish: Chestnut brown.

        - Exterior Textured Color Finish: Cocoa.

        - Exterior Textured Color Finish: Hickory.

        - Exterior Textured Color Finish: Sandalwood.

        - Exterior Textured Color Finish: Sable.

        - Exterior Textured Color Finish: Midnight Black.

        - Exterior Textured Color Finish: Anthracite Grey.

        - Exterior Textured Color Finish: Espresso.

        - Interior Wood Grains: Kolonial Oak.

        - Interior Wood Grains: Light Oak.

        - Interior Wood Grains: Walnut.

        - Interior Wood Grains: Stainable/Paintable Pine.

        - Interior Textured Color Finishes: Espresso.

        - Interior Textured Color Finishes: Hickory.

        - Interior Textured Color Finishes: Midnight Black.

** NOTE TO SPECIFIER ** Delete grilles paragraph if not required or delete grille options not required.

    - Grilles: Between glass panes.

      - Grille Width: 5/16 inch (8 mm) narrow

        - Finish: White, pewter and brass.

      - Grille Width: 5/8 inch (16 mm) flat rectangular.

        - Finish Interior: White or wood grain

        - Finish Exterior: Finish Exterior: White or color as determined by the Architect from manufacturer's standard color range.

      - Grille Width: 5/8 inch (16 mm) flat rectangular grilles.

        - Finish: Pewter all sides.

        - Finish: White all sides.

        - Finish: Black all sides.

      - Contoured 3/4 inch (19 mm) wide grilles are optional in white or white/colored exterior.

        - Finish Interior: White.

        - Finish Exterior: White or color as determined by the Architect from manufacturers standard color range.

** NOTE TO SPECIFIER ** Delete divided lites paragraph if not required or delete divided light options not required.

    - Divided Lites: Classic Simulated Divided Lites (SDL): SDL Grille bars are permanently applied to interior and exterior glass surface.

      - Grille Width: 7/8 inches (22 mm).

      - Grille Width: 2 inches (51 mm).

** NOTE TO SPECIFIER ** Airspace grilles are optional on double pane units. Delete if not required.

      - Airspace Grille: 1/4 x 5/16 inch (6 x 8 mm) pewter.

SDL ** NOTE TO SPECIFIER ** Delete blinds in between lites if not required.

    - Blinds in Between Lites: Mini blinds are a dust free solution built in between clear insulated glass.

** NOTE TO SPECIFIER ** Delete accessories paragraph if not required or delete accessories not required.

    - Accessories:

      - Nailing fin.

      - Installation clips.

** NOTE TO SPECIFIER ** Used mostly for new construction and walk-in bays and bows.

      - Brickmolds.

      - Drywall/wood return.

      - Couplers.

      - Jamb extensions.

    - Patio Door Types:

** NOTE TO SPECIFIER ** Delete patio door type options not required.

      - Patio Door Height: 79-1/2 inch (2019 mm).

        - Side Lite (O): 5 ft (1.524 m) nominal.

        - Side Lite (O): 6 ft (1.829 m) nominal.

        - Side Lite (O): 8 ft (2.438 m) nominal.

        - Two-Panel (OX): 5 ft (1.524 m) nominal.

        - Two-Panel (OX): 6 ft (1.829 m) nominal.

        - Two-Panel (OX): 8 ft (2.438 m) nominal.

        - Three-Panel (OXO): 7.5 ft (2.286 m) nominal.

        - Three-Panel (OXO): 9 ft (2.743 m) nominal.

        - Three-Panel (OXO): 12 ft (3.658 m) nominal.

        - Four-Panel (OXXO): 9.5 ft (2.896 m) nominal.

        - Four-Panel (OXXO): 11.5 ft (3.505 m) nominal.

        - Four-Panel (OXXO): 15.5 ft (4.724 m) nominal.

      - Patio Door Height: 95-1/2 inch (2426 mm). 8 ft nominal

        - Side Lite (O): 6 ft (1.829 m) nominal.

        - Side Lite (O): 8 ft (2.438 m) nominal.

        - Two-Panel (OX): 6 ft (1.829 m) nominal.

        - Two-Panel (OX): 8 ft (2.438 m) nominal.

        - Three-Panel (OXO): 9 ft (2.743 m) nominal.

        - Three-Panel (OXO): 12 ft (3.658 m) nominal.

        - Four-Panel (OXXO): 11.5 ft (3.505 m) nominal.

        - Four-Panel (OXXO): 15.5 ft (4.724 m) nominal.

** NOTE TO SPECIFIER ** Delete patio door transoms paragraph if not require or delete transom options not required.

      - Patio Door Transoms: 1-1/4 inch (32 mm) Timberstrand horizontal reinforcement required between patio door and transom.

        - 5 ft (1.524 m) Side Lite: 31-7/16 inches (798 mm).

        - 6 ft (1.829 m) Side Lite: 37-7/16 inches (951 mm).

        - 8 ft (2.438 m) Side Lite: 49-7/16 inches (1256 mm).

        - 5 ft (1.524 m) Nominal: 58-3/4 inches (1492 mm).

        - 6 ft (1.829 m) Nominal: 70-3/4 inches (1797 mm).

        - 8 ft (2.438 m) Nominal: 94-3/4 inches (2407 mm).

        - 7.5 ft (2.286 m) Nominal, 3-Panel: 88 inch (2235 mm).

        - 9 ft (2.743 m) Nominal, 3-Panel: 106 inch (2692 mm).

        - 12 ft (3.658 m) Nominal, 3-Panel: 142 inch (3607 mm).

        - 9.5 ft (2.896 m) Nominal, 4-Panel: 116 inch (2946 mm).

        - 11.5 ft (3.505 m) Nominal, 4-Panel: 140 inch (3556 mm).

        - 15.5 ft (4.724 m) Nominal, 4-Panel: 188 inch (4775 mm).

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly constructed and prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

  - FIELD QUALITY CONTROL

    - Field Inspection: Coordinate field inspection in accordance with appropriate sections in Division 01.

** NOTE TO SPECIFIER ** Include if manufacturer provides field quality control with onsite personnel for instruction or supervision of product installation, application, erection, or construction. Delete if not required.

    - Manufacturer's Services: Coordinate manufacturer's services in accordance with appropriate sections in Division 01.

  - CLEANING AND PROTECTION

    - Clean products in accordance with the manufacturer's recommendations.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION